AJ Processor ISA

The Instruction Set Architecture for the AJ Processor

November 26, 2017

Jonathan Shihata & Chanartip Soonthornwan

Cal State University, Long Beach

this page is intentionally left blank

Table of Contents

Introduction	5

Memory Structure	6

Harvard Memory Architcture	6

Machine Register Set	7

Register Table	7

Flags Register	8

PC Register	8

IR Register	8

Data Types	8

Addressing Modes	9

Immediate 16-bitAddressing	9

PC-Relative Addressing	9

Register Addressing	9

Register Indirect Addressing	9

Shift Ability	10

Barrel Shifter Diagram	10

CPU Bus Interface	11

ALU Diagram	12

CPU Architecture Diagram	13

Instruction Type Formats	14

R-Type Instruction	14

I-Type Instruction	14

J-Type Instruction	14

R-Type Instructions	15

Shift Left Logic – SLL	16

Shift Right Logic – SRL	17

Shift Right Arithmetic – SRA	18

Jump Register – JR	19

Move From Hi – MFHI	20

Move From Lo – MFLO	21

Multiply – MULT	22

Divide – DIV	23

Addition – ADD	24

Unsigned Addition – ADDU	25

Subtract – SUB	26

Unsigned Subtraction – SUBU	27

AND	28

OR	29

Exclusive Or – XOR	30

Not Or – NOR	31

Set On Less Than – SLT	32

Set On Less Than Unsigned – SLTU	33

Breakpoint – BREAK	34

Set Interrupt – SETIE	35

I-Type Instructions	36

Branch on Equal – BEQ	37

Branch on Not Equal – BNE	38

Add Immediate – ADDI	39

Set on Less Than Immediate – SLTI	40

Set on Less Than Immediate Unsigned – SLTIU	41

And Immmediate – ANDI	42

Or Immediate – ORI	43

Exclusive Or Immediate – XORI	44

Load Upper Immediate – LUI	45

Load Word – LW	46

Store Word – SW	47

J-Type Instructions	48

Unconditional Jump – J	49

Jump And Link – JAL	50

Enhanced E-Type Instructions	51

Input	52

Output	53

Reti—return from interupt	54

Bge – BRANCH GREATER THAN EQUAL	55

Djnz – DECREMENT AND JUMP IF NOT ZERO	56

Rotl – rotate left	57

Rotr – rotate RIGHT	58

Clr – CLEAR	59

Mov – MOVE	60

Nop – no operation	61

Push	62

Pop	63

Verilog Implementation	65

Top Level Module	65

CPU Module	68

MCU	71

ALU	102

Barrell Shifter	105

ALU MIPS	109

ALU Multiplier	112

ALU Divider	113

Instruction Unit	114

IDP	116

Data Memory	119

IO Memory	121

PC	123

Memory	124

32-bit loadable register	126

32-bit Non-Loadable Regiter	127

Register File	128

Verification	129

# Introduction

The purpose of this document is for the operation and function of the Instruction Set Architecture function of the AJ Processor, a non-pipelined RISC MIPS based machine. Presented is the memory architecture and how it operates with the processor, the CPU architecture and all of its registers and their functions, as well as the CPU control path and data path of the machine. The machine contains 32 general purpose registers some of which may be accessible by the programmer as described. The machine contains dedicated flag registers that are documented here in their operation. The addressing modes along with the instruction type formats are described in detail as well. The processor is MIPS based but contains enhancements influenced by the 8051 processor. This makes the AJ processor more programmer friendly while maintaining the proven architecture of a MIPS cpu.

# Memory Structure

### Harvard Memory Architcture

The Memory is structured using a Harvard Memory architecture. The Instruction memory is separated from the Data Memory and I/O Memory. The memory has 8-bit wide words and is 4KB deep, each address is 32-bits wide and is byte addressable. The memory is implemented in a “Big Endian” format. Since each memory locations holds an 8-bit word, 4 consecutive memory locations will be used to create a single 32-bit memory operand. The processor is optimized to execute instructions while reading/writing data in parallel.

| Address | Memory |
| --- | --- |
| 4095 | __ |
| … | __ |
| 3 | AB |
| 2 | CD |
| 1 | 12 |
| 0 | 34 |

32’h ABCD_1234

1 Instruction

CPU

Instruction Memory

4K

Data Memory

4K

I/O Memory

1K

# Machine Register Set

The machine contains 32 user-available register each 32-bits wide; some of which are preserved across function calls. There are certain non-user directly modifiable registers such as, the $zero register which always contains 32’b0, the $at register which is used for temporary calculations by the assembler, $gp which is used for the global pointer, $sp which is used for the stack pointer, $fp which is used for the frame pointer, and the $ra holds the return address. The $s and $t registers are the most often used registers for operands of the ALU. Below is a table of the 32 machine registers of the CPU.

### Register Table

| Name | Number | USE | Preservation on Call? |
| --- | --- | --- | --- |
| $zero | 0 | Constant value 0 | N/A |
| $at | 1 | Assembler Temporary | No |
| $v0 - $v1 | 2 – 3 | Value for Function results and expression evaluation | No |
| $a0 - $a3 | 4 – 7 | Arguments | No |
| $t0 - $t7 | 8 – 15 | Temporaries | No |
| $s0 - $s7 | 16 – 23 | Saved Temporaries | Yes |
| $t8 - $t9 | 24 – 25 | Temporaries | No |
| $k0 - $k1 | 26 – 27 | Reserved for OS kernel | No |
| $gp | 28 | Global Pointer | Yes |
| $sp | 29 | Stack Pointer | Yes |
| $fp | 30 | Frame Pointer | Yes |
| $ra | 31 | Return Address | Yes |

### Flags Register

The status of the flags are held in this register. Instructions that are affected or affect any of these registers are compared to these registers and update their contents. Most are set by arithmetic or logical operations except for the Interrupt Enable. The flags registers are shown in the table below.

| 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- |
| IE | C | N | Z | V |
| Interrupt Enable | Carry Flag | Negative Flag | Zero Flag | Overflow Flag |

### PC Register

The PC (Program Counter) register holds the 32-bit value of the address of the next instruction to be fetched. After each instruction is executed the PC register is incremented by four since each instruction consumes 4 memory locations. the register may be accessed directly by the programmer, or through I or J type instructions such as Branches and Jumps.

### IR Register

The IR (Instruction Register) is a 32-bit user direct addressable register that holds the current instruction being executed or decoded. In it is contained the opcode, operands, a function, and shift amount, or immediate value, or jump address as defined by the instruction format type shown in this document.

# Data Types

The Processor has two possible data types: 32-bit signed and unsigned integers. Instructions can utilize integers smaller than 32-bits, but no larger. In the case of multiply and divide instructions the resultant is stored in two 32-bit registers (HI and LO), thus creating a 64-bit resultant. In a divide instruction, the 2 32-bit registers are used for the 32-bit remainder and 32-bit quotient.

- 32-bit Unsigned Integer

  - Updated Flags consist of Overflow, Carry, and Zero

- 32-bit Signed Integer

  - Updated Flags consist of Overflow, Negative, Carry, and Zero

# Addressing Modes

### Immediate 16-bitAddressing

  - I type instructions use a 16-bit immediate field at the least significant bit field of the instruction.

16 bits

Immediate

  - Used in branch instructions where if condition is met, the 16-bit immediate field is sign extended into 32 bits with 2 bits of 0, and shifted left 2 places, then written to the program counter and instructions continue executing.

i.e. PC  { Sign Ext[imm]<<2,  2’b0}

| 14 bits | 16 bits | 2 bits |
| --- | --- | --- |
| Sign Extension | Immediate << 2 | 00 |

### PC-Relative Addressing

  - J-Type instructions (Jumps) use a 26-bit immediate addressing mode, where the lower 26-bits of the instruction represent the immediate field. The 26-bits are shifted left by 2 with 2 bits of 0 and the upper 4 bits of the PC are concatenated together to create the 32-bit address to jump to.

Example: PC  {4’(PC+4), Sign Ext[26’imm]<<2,  2’b0}

| 4 bits | 26 bits | 2 bits |
| --- | --- | --- |
| PC+4 | Sign Extension Immediate 16 << 2 | 00 |

### Register Addressing

  - Used in R-Type instructions, operands of instruction are pointed by $rs and $rt, while the output value is placed in the destination register pointed by $rd.

Example: 	ADD $s1, $s2, $s3 			R[$s1]  $s2 + $s3

### Register Indirect Addressing

  - Used for LOAD/STORE instructions.

  - Format: 	LOAD/STORE $rt offset($rs)

    - LOAD uses address from $rs and adds a 16-bit offset and loads $rt with the memory contents at that location

    - STORE uses the value in $rt and stores its contents into the memory at location pointed at by $rs with a 16-bit offset

  - Example: 	LW $rt 8($rs) 				R[$rt]  Mem[8+$rs]

# Shift Ability

The processor functionality is expanded to add the ability to perform shifts. A barrel shifter is implemented into the design to allow for this. The implementation of the barrel shifter allows for the shifting of up to 32-bits in either direction in one clock cycle. This allows our design to preform right rotations. The shifter operates in parallel to the ALU independently. The diagram below shows the simplified path for the barrel shifter.

### Barrel Shifter Diagram

Register File

RS			RT

Shamt

Barrel Shifter

ALU

Sel

Y-mux

Y

# CPU Bus Interface

CPU

D_in

D_out

ALU_out

Address

Data

Data

I/O Memory

Data Memory

Addr

D_in

D_out

Addr

D_in

D_out

# ALU Diagram

Multiplier

S x T

[63:0] Product

Product

S

S

ALU

T

T

V

C

C

V

S

{Y_Hi. Y_Lo}

Y-MUX

{Y_Hi. Y_Lo}

FS

T

Z

=

FS

0

FS

Divider

S / T

S

N

Y_Hi[31]

0

1

T

Y_Lo[31]

FS

Remainder, Quotient

[63:0]

{Remainder, Quotient}

# CPU Architecture Diagram

S_sel

SP_sel

Instruction Data Path

Jump

Branch

DY (Mem/IO)

0

1

S_Addr

PC_in

ALU_out

$sp

S_Addr

D_EN

T_Addr

S

Register File

T

D_Addr

D_in

RS

PC_in

PC_out

Instruction Unit

Sign_ext 16

2

1

0

D_in

0

1

S

Y_Lo

ALU

Y_Hi

T

T_addr

$sp

$ra

D_addr

ALU_out

100

011

Y-MUX

000

010

001

RT

0

1

2

3

ALU_out

T-MUX

0       1      2      3

D_out

Barrel Shifter

D_in

Sh_sel

Shamt

Hi

Lo

Y_sel

{27’b0,flags}

PC_in

DA_sel

shamt

SH_sel

T_sel

# Instruction Type Formats

### R-Type Instruction

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| opcode 000000 | rs 00000 | rt 00000 | rd 00000 | shamt 00000 | funct 000000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

### I-Type Instruction

31                        26  25                       21  20                      16  15                                                                                                    0

| opcode 000000 | rs | rt | offset |
| --- | --- | --- | --- |

6 bits                            5 bits                         5 bits                                                            16 bits

### J-Type Instruction

31                      26   25                                                                                                                                                                             0

| opcode 000000 | offset |
| --- | --- |

6 bits                                                                                                  26 bits

# R-Type Instructions

Instruction Format Bit Assignment:

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| opcode 000000 | rs 00000 | rt 00000 | rd 00000 | shamt 00000 | funct 000000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

Flags:

C: Carry flag

N: Negative flag

Z: Zero flag

O: Overflow flag

## Shift Left Logic – SLL

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | rd | shamt | funct |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:         sll rd, rt, shamt

purpose:        shift left contents of rt by shamt and store it in rd

description:    R[rd]= R[rt] << shamt

the destination register(rd) gets the contents of the source(rt) register shifted logically left by the shamt field. zeros are shifted in for the shamt.

restrictions:

the shift amount is limited by what can be specified using only 5 bits or a value of 32.

operation:

PC  PC +4; IR  M[PC];

RS  reg[IR[25-21]]; RT  reg[IR[20-16]];

ALU_OUT <- RT << shamt;

reg[IR[15-11]]  ALU_OUT(RT << shamt);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SLL R3, R1, 29 | 000000_00000_00001_00011_11101_000000 |
| SLL R10, R13, 4 | 000000_00000_01101_01010_00100_000000 |

## Shift Right Logic – SRL

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | rd | shamt | funct |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:         srl rd, rt, shamt

purpose:        shift right the contents of rt by shamt and store it in rd

description:    R[rd]= R[rt] >> shamt

the destination register(rd) gets the contents of the source(rt) register shifted logically right by the shamt field. zeros are shifted in for the shamt.

restrictions:

the shift amount is limited by what can be specified using only 5 bits or a value of 32.

operation:

PC  PC +4; IR <- M[PC];

RS  reg[IR[25-21]]; RT  reg[IR[20-16]];

ALU_OUT <- RT >> shamt;

reg[IR[15-11]]  ALU_OUT(RT >> shamt);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SRL R3, R1, 29 | 000000_00000_00001_00011_11101_000000 |
| SRL R10, R13, 4 | 000000_00000_01101_01010_00100_000000 |

## Shift Right Arithmetic – SRA

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | rd | shamt | 000011 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:         sra rd, rt, shamt

purpose:        shift right the contents of rt by shamt and store it in rd

description:    R[rd]= R[rt] >>> shamt

the destination register(rd) gets the contents of the source(rt) register shifted arithmetically right by the shamt field. sign bit is shifted in.

restrictions:

the shift amount is limited by what can be specified using only 5 bits or a value of 32.

operation:

PC  PC +4; IR  M[PC];

RS  reg[IR[25-21]]; RT  reg[IR[20-16]];

ALU_OUT  RT >>> shamt;

reg[IR[15-11]] <- ALU_OUT(RT >>> shamt);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SRA R3, R1, 6 | 000000_00001_00000_00011_00110_00011 |
| SRA R10, R13, 20 | 000000_01101_00000_01010_10100_00011 |

## Jump Register – JR

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | 00000 | 00000 | 00000 | 001000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            jr rs

purpose:           jump to the address stored in rs.

description:       pc  M[rs]

jumps to the effective address specified by $rs.

restrictions:

if either of the two least significant bits are not zero, then an error occurs and will violate the branch address.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS;

REG[IR[15-11]]  ALU_OUT(RT >>> SHAMT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| JR R31 | 000000_11111_00000_00000_00000_001000 |
| JR R16 | 000000_10000_00000_00000_00000_001000 |

## Move From Hi – MFHI

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | 00000 | rd | 00000 | 010000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            mfhi rd

purpose:           to copy the special hi register to the general purpose register rd.

description:       R[rd]  hi

the contents of special register hi is loaded into rd.

restrictions:

mfhi will not allow a multiply or divide instruction within two instructions after it has been executed because it violates how the mips pipeline works.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

REG[IR[15-11]]  ALU_OUT[31-16];

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| MFHI R6 | 000000_00000_00000_00000_00110_010000 |
| MFHI R8 | 000000_00000_00000_00000_01000_010000 |

## Move From Lo – MFLO

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | 00000 | rd | 00000 | 010010 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            mflo rd

purpose            to copy the special lo register to the general purpose register rd.

description:       R[rd]  lo

the contents of special register lo is loaded into rd.

restrictions:

mflo will not allow a multiply or divide instruction within two instructions after it has been executed because it violates how the mips pipeline works.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

REG[IR[15-11]]  ALU_OUT[15-16];

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| MFLO R6 | 000000_00000_00000_00000_00110_010010 |
| MFLO R8 | 000000_00000_00000_00000_01000_010010 |

## Multiply – MULT

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | 00000 | 00000 | 011000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            mult rs, rt

purpose            to multiply 32 bit signed integers

description:       {hi, lo}  R[rs] * R[rt]

the 32-bit value in $rt are multiplied by the 32-bit value in $rs, and storing the the 64-bit result in {hi,lo}

restrictions:

32-bit values in $rs and $rt must be signed.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT[15-16]  RS * RT

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| MULT R3, R1 | 000000_00011_00001_00000_00000_011000 |
| MULT R10, R13 | 000000_01010_01101_00000_00000_011000 |

## Divide – DIV

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | 00000 | 00000 | 011010 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            div rs, rt

purpose            to divide 32 bit signed integers

description:       lo  R[rs] / R[rt], hi  R[rs] % R[rt]

the 32-bit value in $rt are divided by the 32-bit value in $rs, and storing the the 64-bit result in {hi,lo}

restrictions:

32-bit values in $rs and $rt must be signed.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT[15-16]  RS / RT

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| DIV R3, R1 | 000000_00011_00001_00000_00000_011010 |
| DIV R10, R13 | 000000_01010_01101_00000_00000_011010 |

## Addition – ADD

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            add rd, rs, rt

purpose            to add 32 bit signed integers

description:       R[rd]  R[rs] + R[rt]

the 32-bit value in $rt is added to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

32-bit values in $rs and $rt must be signed.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS + RT

REG[IR[15-11]]; RT  ALU_OUT(RS + RT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| ADD R3, R1, R2 | 000000_00001_00010_00011_00000_100000 |
| ADD R10, R13, R6 | 000000_01101_00110_01010_00000_100000 |

## Unsigned Addition – ADDU

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100001 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            addu rd, rs, rt

purpose            to add 32 bit signed integers

description:       R[rd]  R[rs] + R[rt]

the 32-bit value in $rt is added to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

32-bit values in $rs and $rt must be unsigned.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS + RT

REG[IR[15-11]]; RT  ALU_OUT(RS + RT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| ADDU R3, R1, R2 | 000000_00001_00010_00011_00000_100001 |
| ADDU R10, R13, R6 | 000000_01101_00110_01010_00000_100001 |

## Subtract – SUB

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100010 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            sub rd, rs, rt

purpose            to subtract 32 bit signed integers

description:       R[rd]  R[rs] - R[rt]

the 32-bit value in $rt is sutracted to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

32-bit values in $rs and $rt must be signed.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS - RT

REG[IR[15-11]]; RT  ALU_OUT(RS - RT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SUB R3, R1, R2 | 000000_00001_00010_00011_00000_100010 |
| SUB R10, R13, R6 | 000000_01101_00110_01010_00000_100010 |

## Unsigned Subtraction – SUBU

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100011 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            subu rd, rs, rt

purpose            to subtract 32 bit signed integers

description:       R[rd]  R[rs] - R[rt]

the 32-bit value in $rt is subtracted to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

32-bit values in $rs and $rt must be unsigned.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS - RT

REG[IR[15-11]]; RT  ALU_OUT(RS - RT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SUBU R3, R1, R2 | 000000_00001_00010_00011_00000_100011 |
| SUBU R10, R13, R6 | 000000_01101_00110_01010_00000_100011 |

## AND

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100100 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            and rd, rs, rt

purpose            to do a bitwise and

description:       R[rd]  R[rs] & R[rt]

the 32-bit value in $rt is and’d bit by bit to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

n/a

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS & RT

REG[IR[15-11]]; RT  ALU_OUT(RS & RT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| AND R3, R1, R2 | 000000_00001_00010_00011_00000_100100 |
| AND R10, R13, R6 | 000000_01101_00110_01010_00000_100100 |

## OR

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100101 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            or rd, rs, rt

purpose            to do a bitwise or

description:       R[rd]  R[rs] | R[rt]

the 32-bit value in $rt is or’d bit by bit to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

n/a

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS | RT

REG[IR[15-11]]; RT  ALU_OUT(RS | RT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| OR R3, R1, R2 | 000000_00001_00010_00011_00000_100101 |
| OR R10, R13, R6 | 000000_01101_00110_01010_00000_100101 |

## Exclusive Or – XOR

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100111 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            xor rd, rs, rt

purpose            to do a bitwise exclusive or

description:       R[rd]  R[rs] ^ R[rt]

the 32-bit value in $rt is exclusive or’d bit by bit to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

n/a

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  RS ^ RT

REG[IR[15-11]]; RT  ALU_OUT(RS ^ RT);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| XOR R3, R1, R2 | 000000_00001_00010_00011_00000_100110 |
| XOR R10, R13, R6 | 000000_01101_00110_01010_00000_100110 |

## Not Or – NOR

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 100111 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            nor rd, rs, rt

purpose            to do a bitwise nor

description:       R[rd]  ~(R[rs] | R[rt])

the 32-bit value in $rt is nor’d bit by bit to the 32-bit value in $rs, and storing the the 32-bit result in $rd

restrictions:

n/a

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  ~(RS | RT)

REG[IR[15-11]]; RT  ALU_OUT(~(RS | RT));

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| NOR R3, R1, R2 | 000000_00001_00010_00011_00000_100111 |
| NOR R10, R13, R6 | 000000_01101_00110_01010_00000_100111 |

## Set On Less Than – SLT

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 101010 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            slt rd, rs, rt

purpose            to compare for less than and return the boolean result as a 0 or 1

description:       R[rd]  (R[rs] < R[rt]) ? 1:0

the 32-bit value in $rt is compared to be less than the 32-bit value in $rs, and storing a 1 in $rd if its true or 0 if its false.

restrictions:

n/a

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  (RS < RT) ? 1:0;

REG[IR[15-11]]; RT  ALU_OUT((RS < RT) ? 1:0);

exceptions:

$rd is set to 0 if $rs and $rt are equal to each other.

example:

| Assembly Code | Machine Code |
| --- | --- |
| SLT R3, R1, R2 | 000000_00001_00010_00011_00000_101010 |
| SLT R10, R13, R6 | 000000_01101_00110_01010_00000_101010 |

## Set On Less Than Unsigned – SLTU

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | rd | 00000 | 101011 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            sltu rd, rs, rt

purpose            to compare for an unsigned less than and return the Boolean result	as a 0 or 1.

description:       R[rd]  (R[rs] < R[rt]) ? 1:0

the 32-bit value in $rt is compared to be less than the 32-bit value in $rs, and storing a 1 in $rd if its true or 0 if its false.

restrictions:

The 32-bit values of register $rs and $rt must be unsinged.

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

ALU_OUT  (RS < RT) ? 1:0;

REG[IR[15-11]]; RT  ALU_OUT((RS < RT) ? 1:0);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SLT R3, R1, R2 | 000000_00001_00010_00011_00000_101011 |
| SLT R10, R13, R6 | 000000_01101_00110_01010_00000_101011 |

## Breakpoint – BREAK

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | 00000 | 00000 | 00000 | 001101 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            break

purpose            to stop in a program

description:

will display that the break instruction has been fetched with the time, then inform of a safe break and complete a register dump task and a memory dump task.

restrictions:

n/a

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

NS  BREAK;

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| BREAK | 000000_00000_00000_00000_00000_001101 |

## Set Interrupt – SETIE

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | 00000 | 00000 | 00000 | 011111 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            setie rd, rs, rt

purpose            to set the interrupt flag high

description:

the current interrupt flag will be updated to a value of 1’b1 and pass it to the next interrupt enable flag.

restrictions:

n/A

operation:

PC  PC +4; IR  M[PC];

RS  REG[IR[25-21]]; RT  REG[IR[20-16]];

NS

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SETIE | 000000_00000_00000_00000_00000_011111 |

# I-Type Instructions

31                        26  25                       21  20                      16  15                                                                                                    0

| OPCODE | RS | RT | IMMEDIATE / OFFSET |
| --- | --- | --- | --- |

6 bits                              5 bits                        5 bits                                                               16 bits

Flags:

C: Carry flag

N: Negative flag

Z: Zero flag

O: Overflow flag

## Branch on Equal – BEQ

31                        26  25                       21  20                      16  15                                                                                                    0

| 000100 | rs | rt | offset |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            beq rt, rs, offset

purpose:           equal register-register compare and branch with 16-bit offest.

description:       if(r[rs] == r[rt]) pc  pc + 4 + branch_addr

if the condition is true, the branch is taken and the follwed instruction will not be executed. if the branch is taken, 16-bit offset will be used to reform a pc-relative effective address by concatenated with 14-bit of offset[15], the offset, and two zeroes.

restrictions:

the branch may be followed by another branch or jump that would delay the execution

operation:

branch_addr  {14{offset[15]}, offset, {00}}

beq: cond  r[rs] = r[rt]

if cond then

pc  (pc+4+ branch_addr)

end if

exceptions:

The offset must be within the range -32,768 and 32,767.

example:

| Assembly Code | Machine Code |
| --- | --- |
| BEQ R1, R2, LOOP | 001100_00001_00010_0000101001011010 |
| BEQ R0, R3, EXIT | 001100_00000_00011_1111111111111111 |

## Branch on Not Equal – BNE

31                        26  25                       21  20                      16  15                                                                                                    0

| 000101 | rs | rt | offset |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            bne rt, rs, offset

purpose:           not equal register-register compare and branch with 16-bit offest.

description:       if(r[rs] != r[rt]) pc  pc + 4 + branch_addr

if the condition is true, the branch is taken and the follwed instruction will not be executed. if the branch is taken, 16-bit offset will be used to reform a pc-relative effective address by concatenated with 14-bit of offset[15], the offset, and two zeroes.

restrictions:

the branch may be followed by another branch or jump that would delay the execution

operation:

branch_addr  {14{offset[15]}, offset, {00} }

bne: cond  r[rs] ≠ r[rt]

if cond then

pc  (pc+4+ branch_addr)

end if

exceptions:

The offset must be within the range -32,768 and 32,767.

example:

| Assembly Code | Machine Code |
| --- | --- |
| BNE R1, R2, LOOP | 000101_00001_00010_0000101001011010 |
| BNE R0, R3, EXIT | 000101_00000_00011_1111111111111111 |

## Add Immediate – ADDI

31                        26  25                         21  20                        16  15                                                                                                0

| 001000 | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            addi rt, rs, offset

purpose:           add a constant with 16-bit offest.

description:       r[rt]  r[rs] + sign_extend(immediate)

adding 32-bit value in r[rs] with 16-bit signed offet and the 32-bit logical result is placed into r[rt].

restrictions:

The Immediate value must be within the range -32,768 and 32,767.

operation:

sign_extend  {16{immediate[15]}, immediate}

temp  r[rs] + sign_extend(immediate)

r[rt]  temp

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| ADDI R2, R1, 5 | 001000_00001_00010_0000000000001010 |
| ADDI R3, R0, -1 | 001000_00000_00011_1111111111111111 |

## Set on Less Than Immediate – SLTI

31                        26  25                       21  20                      16  15                                                                                                    0

| 001010 | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            slti rt, rs, offset

purpose:           record the result of a less-than comparison with a constant.

description:       r[rt]  r[rs] < sign_extend(immediate)

compare the content of r[rs] and the 16-bit signed immediate, and then record a boolean result of the comparison in r[rt]. if r[rs] less than the immediate is true, return 1 (true). Or 0 (flase) otherwise.

restrictions:

The Immediate value must be within the range -32,768 and 32,767.

operation:

sign_extend  {16{immediate[15]}, immediate}

if r[rs] < sign_extend(immediate) then

r[rt]  zero || 1

else

r[rt]  zero

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SLTI R2, R1, 5 | 001010_00001_00010_0000000000000101 |
| SLTI R3, R0, -1 | 001010_00000_00011_1111111111111111 |

## Set on Less Than Immediate Unsigned – SLTIU

31                        26  25                       21  20                      16  15                                                                                                    0

| 001011 | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            sltiu rt, rs, immediate

purpose:           record the result of a less-than comparison with a constant.

description:       r[rt]  r[rs] < sign_extend(immediate)

compare the content of r[rs] and the 16-bit signed immediate as unsigned integer, and then record a boolean result of the comparison in r[rt]. if r[rs] less than the immediate is true, return 1 (true). Or 0 (flase) otherwise.

restrictions:

The Immediate value must be within the range -32,768 and 32,767.

operation:

if r[rs] < sign_extend(immediate) then

r[rt]  zero || 1

else

r[rt]  zero

exceptions:

n/a

programming notes:

since sign-extend of immediate is 16 bit, 16th bit could extend the range of unsigned number from [0,32767] to [0, 65535]

example:

| Assembly Code | Machine Code |
| --- | --- |
| SLTI R2, R1, 0x4040 | 001010_00001_00010_0100000001000000 |
| SLTI R3, R0, 0x8000 | 001010_00000_00011_1000000000000000 |

## And Immmediate – ANDI

31                        26  25                       21  20                      16  15                                                                                                    0

| 001100 | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            andi rt, rs, immediate

purpose:           perform a bitwise logic and with a constant.

description:       r[rt]  r[rs] & zero_extend(immediate)

extend immediate with 16-bit zero, then bitwise logical and the content of r[rs], and then place the result into r[rt].

restrictions:

The Immediate value must be within the range -32,768 and 32,767.

operation:

zero_extend  {16{zero}, immediate}

r[rt]  r[rs] & zero_extend

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| ANDI R2, R1, 8 | 001100_00001_00010_0000000000001000 |
| ANDI R3, R0, -1 | 001100_00000_00011_1111111111111111 |

## Or Immediate – ORI

31                        26  25                       21  20                      16  15                                                                                                    0

| 001101 | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            ori rt, rs, immediate

purpose:           perform a bitwise logic or with a constant.

description:       r[rt]  r[rs] | zero_extend(immediate)

extend immediate with 16-bit zero, then bitwise logical or the content of r[rs], and then place the result into r[rt].

restrictions:

The Immediate value must be within the range -32,768 and 32,767.

operation:

zero_extend  {16{zero}, immediate}

r[rt]  r[rs] | zero_extend

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| ORI R2,0xFFFF | 001101_00000_00010_1111111111111111 |
| ORI R3, 0x0001 | 001101_00000_00011_0000000000000001 |

## Exclusive Or Immediate – XORI

31                        26  25                       21  20                      16  15                                                                                                    0

| 001110 | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            xori rt, rs, immediate

purpose:           perform a bitwise logic xor with a constant.

description:       r[rt]  r[rs] ^ zero_extend(immediate)

extend immediate with 16-bit zero, then bitwise logical xor the content of r[rs], and then place the result into r[rt].

restrictions:

The Immediate value must be within the range -32,768 and 32,767.

operation:

zero_extend  {16{zero}, immediate}

r[rt]  r[rs] ^ zero_extend

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| XORI R2, R1, 8 | 001100_00001_00010_0000000000001000 |
| XORI R3, R0, -1 | 001100_00000_00011_1111111111111111 |

## Load Upper Immediate – LUI

31                        26  25                       21  20                      16  15                                                                                                    0

| 001111 | rt | 00000 | immediate |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            lui rt, offset

purpose:           load an upper half word to a register

description:       r[rt]  {immediate, 16{zero}}

extend immediate with 16-bit zero, then bitwise logical xor the content of r[rs], and then place the result into r[rt].

restrictions:

The Immediate value must be within the range -32,768 and 32,767.

operation:

r[rt]  {immediate, 16{zero}}

exceptions:

n/a

programming notes:

assign rs as 5-bit zero at rs bits in the instruction.

example:

| Assembly Code | Machine Code |
| --- | --- |
| LUI R2,0xFFFF | 001111_00000_00010_1111111111111111 |
| LUI R3, 0x0001 | 001111_00000_00011_0000000000000001 |

## Load Word – LW

31                        26  25                       21  20                      16  15                                                                                                    0

| 100011 | base | rt | offset |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            lw rt, offset(base)

purpose:           load a word from memory.

description:       r[rt]  m[gpr[base] + offset]

load 32-bit word from a memory located at the fetched effective address (base+offset) and then place to r[rt].

restrictions:

effective address 2 least-significant bits have to be two zeroes, otherwise, address error exception will occurs.

operation:

sign_extend  {16{offset}, offset}

maddr  sign_extend + gpr[base]

r[rt]  memory[maddr]

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| LW R2,0(R15) | 100011_01111_00010_0000000000000000 |
| LW R3, 4(R15) | 100011_01111_00011_0000000000000100 |

## Store Word – SW

31                        26  25                       21  20                      16  15                                                                                                    0

| 101011 | base | rt | offset |
| --- | --- | --- | --- |

6 bits                                 5 bits                        5 bits                                                               16 bits

format:            lw rt, offset(base)

purpose:           store a word from memory.

description:       M[gpr[base]+offset]  r[rt]

store 32-bit word to a memory located at the fetched effective address (base+offset) and then place to r[rt].

restrictions:

effective address 2 least-significant bits have to be two zeroes, otherwise, address error exception will occurs.

operation:

sign_extend  {16{offset}, offset}

maddr  sign_extend + gpr[base]

memory[maddr]  r[rt]

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| SW R2,0(R15) | 101011_01111_00010_0000000000000000 |
| SW R3, 4(R15) | 101011_01111_00011_0000000000000100 |

# J-Type Instructions

31                      26   25                                                                                                                                                                             0

| OPCODE | ADDRESS |
| --- | --- |

6 bits                                                                                                   26 bits

Flags:

C: Carry flag

N: Negative flag

Z: Zero flag

O: Overflow flag

## Unconditional Jump – J

31                      26   25                                                                                                                                                                             0

| 000010 | address |
| --- | --- |

6 bits                                                                                                   26 bits

format:            j address

purpose:           unconditional branch (or jump)

description:       pc  jmp_address

form a pc-related effective target address from concatenated 4 most-significant bit of pc+4, offset, and two zeroes, and then update the program counter (pc) with the new pc.

restrictions:

the branch may be followed by another branch or jump that would delay the execution

operation:

jmp_address  {pc+4 [31:28], offset, 2{zero}}

pc  jmp_address

exceptions:

n/a

programming notes:

example:

| Assembly Code | Machine Code |
| --- | --- |
| J top | 100011_01111000100000000000000000 |
| J exit | 100011_01111000110000000000000100 |

## Jump And Link – JAL

31                      26   25                                                                                                                                                                             0

| 000011 | address |
| --- | --- |

6 bits                                                                                                   26 bits

format:            jal address

purpose:           jump and link

description:       gpr[31]  pc+8; pc  jmp_address

store the return address in returning address register (gpr[31]). form a pc-related effective target address from concatenated 4 most-significant bit of pc+4, offset, and two zeroes, and then update the program counter (pc) with the new pc.

restrictions:

the branch may be followed by another branch or jump that would delay the execution

operation:

jmp_address  {pc+4 [31:28], offset, 2{zero}}

gpr[31]  pc+8

pc  jmp_address

exceptions:

n/a

programming notes:

example:

| Assembly Code | Machine Code |
| --- | --- |
| JAL top | 000011_01111000100000000000000000 |
| JAL exit | 000011_01111000110000000000000100 |

# Enhanced E-Type Instructions

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| opcode | rs 00000 | rt 00000 | rd 00000 | shamt 00000 | funct 000000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

31                        26  25                       21  20                      16  15                                                                                                    0

| opcode | rs 00000 | rt 00000 | immediate |
| --- | --- | --- | --- |

6 bits                            5 bits                         5 bits                                                            16 bits

31                      26   25                                                                                                                                                                             0

| opcode | offset |
| --- | --- |

6 bits                                                                                                  26 bits

26 bits

Flags:

C: Carry flag

N: Negative flag

Z: Zero flag

O: Overflow flag

## Input

31                        26  25                       21  20                      16  15                                                                                                    0

| 0x1C | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                            5 bits                         5 bits                                                            16 bits

format:            INPUT rt offset(rs)

purpose:           to receive an input from IO and load it into a GPR

description:       $rt  ioM[rs + offset]

The 32-bit value in register $rs is added to the sign-extended 16-bit immediate value to produce a memory address to read from in the IO memory. The 32-bit value that was read is then loaded into register $rt.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- IR[15:0]

INPUT: ALU_OUT <- RS($rs) + RT(IR[15:0])

INPUT2: D_in <- ioM[ALU_OUT($rs+imm)]

INPUT3: $rt <- D_in(ioM[($rs+imm)]

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| input $R5, 0($R3) | 011100_00011_00101_0000000000000000 |
| input $R1, 4($R0) | 011100_00001_00000_0000000000000100 |

## Output

31                        26  25                       21  20                      16  15                                                                                                    0

| 0x1D | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                            5 bits                         5 bits                                                            16 bits

format:            OUTPUT rt offset(rs)

purpose:           to receive an input from IO and load it into a GPR

description:       ioM[rs + offset] $rt

The 32-bit value in register $rs is added to the sign-extended 16-bit immediate value to produce a memory address to write in to the IO memory. The 32-bit value in $rt will then be stored into that memory location.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- IR[15:0]

OUTPUT: ALU_OUT <- RS($rs) + RT(imm)

OUTPUT2: ioM[ALU_OUT($rs+imm)] <- RT($rt)

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| OUTPUT $R5, 0($R3) | 011101_00011_00101_0000000000000000 |
| OUTPUT $R1, 4($R0) | 011101_00001_00000_0000000000000100 |

## Reti—return from interupt

31                        26  25                       21  20                      16  15                                                                                                    0

| 0x1E | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                            5 bits                         5 bits                                                            16 bits

format:            RETI

purpose:           to return from a interrupt service routine to the location called into the ISR from.

description:    PC  dM[$sp]

Passes in the value at $sp and at the memory location pointed at to by the stack pointer, POP’s the top of stack twice to get the status flags and the return PC. It then updates an saves the new SP value into $sp.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- IR[15:0]

RETI: ALU_OUT <- RS($sp)

RET2I: D_in <- dMem[ALU_OUT($sp)]

RETI3: Flags <- D_in, ALU_OUT <- ALU_OUT($sp) + 4

RETI4: D_in <- dMem[ALU_OUT($sp+4)], ALU_OUT <- ALU_OUT($sp+4) + 4

RETI5: PC <- D_in(PC), ALU_OUT <- ALU_OUT($sp+8)

RETI6: RF[$sp] <- ALU_OUT($sp+8)

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| RETI | 011110_00000_00000_0000000000000000 |

## Bge – BRANCH GREATER THAN EQUAL

31                        26  25                       21  20                      16  15                                                                                                    0

| 0x2D | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                            5 bits                         5 bits                                                            16 bits

format:            BGE rs, rt, branch offset

purpose:           to branch only if $Rs >= $Rt

description:     IF ($Rs >= $Rt) PC  branch offset

$rs and $rt are added and if the zero flag is high or if the negative flag is not high, then  the branch to an address calculated using the immediate offset is performed.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

BGE: ALU_OUT <- RS($rs) - RT($rt)

BGE2: if(Z || ~N); PC <- branch address

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| BGE $R5, $R3, 3 | 101101_00101_00011_0000000000000011 |
| BGE $R1, $R4, 256 | 101101_00001_00100_0000000100000000 |

## Djnz – DECREMENT AND JUMP IF NOT ZERO

31                        26  25                       21  20                      16  15                                                                                                    0

| 0x2E | rs | rt | immediate |
| --- | --- | --- | --- |

6 bits                            5 bits                         5 bits                                                            16 bits

format:            DJNZ $Rs, branch offset

purpose:           to branch only if the register decremented is not zero.

description:     IF ($Rs!>= 0) PC  branch address

Decrements a 32-bit value from a GPR and branches to a branch address if that value isn’t zero. If the value is zero, no jump is done.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- IR[15:0]

DJNZ: ALU_OUT <- RS($rs) – 1

DJNZ2: $rt <- ALU_OUT($rs - 1)

if zero flag is set, PC <- branch address

else, pass through

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| DJNZ $R5, 3 | 101110_00011_00000_0000000000000011 |
| DJNZ $R1, 128 | 101110_00001_00000_0000000010000000 |

## Rotl – rotate left

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | rd | shamt | 000001 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            ROTL rd, rt, shamt

purpose:           To rotate a word to the left by a specified number of bits, with no carry

description:

The value that register $rt holds is rotated to the left as many times as shamt specifies. The most significant bit and the least significant bit will be wrapped around. The result is placed in register $rd.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

SLL: ALU_OUT <- RT << shamt;

WB_alu: Reg[IR[15-11]] <- ALU_OUT(RT << shamt);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| ROTL $R5, $R2, 4 | 000000_00000_00101_00010_00100_000001 |
| ROTL $R1, $R3, 25 | 000000_00000_00001_00011_11001_000001 |

## Rotr – rotate RIGHT

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | rd | shamt | 000010 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            ROTR rd, rt, shamt

purpose:           To rotate a word to the right by a specified number of bits, with no carry

description:

The value that register $rt holds is rotated to the right as many times as shamt specifies. The most significant bit and the least significant bit will be wrapped around. The result is placed in register $rd.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

SLL: ALU_OUT <- RT >> shamt;

WB_alu: Reg[IR[15-11]] <- ALU_OUT(RT >> shamt);

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| ROTR $R5, $R2, 4 | 000000_00000_00101_00010_00100_000010 |
| ROTR $R1, $R3, 25 | 000000_00000_00001_00011_11001_000010 |

## Clr – CLEAR

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | 00000 | 00000 | 000101 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            CLR rt

purpose:           To clear a GPR to all 0.

description:   $rt  32’B0

Load 32 bits of 0’s into the specified GPR.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

CLR: ALU_OUT <- 0x0;

CLR2: $rt <- ALU_OUT(0x0)

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| CLR $R5 | 000000_00000_00101_00000_00000_000101 |
| CLR $R1 | 000000_00000_00001_00000_00000_000101 |

## Mov – MOVE

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | rs | rt | 00000 | 00000 | 000111 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            MOV rs, rt

purpose:           To move a 32 bit value from one register to another

description:   $Rt  $Rs

moves the contents of $Rs into $Rt.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

MOV: ALU_OUT <- RS($rs)

MOV2: $rt <- ALU_OUT($rs)

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| MOV $R5, $R7 | 000000_00101_00111_00000_00000_000111 |
| MOV $R1, $R2 | 000000_00001_00010_00000_00000_000111 |

## Nop – no operation

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | 00000 | 00000 | 00000 | 001000 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            NOP

purpose:          to provide 1 “empty” clock tick where no instruction is being preformed

description:

The value in the ALU_OUT register is passed back into ALU_OUT so that the value isn’t changed, while using up a clock tick to do it.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

NOP: ALU_OUT <- ALU_OUT

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| NOP | 000000_00000_00000_00000_00000_001000 |

## Push

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | 00000 | 00000 | 001001 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            PUSH rt

purpose:           To push a 32 bit value from a register onto the stack

description:   dM[$sp]  $Rt

Pushes $rt into the stack. push is defined as a pre-decrement, updates the $sp before pushing into stack.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

PUSH: RS <- RF[$sp], RT <- $rt

PUSH2: ALU_OUT <- RS($sp) - 4

PUSH3: dM[ALU_OUT($sp-4)] <- RT($rt)

PUSH4: $sp <- ALU_OUT($sp)

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| PUSH $R5 | 000000_00000_00101_00000_00000_001001 |
| PUSH $R7 | 000000_00000_00111_00000_00000_001001 |

## Pop

31                       26  25                        21  20                      16  15                        11  10                          6   5                            0

| 000000 | 00000 | rt | 00000 | 00000 | 001010 |
| --- | --- | --- | --- | --- | --- |

6 bits                            5 bits                          5 bits                         5 bits                         5 bits                           6 bits

format:            POP rt

purpose:           To pop a 32 bit value from the top of the stack into a register

description:    $Rt  dM[$sp]

Pops the top of the stack into a GPR specified by rt. Post-increment, updates the $sp after popping.

restrictions:

N/A

operation:

Fetch: PC <- PC +4; IR <- M[PC];

Decode: RS <- Reg[IR[25-21]]; RT <- Reg[IR[20-16]];

POP: RS <- RF[$sp]

POP2: ALU_OUT <- RS($sp)

POP3: D_in <- dM[ALU_OUT($sp)]

POP4: $rt <- D_in, ALU_OUT <- ALU_OUT($sp) + 4

POP5: $sp <- ALU_OUT

exceptions:

n/a

example:

| Assembly Code | Machine Code |
| --- | --- |
| POP $R5 | 000000_00000_00101_00000_00000_001010 |
| POP $R7 | 000000_00000_00111_00000_00000_001010 |

this page is intentionally left blank

# Verilog Implementation

## Top Level Module

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  CPU_Test_Module.v

* Project:    Final_Project

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Date 11/17/2017

*

* Rev. No.1:  Version 1.1

* Rev. Date:  Current Rev. Date 11/21/2017

* Update:     Discarded readmemh for other DM, only left for DM14

*             to perform INTR and RETI

*

* Purpose:    Top level of CPU, Data Memory, and IO Memory.

*             Performing as a test fixture of CPU and utilizing instruction

*             memory and data memory from dat files.

*

****************************************************************************/

module CPU_Test_Module;

// Inputs

reg sys_clk;

reg sys_rst;

// Interconnection wires

wire [31:0] MAddr;          // Memory Address from CPU

wire [31:0] idp_out;        // Data output from IDP in CPU

wire [31:0] Mem_out, Mem_in;// DM and IO Memory bus

// Wires for Data Memory

wire dm_cs, dm_rd, dm_wr;   // chip select, write enable, read enable

wire [31:0] DM_out;         // Data Memory output

// Wires for IO Memory

wire  io_intr;              // IO interrupt request input

wire io_int_ack;            // IO interrupt acknowledge output

wire io_cs, io_wr, io_rd;   // chip select, write enable, read enable

//wire [31:0] IO_out, IO_in; // IO Memory output

// Instantiate CPU

CPU cpu (

.sys_clk(sys_clk),

.reset(sys_rst),

.intr(io_intr),         // interrupt input from IO memory

.int_ack(io_int_ack),   // interrupt ourput to IO memory

.DM_out(DM_out),        // Data input from both memories

.MAddr(MAddr),          // Memory address to access both memories

.idp_out(idp_out),      // Data output to both memories

.dm_cs(dm_cs),          // Data memory

.dm_rd(dm_rd),          // control

.dm_wr(dm_wr),          // words

.io_cs(io_cs),          // IO memory

.io_rd(io_rd),        // control

.io_wr(io_wr)         // words

);

// Instantiate Data Memory

Data_Memory DM(

.clk(sys_clk),

.dm_cs(dm_cs),

.dm_wr(dm_wr),

.dm_rd(dm_rd),

.Addr({20'h0,MAddr[11:0]}),

.DM_In(idp_out),

.DM_Out(DM_out)

);

// Instancitate IO Memory

/* Will Change this one later*/

IO_Memory IO(

.clk(sys_clk),

.cs(io_cs),

.wr(io_wr),

.rd(io_rd),

.int_r(io_intr),

.int_ack(io_int_ack),

.Addr({20'h0,MAddr[11:0]}),

.IO_In(idp_out),

.IO_Out(DM_out)

);

// Generate 10ns clock period

always #5 sys_clk = ~sys_clk;

// Initialization

initial begin

$timeformat(-9, 1, " ns", 9);

sys_clk = 0;

sys_rst = 1;

//*******************************//

// Initialize Instruction Memory //

//*******************************//

@(negedge sys_clk)

//$readmemh("iMem01_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem02_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem03_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem04_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem05_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem06_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem07_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem08_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem09_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem10_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem11_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem12_Sp17_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem13_Fa17_w_isr_commented.dat",cpu.iu.IM.Mem);

//$readmemh("iMem14_Fa17_w_isr_commented.dat",cpu.iu.IM.Mem);

$readmemh("enhanced.dat",cpu.iu.IM.Mem);

//************************//

// Initialize Data Memory //

//************************//

@(negedge sys_clk)

//$readmemh("dMem01_Sp17.dat",DM.Mem);

//$readmemh("dMem02_Sp17.dat",DM.Mem);

//$readmemh("dMem03_Sp17.dat",DM.Mem);

//$readmemh("dMem04_Sp17.dat",DM.Mem);

//$readmemh("dMem05_Sp17.dat",DM.Mem);

//$readmemh("dMem06_Sp17.dat",DM.Mem);

//$readmemh("dMem07_Sp17.dat",DM.Mem);

//$readmemh("dMem08_Sp17.dat",DM.Mem);

//$readmemh("dMem09_Sp17.dat",DM.Mem);

//$readmemh("dMem10_Sp17.dat",DM.Mem);

//$readmemh("dMem11_Sp17.dat",DM.Mem);

//$readmemh("dMem12_Sp17.dat",DM.Mem);

//$readmemh("dMem13_Sp17.dat",DM.Mem);

//$readmemh("dMem14_Sp17.dat",DM.Mem);

$readmemh("dMem_enhanced.dat",DM.Mem);

// Bring system to 'known state'

@(negedge sys_clk)

sys_rst = 0;

end

endmodule

## CPU Module

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  CPU.v

* Project:    Final_Project

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Date 11/17/2017

*

* Rev. No.1:  Version 1.1

* Rev. Date:  Current Rev. Date 11/21/2017

* update:     Add wires for perform RETI and INTR (SP_Sel, S_Sel,

*                flag_in, flag_out).

*             Update MCU and IDP Port list.

*

* Purpose:    Top level of Instruction Unit(IU, Integer Data Path(IDP),

*             and Control Unit(MCU). Shows connection between modules above.

*

****************************************************************************/

module CPU(

input       sys_clk, reset, intr,  // System clock, reset, IO_interupt

output                   int_ack,  // IO Interrupt Acknowledge

//  From and To Memory

input  [31:0] DM_out,              // Data output from Memory

output [31:0] MAddr,  idp_out,     // Memory Address and Data in to memory

output        dm_cs,  dm_rd,  dm_wr,   // Data Memory Controls

output        io_cs,  io_rd,  io_wr    // IO Memory Controls

);

//  Controls from MCU to Instruction Unit (IU)

wire        pc_ld,  pc_inc, ir_ld;     // Program Counter Register

wire        im_cs,  im_rd,  im_wr;     // Instruction Memory

//  Controls from MCU to Integer Data Path (IDP)

wire            D_En,   HILO_ld;       // Register File

wire  [1:0] T_Sel;                     //    Controls

wire        SP_Sel, S_Sel;             // Stack Pointer Control

wire  [1:0] pc_sel, DA_sel;            // T-MUX and DA_MUX for T or D_Addr

wire  [2:0] Y_Sel;                     // ALU_Out select

wire  [4:0] FS;                        // Function select

// Flags status

//  From IDP and MCU

wire          c,n,z,v;

wire  [4:0]   flag_in, flag_out;       // input-output flag

// Interconnection

//    From Instruction Unit(IU) to IDP

wire [31:0] IR_out, pc_out, se_16;

//*****************//

// Instantiate MCU //

//*****************//

MCU mcu (

// Inputs

.sys_clk(sys_clk), .reset(reset),   .intr(intr),

.c(c), .n(n), .z(z), .v(v),

.IR(IR_out),

.sp_flags_in(flag_in),

// Outputs

.int_ack(int_ack),

// Controls For Instruction Unit

.pc_sel(pc_sel),   .pc_ld(pc_ld),   .pc_inc(pc_inc), .ir_ld(ir_ld),

.im_cs(im_cs),     .im_rd(im_rd),   .im_wr(im_wr),

// Controls For Integer Data Path (IDP)

.D_En(D_En),       .DA_sel(DA_sel), .T_Sel(T_Sel),

.HILO_ld(HILO_ld), .Y_Sel(Y_Sel),   .FS(FS),

.SP_Sel(SP_Sel),   .S_Sel(S_Sel),

.sp_flags_out(flag_out),

// Controls For Data Memory

.dm_cs(dm_cs),     .dm_rd(dm_rd),   .dm_wr(dm_wr),

// Controls For IO Memory

.io_cs(io_cs),     .io_rd(io_rd),   .io_wr(io_wr)

);

//******************//

// Instruction Unit //

//******************//

Instruction_Unit iu(

// Scalar IO

.clk(sys_clk),   .reset(reset),

.pc_sel(pc_sel), .pc_ld(pc_ld), .pc_inc(pc_inc), .ir_ld(ir_ld),

.im_cs(im_cs),   .im_wr(im_wr), .im_rd(im_rd),

// 32-bit IO

.PC_in(MAddr),   .PC_out(pc_out),

.IR_out(IR_out), .SE_16(se_16)

);

//******************//

// Integer Datapath //

//******************//

IDP idp(  // Inputs

.clk(sys_clk),

.reset(reset),

.D_En(D_En),                  // Data write Enable

.DA_Sel(DA_sel),              // Destination Address Select

.D_Addr(IR_out[15:11]),       // $rd from IR_out

.S_Addr(IR_out[25:21]),       // $rs from IR_out

.T_Addr(IR_out[20:16]),       // $rt from IR_out

.DT(se_16),                   // SignExtension from IU

.T_Sel(T_Sel),

.FS(FS),                      // Address from IR_out

.shamt(IR_out[10:6]),         // Shifting amount from IR_out

.HILO_ld(HILO_ld),

.DY(DM_out),                  // Input from Data Memory output

.pc_in(pc_out),               // Input from Instruction Unit PC_out

.Y_Sel(Y_Sel),

.SP_Sel(SP_Sel),              // Stack

.S_Sel(S_Sel),                //       Pointer Control

.sp_flags_in(flag_out),       //

// Outputs

.sp_flags_out(flag_in),       //

.C(c),                        // Flag

.V(v),                        //    status

.N(n),                        //       to

.Z(z),                        //          MCU

.ALU_OUT(MAddr),              // Memory Address to Data and IO memory

.D_OUT(idp_out)               // Data output to Data and IO memory

);

endmodule

## MCU

`timescale 1ns / 1ps

/**************************** C E C S 4 4 0 *********************************

*

* File Name:  MCU.v

* Project:    Final Project

* Designer:   Chanartip Soonthornwan

*                 Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com

*                  Jonnyshihata@gmail.com

*

* Rev. No.1:  Version 1.0

* Rev. Date:  10/24/2017

*

* Rev. No.2:  Version 1.1

* Rev. Date:  11/18/2017

* update   :  add Dump_mem to display 32 registers on Data Memory and IO Memory

*

* Rev. No.3:  Version 2.0

* Rev. Date:  Current Rev. 11/21/2017

* update   :  Added SP_Sel, S_Sel for stack PC and flags on Memory.

*             Added ports for stacked flags.

*             Added RETI and Edited INTR states

*

* Purpose:    A state machine implementing the MIPS Control Unit (MCU)

*             for the major cycles of fetch, execute and some MIPS instructions

*             from memory, including checking for interrupts.

*

****************************************************************************

*---------------------------------------------------------------------------

* MCU C O N T R O L W O R D

*---------------------------------------------------------------------------

*                            int_ack=0;   FS=5'h0;

*        {pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

*                 {im_cs, im_rd, im_wr} = 3'b0_0_0;

* {D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

*                 {dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

*                 {io_cs, io_rd, io_wr} = 3'b0_0_0;

*                       {SP_Sel, S_Sel} = 2'b0_0;

*          #1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

*

*****************************************************************************/

module MCU (sys_clk, reset, intr,              // system inputs

c, n, z, v,                        // ALU status inputs

IR,                                // Instruction Register input

int_ack,                           // output to I/O subsystem

pc_sel,  pc_ld,  pc_inc, ir_ld,    // rest of control word fields

im_cs,   im_rd,  im_wr,

D_En,    DA_sel, T_Sel,  HILO_ld,  Y_Sel,

SP_Sel,  S_Sel,  sp_flags_in, sp_flags_out,

dm_cs,   dm_rd,  dm_wr,

io_cs,   io_rd,  io_wr,

FS);

//***************************************************************************

input    sys_clk, reset, intr;   // system clock, reset, interrupt request

input             c, n, z, v;    // Integer ALU status inputs

input    [31:0]   IR;            // Instruction Register input from IU

output   reg      int_ack;     // Interrupt acknowledge

input    [4:0]    sp_flags_in;   // Stack Pointer flags input

// All OF THE REMAINING CONTROL WORD OUTPUTS

//       For Instruction Unit

output   reg     pc_ld, pc_inc, ir_ld;     // Program Counter Register

output   reg     im_cs, im_rd,  im_wr;     // Instruction Memory

//       For Data and IO Memory

output   reg     dm_cs, dm_rd,  dm_wr;     // Data Memory

output   reg     io_cs, io_rd,  io_wr;     // IO Memory

//

//       For Integer Data Path (IDP)

output   reg       D_En,   HILO_ld;    // Register File

output   reg [1:0] T_Sel;              //      Controls

output   reg       SP_Sel, S_Sel;      // Stack Pointer Controls

output   reg [4:0] sp_flags_out;       // Stack Pointer flags output

output   reg [1:0] pc_sel, DA_sel;     // T-MUX and DA_MUX for T or D_Addr

output   reg [2:0] Y_Sel;              // ALU_Out select

output   reg [4:0] FS;                 // Function select

// Counter variables

integer i, X, Y;

//****************************

// internal data structures

//****************************

// state assignments

parameter

RESET = 00, FETCH = 01, DECODE = 02,

// R-type (20 instructions)

SLL =  3, SRL =  4, SRA  =  5, JR   =  6, MFHI  =  7, MFLO =  8, MULT =  9,

DIV = 10, ADD = 11, ADDU = 12, SUB  = 13, SUBU  = 14, AND  = 15, OR   = 16,

XOR = 17, NOR = 18, SLT  = 19, SLTU = 20, SETIE = 21,

//R-type Continued

JR_2 = 106,

// I-type (13 instructions)

BEQ  = 24, BNE   = 25, BLEZ = 26, BGTZ = 27, ADDI = 28,

SLTI = 29, SLTIU = 30, ANDI = 31, ORI  = 32, XORI = 33,

LUI  = 34, LW    = 35, SW   = 36,

// I-type Continued

BEQ_2 = 124, BNE_2 = 125, BLEZ_2 = 126, BGTZ_2 = 127,

LW_2  = 135,

// J-type (2 instructions)

J = 37 , JAL = 38,

// J-typd Continued

JAL_2 = 138,

// E-type

INPUT   =  80, OUTPUT   =  81, RETI   =  82,

INPUT_2 = 180, OUTPUT_2 = 181, RETI_2 = 182,

ROTL    =  84, ROTR     =  85, RETI_3 = 282,

ROTL_2  = 184, ROTR_2   = 185, RETI_4 = 382,

BLT     =  86, BGE      =  87, RETI_5 = 482,

BLT_2   = 186, BGE_2    = 187, RETI_6 = 483,

CLR     =  88, DJNZ     =  89, MOV    =  90,

CLR_2   = 188, DJNZ_2   = 189, MOV_2  = 190,

NOP    =  91,

PUSH   =  92, POP    =  96,

PUSH_2 =  93, POP_2  =  97,

PUSH_3 =  94, POP_3  =  98,

PUSH_4 =  95, POP_4  =  99,

POP_5  = 100,

// Extras

WB_alu      = 50,  WB_imm = 51,  WB_Din = 52, INTR_1 = 501,

WB_hi       = 53,  WB_lo  = 54,  WB_mem = 55, INTR_2 = 502,

BREAK       = 510,                            INTR_3 = 503,

ILLEGAL_OP  = 511,                            INTR_4 = 504,

INTR_5 = 505,

INTR_6 = 506;

//state register (up to 512 states)

reg [8:0] state;

/******************

* Flags register *

******************/

reg   psi, psc, psv, psn, psz;   // flags present state registers

reg   nsi, nsc, nsv, nsn, nsz;   // flags next state registers

// Updating flags register

always @(posedge sys_clk, posedge reset)

if(reset)

{psi, psc, psv, psn, psz} = 5'b0;

else

{psi, psc, psv, psn, psz} = {nsi, nsc, nsv, nsn, nsz};

/************************************************

* 440 MIPS CONTROL UNIT (Finite State Machine) *

************************************************/

always @(posedge sys_clk, posedge reset)

if (reset)

begin

// ALU_Out <- 32'h3FC

@(negedge sys_clk)

int_ack=0;   FS=5'h15;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = 5'b0;

state = RESET;

end

else

case (state)

FETCH:

if (int_ack==0 & (intr==1 & psi==1)) // Recieve Interrupt Signal

begin //*** new interrupt pending; prepare for ISR ***

// control word assignments for "deasserting" everything

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = INTR_1;

end

else  // No Interrupt Signal involved

begin //*** no new interrupt pending; fetch and instruction ***

if((int_ack==1 & intr==1) || (psi==1 & intr==0)) int_ack=1'b0;

// control word assignments for IR <- iM[PC]; PC <- PC+4

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_1_1;

{im_cs, im_rd, im_wr} = 3'b1_1_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = DECODE;

end

RESET:

begin

// control word assignments for $sp <- ALU_Out(32'h3FC)

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_11_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

DECODE:

begin

@(negedge sys_clk)

// check for MIPS format

// [000000][rs][rt][rd][shmt][func] - R type

if (CPU.iu.IR_out[31:26] == 6'h0)

begin

// it is an R-type format

// control word assignments: RS <- $rs, RT <- $rt (default)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

// check for function for R type

case (CPU.iu.IR_out[5:0])

6'h00 :  state = SLL;

6'h02 :  state = SRL;

6'h03 :  state = SRA;

6'h08 :  state = JR;

6'h10 :  state = MFHI;

6'h12 :  state = MFLO;

6'h18 :  state = MULT;

6'h1A :  state = DIV;

6'h20 :  state = ADD;

6'h21 :  state = ADDU;

6'h22 :  state = SUB;

6'h23 :  state = SUBU;

6'h24 :  state = AND;

6'h25 :  state = OR;

6'h26 :  state = XOR;

6'h27 :  state = NOR;

6'h2A :  state = SLT;

6'h2B :  state = SLTU;

6'h0D :  state = BREAK;

6'h1F :  state = SETIE;

default: state = ILLEGAL_OP;

endcase

end // end of if for R-type Format

else if(CPU.iu.IR_out[31:26] == 6'h1F)

begin

// E-key is R-type format

// control word assignments: RS <- $rs, RT <- $rt (default)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

// check for function for E-key type

case (CPU.iu.IR_out[5:0])

6'h01  : state = ROTL;

6'h02  : state = ROTR;

6'h05  : state = CLR;

6'h07  : state = MOV;

6'h08  : state = NOP;

6'h09  : state = PUSH;

6'h0A  : state = POP;

default: state = ILLEGAL_OP;

endcase

end  // end of if E-key format

else

begin

// it is an I-type or J-type format

// [pppppp][rs][rt][16'b imme] - I type

// [pppppp][26'b imme] - J type

// control word assignments: RS <- $rs, RT <- DT(se_16)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_01_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

// Check opcode for I and J type

case (CPU.iu.IR_out[31:26])

6'h02 : state = J;

6'h03 : state = JAL;

6'h04 : state = BEQ;

6'h05 : state = BNE;

6'h06 : state = BLEZ;

6'h07 : state = BGTZ;

6'h08 : state = ADDI;

6'h0A : state = SLTI;

6'h0B : state = SLTIU;

6'h0C : state = ANDI;

6'h0D : state = ORI;

6'h0E : state = XORI;

6'h0F : state = LUI;

6'h23 : state = LW;

6'h2B : state = SW;

6'h1C : state = INPUT;

6'h1D : state = OUTPUT;

6'h1E : state = RETI;

6'h2C : state = BLT;

6'h2D : state = BGE;

6'h2E : state = DJNZ;

default: state = ILLEGAL_OP;

endcase

// Case of Branches

//    if T_Sel = 0, RT <- $rt

//    so IR[15:0] will be used to calculate

//    Branch address.

if(state == BEQ || state == BNE ||

state == BLEZ|| state == BGTZ||

state == BLT || state == BGE  )

T_Sel = 2'b00;

else

T_Sel = 2'b01;

end // end of else for I-type or J-type formats

end // end of DECODE

/***************************************************************************

R-type   Instruction

***************************************************************************/

SLL:

begin

// control word assignments: ALU_Out <- $rt << shamt

@(negedge sys_clk)

int_ack=0;  FS=5'h0C;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_alu;

end

SRL:

begin

// control word assignments: ALU_Out <- $rt >> shamt

@(negedge sys_clk)

int_ack=0;  FS=5'h0D;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_alu;

end

SRA:

begin

// control word assignments: ALU_Out <- $rt >> shamt

@(negedge sys_clk)

int_ack=0;  FS=5'h0E;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

JR:

begin

// control word assignments: PC <- [$rt]

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = JR_2;

end

JR_2:

begin

// control word assignments: PC <- [$rt]

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_1_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

MFHI:

begin

// control word assignments: R[$rd] <- Hi

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_00_00_0_001;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

MFLO:

begin

// control word assignments: R[$rd] <- Lo

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_00_00_0_010;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

MULT:

begin

// control word assignments: {Hi,Lo} <- R[$rs] * R[$rt]

@(negedge sys_clk)

int_ack=0;   FS=5'h1E;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_1_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = FETCH;

end

DIV:

begin

// control word assignments: Lo <- R[$rs] / R[$rt], Hi <- R[$rs] % R[$rt]

@(negedge sys_clk)

int_ack=0;   FS=5'h1F;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_1_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

ADD:

begin

// control word assignments: ALU_Out <- $rs + $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h02;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

ADDU:

begin

// control word assignments: ALU_Out <- $rs + $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h02;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

SUB:

begin

// control word assignments: ALU_Out <- $rs - $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h04;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

SUBU:

begin

// control word assignments:

@(negedge sys_clk)

// control word assignments: ALU_Out <- $rs - $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h05;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

AND:

begin

// control word assignments: ALU_Out <- $rs & $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h08;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_alu;

end

OR:

begin

// control word assignments: ALU_Out <- $rs | $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h09;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_alu;

end

XOR:

begin

// control word assignments: ALU_Out <- $rs ^ $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h0A;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_alu;

end

NOR:

begin

// control word assignments: ALU_Out <- ~($rs | $rt)

@(negedge sys_clk)

int_ack=0;   FS=5'h0B;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_alu;

end

SLT:

begin

// control word assignments: ALU_Out <- $rs < $rt ? 1 : 0

@(negedge sys_clk)

int_ack=0;   FS=5'h06;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

SLTU:

begin

// control word assignments: ALU_Out <- $rs < $rt ? 1 : 0

@(negedge sys_clk)

int_ack=0;   FS=5'h07;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

BREAK:

begin

$display("BREAK INSTRUCTION FETCHED %t",$time);

// control word assignments for "deasserting" everything

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

$display(" R E G I S T E R ' S  A F T E R  B R E A K");

$display(" ");

Dump_Registers; // task to output MIPS RegFile

$display(" ");

$display("time=%t M[3F0]=%h", $time, {CPU_Test_Module.DM.Mem[12'h3F0],

CPU_Test_Module.DM.Mem[12'h3F1],

CPU_Test_Module.DM.Mem[12'h3F2],

CPU_Test_Module.DM.Mem[12'h3F3]});

$display(" ");

Dump_Mem;

$display(" ");

$finish;

end

SETIE:

begin

// control word assignments: psi <- 1'b1

@(negedge sys_clk)

int_ack=0;   FS=5'h0;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {1'b1, psc, psv, psn, psz};

state = FETCH;

end

/***************************************************************************

****************          I-type   Instruction           ******************

***************************************************************************/

J:

begin

// ctrl word assignments for PC <- PC + signext(IR[25:0])<<2

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b01_1_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_100;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

JAL:

begin

// ctrl word assignments for PC <- PC + signext(IR[25:0])<<2

//                         R[ra]<- PC

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b01_1_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_10_00_0_100;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

BEQ:

begin

// ctrl word assignments for AluOut <- $rs - $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h04;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = BEQ_2;

end

BEQ_2:

begin

// ctrl word assignments for if(z==1), PC <- PC+signext(IR[15:0])<<2

int_ack=0;   FS=5'h00;

if(psz == 1) // rs - rt == 0

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b10_1_0_0;

else

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1    {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

BNE:

begin

// ctrl word assignments for AluOut <- $rs - $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h04;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = BNE_2;

end

BNE_2:

begin

// ctrl word assignments for if(z!=1), PC <- PC + signext(IR[15:0])<<2

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

if(psz == 0) // rs - rt != 0

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b10_1_0_0;

else

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

BLEZ:

begin

// ctrl word assignments for RS <- $rs, RT <- $zero

@(negedge sys_clk)

int_ack=0;   FS=5'h04;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = BLEZ_2;

end

BLEZ_2:

begin

// ctrl word assignments for if(RS<=0), PC <- PC+signext(IR[15:0])<<2

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

if(psn == 1 || psz == 1)

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b10_1_0_0;

else

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

BGTZ:

begin

// ctrl word assignments for if(RS >= 0), PC <- PC + signext(IR[15:0])<<2

@(negedge sys_clk)

int_ack=0;   FS=5'h04;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = BGTZ_2;

end

BGTZ_2:

begin

// ctrl word assignments for if(RS >= 0), PC <- PC + signext(IR[15:0])<<2

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

if(psn == 0 || psz == 1)

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b10_1_0_0;

else

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

#1  {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_imm;

end

ADDI:

begin

// control word assignments for ALU_Out <- $rs + $rt(se_16)

@(negedge sys_clk)

int_ack=0;   FS=5'h02;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_imm;

end

SLTI:

begin

// ctrl word assignments for ALU_Out <- if($rs < $rt(se_16)) 1:0

@(negedge sys_clk)

int_ack=0;   FS=5'h06;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_imm;

end

SLTIU:

begin

// ctrl word assignments for ALU_Out <- if($rs < $rt(se_16)) 1:0

@(negedge sys_clk)

int_ack=0;   FS=5'h07;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_imm;

end

ANDI:

begin

// ctrl word assignments for ALU_Out <- $rs & $rt(se_16)

@(negedge sys_clk)

int_ack=0;   FS=5'h16;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_imm;

end

ORI:

begin

// ctrl word assignments for ALU_Out <- $rs | {16'h0, RT[15:0]}

@(negedge sys_clk)

int_ack=0;   FS=5'h17;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_imm;

end

XORI:

begin

// ctrl word assignments for ALU_Out <- $rs ^ {16'h0, RT[15:0]}

@(negedge sys_clk)

int_ack=0;   FS=5'h18;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_imm;

end

LUI:

begin

// control word assignments for ALU_Out <- { RT[15:0], 16'h0}

@(negedge sys_clk)

int_ack=0;   FS=5'h19;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_imm;

end

LW:

begin

// control word assignments for ALU_Out <- $rs + $rt(se_16) "EA calc"

@(negedge sys_clk)

int_ack=0;   FS=5'h02;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = LW_2;

end

LW_2:

begin

// control word assignments for D_in <- M[ALU_Out]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_1_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_Din;

end

SW:

begin

// control word assignments for ALU_Out <- $rs + $rt(se_16) "EA calc"

@(negedge sys_clk)

int_ack=0;   FS=5'h02;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_mem;

end

WB_Din:

begin

// control word assignments for R[rt] <- D_in[M[ALUout]]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_01_00_0_011;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

WB_alu:

begin

// control word assignments for R[rd] <- ALU_Out

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

WB_imm:

begin

// control word assignments for R[rt] <- ALU_Out

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_01_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

WB_mem:

begin

// control word assignments for M[ ALU_Out(rs+se_16)] <- RT(rt)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_0_1;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

/***************************************************************************

****************          E-key   Instruction           ******************

***************************************************************************/

INPUT:

begin

// control word assignments for ALU_Out <- $rs + $rt(se_16) "EA calc"

@(negedge sys_clk)

int_ack=0;   FS=5'h02;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = INPUT_2;

end

INPUT_2:

begin

// control word assignments for D_in <- IOM[ALU_Out]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b1_1_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = WB_Din;

end

OUTPUT:

begin

// control word assignments for ALU_Out <- $rs + $rt(se_16) "EA calc"

@(negedge sys_clk)

int_ack=0;   FS=5'h02;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = OUTPUT_2;

end

OUTPUT_2:

begin

// control word assignments for IOM[ALU_Out] <- D_in

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b1_0_1;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

RETI:

begin

// Pass $sp value to ALU_Out as an Effective Address

// control word assignments for ALU_Out <- RS($sp)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = RETI_2;

end

RETI_2:

begin

// POP flag status to D_in

// control word assignments for D_in <- DM[ALU_Out($sp)]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_1_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = RETI_3;

end

RETI_3:

begin

// POP flag status from D_in to current flag status

//     and increment $sp

// control word assignments for flags <- D_in[4:0]

//                              ALU_Out <- ALU_Out($sp)+4

@(negedge sys_clk)

int_ack=0;   FS=5'h11;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {sp_flags_in};

state = RETI_4;

end

RETI_4:

begin

// POP Returning Address(ra) to D_in before load it to PC

//     and increment $sp

// control word assignments for D_in <- DM[ALU_Out($sp+4)]

//                              ALU_Out <- ALU_Out($sp+4)+4

@(negedge sys_clk)

int_ack=0;   FS=5'h11;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_1_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = RETI_5;

end

RETI_5:

begin

// Load Return Address from Memory to PC register

//     and hold the $sp

// control word assignments for PC <- D_in(PC),

//                              ALU_Out <- ALU_Out($sp+8)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_1_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_011;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = RETI_6;

end

RETI_6:

begin

// Write back the current $sp in regfile

// control word assignments for R[$sp] <- ALU_Out($sp+8)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_11_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

ROTL:

begin

// control word assignments

@(negedge sys_clk)

int_ack=0;   FS=5'h1A;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

ROTR:

begin

// control word assignments

@(negedge sys_clk)

int_ack=0;   FS=5'h1B;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = WB_alu;

end

BLT:

begin

// control word assignments ALU_Out <- $rs - $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h04;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = BLT_2;

end

BLT_2:

begin

// control word assignments

//     if(n==1)  PC <- PC + {sign_ext(ir[15:0]),00}

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

if(psn == 1)

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b10_1_0_0;

else

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

BGE:

begin

// control word assignments ALU_Out <- $rs - $rt

@(negedge sys_clk)

int_ack=0;   FS=5'h04;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = BGE_2;

end

BGE_2:

begin

// control word assignments

// if(n==0 || z==1)  PC <- PC + {sign_ext(ir[15:0]),00}

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

if(psn == 0 || psz == 1)

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b10_1_0_0;

else

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

CLR:

begin

// control word assignments ALU_Out <- 0x0000;

@(negedge sys_clk)

int_ack=0;   FS=5'h13;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = CLR_2;

end

CLR_2:

begin

// control word assignments R[$rt] <- ALU_out

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_01_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

DJNZ:

begin

// DJNZ is an I-type enhanced where $rs and $rt field

// should be the same because ALU's FS Dec is using S

// to decrement, but $rt field is used for writing back

// to register files32.

// control word assignments ALU_Out <- RS-1;

@(negedge sys_clk)

int_ack=0;   FS=5'h10;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = DJNZ_2;

end

DJNZ_2:

begin

// control word assignments R[$rt] <- ALU_Out;

//                          if(z==0) PC <- PC+IR[15:0]<<2

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

if(psz==0)

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b10_1_0_0;

else

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_01_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

MOV:

begin

// Move data from R[$rs] to R[$rt]

// control word assignments ALU_Out <- R[$rs]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = MOV_2;

end

MOV_2:

begin

// control word assignments R[$rt] <- ALU_out

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_01_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

NOP:

begin

// No Operation. Nothing change in a clock

// control word assignments ALU_Out <- ALU_Out

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

PUSH:

begin

// Pre-decrement PUSH DM[--$sp] <- $rt

// control word assignments RS <- R[$sp}; RT <- R[$rt]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b1_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = PUSH_2;

end

PUSH_2:

begin

// control word assignments ALU_Out <- RS(sp)-4; RT <- R[$rt]

@(negedge sys_clk)

int_ack=0;   FS=5'h12;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = PUSH_3;

end

PUSH_3:

begin

// control word assignments DM[ALU_Out] <- RT(rt);

//     and save SP pointer  ALU_Out <- ALU_Out($sp-4)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_0_1;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = PUSH_4;

end

PUSH_4:

begin

// control word assignments R[$sp] <- ALU_Out($sp-4)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_11_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

POP:

begin

// Post-increment POP   R[rt] <- DM[sp++]

// control word assignments RS <- R[$sp]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b1_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = POP_2;

end

POP_2:

begin

// control word assignments ALU_Out <- RS(sp)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = POP_3;

end

POP_3:

begin

// control word assignments D_in <- DM[ALU_Out];

//                          ALU_Out <- ALU_out(sp)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_1_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = POP_4;

end

POP_4:

begin

// control word assignments R[rt] <- D_in;

//                          ALU_Out <- ALU_Out(sp)+4

@(negedge sys_clk)

int_ack=0;   FS=5'h11;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_01_00_0_011;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, c, v, n, z};

state = POP_5;

end

POP_5:

begin

// control word assignments R[sp] <- ALU_Out(sp+4)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_11_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = FETCH;

end

/***************************************************************************

Other types Instruction             ***************************************************************************/

ILLEGAL_OP:

begin

$display("ILLEGAL OPCODE FETCHED %t",$time);

// control word assignments for "deasserting" everything

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

$display(" ");

$display("Memory:");

$display(" ");

Dump_Registers;

$display(" ");

Dump_PC_and_IR;

$finish;

end

INTR_1:

begin

// PC gets address of interrupt vector; Save PC in $ra

// control word assignments for RS <- R[$sp],

//                              R[$ra] <- PC

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_10_00_0_100;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b1_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = INTR_2;

end

INTR_2:

begin

// Pass $sp from RS to ALU_Out

// control word assignments for ALU_Out <- RS($sp)

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = INTR_3;

end

INTR_3:

begin

// Read address of ISR into D_in;

// control word assignments for D_in <- dM[ALU_Out(0x3FC)]

@(negedge sys_clk)

int_ack=0;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_1_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = INTR_4;

end

INTR_4:

begin

// Load PC with M[$sp], Pre-decrement $sp,

//     and prepare PUSH current PC on stack.

// control word assignments for PC <- D_in(DM[$sp]),

//                              ALU_Out <- ALU_Out-4,

//                              RT <- PC

@(negedge sys_clk)

int_ack=0;       FS=5'h12;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_1_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_10_0_011;

{dm_cs, dm_rd, dm_wr} = 3'b0_0_0;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = INTR_5;

end

INTR_5:

begin

// PUSH the current PC on stack, decrement $sp,

//      and prepare PUSH current flags on stack.

// control word assignments for DM[$sp-4] <- RT(PC),

//                              ALU_Out <- ALU_Out($sp-4)-4,

//                              RT <- flags

@(negedge sys_clk)

int_ack=0;  FS=5'h12;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b0_00_11_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_0_1;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_1;

#1 {nsi, nsc, nsv, nsn, nsz} = {psi, psc, psv, psn, psz};

state = INTR_6;

end

INTR_6:

begin

// PUSH flags on stack, write current $sp in regfile,

//     and set interrupt acknowledge

// control word assignments for DM[ALU_Out($sp-8)] <- RT(flags)

//                              R[$sp] <- ALU_Out($sp-8)

//                              int_ack <- 1'b1;

@(negedge sys_clk)

int_ack=1;   FS=5'h00;

{pc_sel, pc_ld, pc_inc, ir_ld} = 5'b00_0_0_0;

{im_cs, im_rd, im_wr} = 3'b0_0_0;

{D_En, DA_sel, T_Sel, HILO_ld, Y_Sel} = 9'b1_11_00_0_000;

{dm_cs, dm_rd, dm_wr} = 3'b1_0_1;

{io_cs, io_rd, io_wr} = 3'b0_0_0;

{SP_Sel, S_Sel} = 2'b0_0;

#1 {nsi, nsc, nsv, nsn, nsz} = {1'b0, psc, psv, psn, psz};

state = FETCH;

end

endcase // end of FSM logic

//*******************************************************

//          Integer Register File Dump Task             *

//*******************************************************

task Dump_Registers;

begin

for(i = 0; i < 16; i = i + 1)

begin

$display ("t=%t   $r%0d = %h  ||  t=%t   $r%0d = %h",

$time, i, CPU_Test_Module.cpu.idp.regfile.reg_array[i],

$time, i+16, CPU_Test_Module.cpu.idp.regfile.reg_array[i+16]);

end

end

endtask

//*******************************************************

//             PC and IR Register Dump Task             *

//*******************************************************

task Dump_PC_and_IR;

begin

$display(" "); $display("PC Register:");

$display("t=%t PC=%h", $time, CPU_Test_Module.cpu.iu.PC.pc_out);

$display(" ");

$display("IR Register:");

$display("t=%t IR=%h", $time, CPU_Test_Module.cpu.iu.IR.q);

$display(" "); $display(" ");

end

endtask

//*******************************************************

//        Data Memory and I/O Memory Dump Task          *

//*******************************************************

task Dump_Mem;

begin

$display("_______Data Memory(DM)______________||_________IO Memory(IOM)_____");

for(i = 9'h0C0; i < 9'h100; i = i + 4) begin

X = {CPU_Test_Module.DM.Mem[i],

CPU_Test_Module.DM.Mem[i+1],

CPU_Test_Module.DM.Mem[i+2],

CPU_Test_Module.DM.Mem[i+3]};

Y = {CPU_Test_Module.IO.Mem[i],

CPU_Test_Module.IO.Mem[i+1],

CPU_Test_Module.IO.Mem[i+2],

CPU_Test_Module.IO.Mem[i+3]};

$display("t=%t DM[%h] = %h || t=%t IOM[%h] = %h",

$time, i, X, $time, i, Y);

end

end

endtask

endmodule

## ALU

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  ALU_32.v

* Project:    Lab_Assignment_1

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Date 9/3/2017

*

* Rev. No.1:  Version 1.1

* Rev. Date:  Date 11/19/2017

* update:     Add BarrialShifter32 to perform 0 to 31 bit shifting using

*                shamt bits from Instruction.

*             Update flag status for BarrialShifting.

*

* Rev. No.2:  Version 1.2

* Rev. Date:  Current Rev. Date 11/26/2017

* update:     Added Rotate Left (ROTL) and Rotate Right (ROTR)

*

* Purpose:    Top View of 32-bit Arithmatics Logic Unit(ALU). Used for

*             computing Arithmatic of two 32-bit inputs, register S and T.

*             Performs 30 different operations where 28 operations in

*             MIPS_32.v, multiplication in MPY_32.v, and division in DIV_32.v

*

* Notes:      ~|{Y1, Y2} Nor all bits in side the concatenated Y1 and Y2

*             if all bits are zero, the result is one (or true).

*

****************************************************************************/

module ALU_32(S, T, FS, shamt, Y_hi, Y_lo, C, V, N, Z);

input  [31:0]  S, T;                         // 32-bit inputs

input  [ 4:0]  FS;                           // Function Select

input  [ 4:0]  shamt;                        // shifting amount

output [31:0]  Y_hi;                         // Upper 32-bit of Output

output [31:0]  Y_lo;                         // Lower 32-bit of Output

output         C, V, N, Z;                   // Flag Status

wire   mips_v, mips_c, mul_z, mip_z, div_z;  // Temporary flag status

wire   [31:0]   mips_y                    ;  // Y Output from MIPS_32

wire   [31:0]   mul_hi, mul_lo            ;  // Y Output from MPY_32

wire   [31:0]   div_rem, div_qout         ;  // Y Output from DIV_32

wire   [31:0]   shifted_T                 ;  // Y Output from BrrialShifter

wire   shifted_C, shifted_z;                 // Temporary shifting flags

parameter  ADDU = 5'h03,

SUBU = 5'h05,

MUL  = 5'h1E,

DIV  = 5'h1F,

SLL  = 5'h0C,

SRL  = 5'h0D,

SRA  = 5'h0E,

ROTL = 5'h1A,

ROTR = 5'h1B;

// Multiplication

// return a64-bit result of multiplying S and T

//             (S, T, _______Y_______ )

MPY_32 alu_mul (S, T, {mul_hi, mul_lo});

// Division

// return 32-bit results where Y_hi = remainder and Y_lo = qoutian

//             (S, T, __rem__, __qout__)

DIV_32 alu_div (S, T, div_rem, div_qout);

// Million Instructions Per Second (MIPS)

// return a 32-bit result(Y), overflow flag(v), and carry flag(c)

//              (S, T, FS, __Y___, __V___, __C___)

MIPS_32 alu_mips(S, T, FS, mips_y, mips_v, mips_c);

// Barrial Shifter

//    shifting 32-bit input with shift amount

//                 (T, shamt, type,         Y,         C)

BarrialShifter32 bs(T, shamt,   FS, shifted_T, shifted_C);

// Multiplexer to assign 64-bit outputs

// If MULtiplication, return a 64-bit output.

// Else if DIVision,  return a remainder and qoutian.

// Else               return 32-bit zeros and 32-bit result.

assign {Y_hi, Y_lo} = (FS == MUL)? { mul_hi , mul_lo  }:

(FS == DIV)? { div_rem, div_qout}:

(FS == SLL ||

FS == SRL ||

FS == SRA ||

FS == ROTL||

FS == ROTR  )?{32'b0, shifted_T}:

{32'b0, mips_y   };

// Checking and assigning temporary the Zero flag

// ~| is nor reduction operand where

// the all output bits will be Nor

// and return 1 if all bits are zero.

// Return 0 if any bit is one.

assign mul_z   = ~|{Y_hi, Y_lo},

div_z   = (T == 32'b0)? 1'bz : ~|{Y_lo},

mip_z   = ~|{Y_lo},

shift_z = ~|{Y_lo};

// Multiplexer to assign the Zero flag output

// assign z flag according to Function Select(FS)

assign Z = (FS == MUL)?   mul_z:

(FS == DIV)?   div_z:

(FS == SLL ||

FS == SRL ||

FS == SRA ||

FS == ROTL||

FS == ROTR  )? shift_z:

mip_z;

// Multiplexer to assign Negative flag

// Assign n flag according to the Most Significant Bit(MSB)

assign N = (FS == MUL) ? Y_hi[31]:

(FS == DIV) ? Y_lo[31]:

(FS == ADDU)?     1'b0:

(FS == SUBU)?     1'b0:

Y_lo[31];

// Multiplexer to assign Overflow flag

// *Note: Multiplication and Division

// are not affected over flow flag

assign V = (FS == MUL ||

FS == DIV ||

FS == SLL ||

FS == SRL ||

FS == SRA ||

FS == ROTL||

FS == ROTR  )? 1'bx: mips_v;

// Multiplexer to assign Carry flag

// *Note: Multiplication and Division

// are not affected carry out flag

assign C = (FS == MUL)? 1'bx:

(FS == DIV)? 1'bx:

(FS == SLL ||

FS == SRL ||

FS == SRA ||

FS == ROTL||

FS == ROTR  )? shifted_C: mips_c;

endmodule

## Barrell Shifter

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  BarrialShifter32.v

* Project:    MIPS_Final_project

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Date 11/13/2017

*

* Rev. No.1:  Version 1.1

* Rev. Date:  Current Rev. Date 11/19/2017

* update:     Correct case statement for type and shamt

*

* Purpose:    Perform Shifting 32-bit input with shifting amount(shamt)

*             in different type of shifting method(type) and output back

*             to ALU_32

*

* Notes:      There are 5 modes but only 3 in used (SLL,SRL,SRA)

*             SLL: type = 5'h0C

*             SRL: type = 5'h0D

*             SRA: type = 5'h0E

*

****************************************************************************/

module BarrialShifter32(T, shamt, type, Y, C);

input       [4:0]   type;           // Function Select from MCU

input       [4:0]   shamt;          // Shifting amount from IR[10:6]

input       [31:0]  T;              // Data input for shifiting

output reg          C;              // Carry flag

output reg  [31:0]  Y;              // Shifted output

always@(*)

case(type)

5'h0C: // SLL

case(shamt)

5'd0:  {C,Y} = {1'b0, T};

5'd1:  {C,Y} = {T[31], T[30:0],  1'b0};

5'd2:  {C,Y} = {T[30], T[29:0],  2'b0};

5'd3:  {C,Y} = {T[29], T[28:0],  3'b0};

5'd4:  {C,Y} = {T[28], T[27:0],  4'b0};

5'd5:  {C,Y} = {T[27], T[26:0],  5'b0};

5'd6:  {C,Y} = {T[26], T[25:0],  6'b0};

5'd7:  {C,Y} = {T[25], T[24:0],  7'b0};

5'd8:  {C,Y} = {T[24], T[23:0],  8'b0};

5'd9:  {C,Y} = {T[23], T[22:0],  9'b0};

5'd10: {C,Y} = {T[22], T[21:0], 10'b0};

5'd11: {C,Y} = {T[21], T[20:0], 11'b0};

5'd12: {C,Y} = {T[20], T[19:0], 12'b0};

5'd13: {C,Y} = {T[19], T[18:0], 13'b0};

5'd14: {C,Y} = {T[18], T[17:0], 14'b0};

5'd15: {C,Y} = {T[17], T[16:0], 15'b0};

5'd16: {C,Y} = {T[16], T[15:0], 16'b0};

5'd17: {C,Y} = {T[15], T[14:0], 17'b0};

5'd18: {C,Y} = {T[14], T[13:0], 18'b0};

5'd19: {C,Y} = {T[13], T[12:0], 19'b0};

5'd20: {C,Y} = {T[12], T[11:0], 20'b0};

5'd21: {C,Y} = {T[11], T[10:0], 21'b0};

5'd22: {C,Y} = {T[10], T[ 9:0], 22'b0};

5'd23: {C,Y} = {T[ 9], T[ 8:0], 23'b0};

5'd24: {C,Y} = {T[ 8], T[ 7:0], 24'b0};

5'd25: {C,Y} = {T[ 7], T[ 6:0], 25'b0};

5'd26: {C,Y} = {T[ 6], T[ 5:0], 26'b0};

5'd27: {C,Y} = {T[ 5], T[ 4:0], 27'b0};

5'd28: {C,Y} = {T[ 4], T[ 3:0], 28'b0};

5'd29: {C,Y} = {T[ 3], T[ 2:0], 29'b0};

5'd30: {C,Y} = {T[ 2], T[ 1:0], 30'b0};

5'd31: {C,Y} = {T[ 1], T[0],    31'b0};

endcase

5'h0D: // SRL

case(shamt)

5'd0:  {C,Y} = {1'b0, T};

5'd1:  {C,Y} = {T[ 0],  1'b0, T[31: 1]};

5'd2:  {C,Y} = {T[ 1],  2'b0, T[31: 2]};

5'd3:  {C,Y} = {T[ 2],  3'b0, T[31: 3]};

5'd4:  {C,Y} = {T[ 3],  4'b0, T[31: 4]};

5'd5:  {C,Y} = {T[ 4],  5'b0, T[31: 5]};

5'd6:  {C,Y} = {T[ 5],  6'b0, T[31: 6]};

5'd7:  {C,Y} = {T[ 6],  7'b0, T[31: 7]};

5'd8:  {C,Y} = {T[ 7],  8'b0, T[31: 8]};

5'd9:  {C,Y} = {T[ 8],  9'b0, T[31: 9]};

5'd10: {C,Y} = {T[ 9], 10'b0, T[31:10]};

5'd11: {C,Y} = {T[10], 11'b0, T[31:11]};

5'd12: {C,Y} = {T[11], 12'b0, T[31:12]};

5'd13: {C,Y} = {T[12], 13'b0, T[31:13]};

5'd14: {C,Y} = {T[13], 14'b0, T[31:14]};

5'd15: {C,Y} = {T[14], 15'b0, T[31:15]};

5'd16: {C,Y} = {T[15], 16'b0, T[31:16]};

5'd17: {C,Y} = {T[16], 17'b0, T[31:17]};

5'd18: {C,Y} = {T[17], 18'b0, T[31:18]};

5'd19: {C,Y} = {T[18], 19'b0, T[31:19]};

5'd20: {C,Y} = {T[19], 20'b0, T[31:20]};

5'd21: {C,Y} = {T[20], 21'b0, T[31:21]};

5'd22: {C,Y} = {T[21], 22'b0, T[31:22]};

5'd23: {C,Y} = {T[22], 23'b0, T[31:23]};

5'd24: {C,Y} = {T[23], 24'b0, T[31:24]};

5'd25: {C,Y} = {T[24], 25'b0, T[31:25]};

5'd26: {C,Y} = {T[25], 26'b0, T[31:26]};

5'd27: {C,Y} = {T[26], 27'b0, T[31:27]};

5'd28: {C,Y} = {T[27], 28'b0, T[31:28]};

5'd29: {C,Y} = {T[28], 29'b0, T[31:29]};

5'd30: {C,Y} = {T[29], 30'b0, T[31:30]};

5'd31: {C,Y} = {T[30], 31'b0, T[31]   };

endcase

5'h0E: // SRA

case(shamt)

5'd0:  {C,Y} = {1'b0, T};

5'd1:  {C,Y} = {T[ 0], T[31], { 1{T[31]}}, T[30: 1]};

5'd2:  {C,Y} = {T[ 1], T[31], { 2{T[31]}}, T[30: 2]};

5'd3:  {C,Y} = {T[ 2], T[31], { 3{T[31]}}, T[30: 3]};

5'd4:  {C,Y} = {T[ 3], T[31], { 4{T[31]}}, T[30: 4]};

5'd5:  {C,Y} = {T[ 4], T[31], { 5{T[31]}}, T[30: 5]};

5'd6:  {C,Y} = {T[ 5], T[31], { 6{T[31]}}, T[30: 6]};

5'd7:  {C,Y} = {T[ 6], T[31], { 7{T[31]}}, T[30: 7]};

5'd8:  {C,Y} = {T[ 7], T[31], { 8{T[31]}}, T[30: 8]};

5'd9:  {C,Y} = {T[ 8], T[31], { 9{T[31]}}, T[30: 9]};

5'd10: {C,Y} = {T[ 9], T[31], {10{T[31]}}, T[30:10]};

5'd11: {C,Y} = {T[10], T[31], {11{T[31]}}, T[30:11]};

5'd12: {C,Y} = {T[11], T[31], {12{T[31]}}, T[30:12]};

5'd13: {C,Y} = {T[12], T[31], {13{T[31]}}, T[30:13]};

5'd14: {C,Y} = {T[13], T[31], {14{T[31]}}, T[30:14]};

5'd15: {C,Y} = {T[14], T[31], {15{T[31]}}, T[30:15]};

5'd16: {C,Y} = {T[15], T[31], {16{T[31]}}, T[30:16]};

5'd17: {C,Y} = {T[16], T[31], {17{T[31]}}, T[30:17]};

5'd18: {C,Y} = {T[17], T[31], {18{T[31]}}, T[30:18]};

5'd19: {C,Y} = {T[18], T[31], {19{T[31]}}, T[30:19]};

5'd20: {C,Y} = {T[19], T[31], {20{T[31]}}, T[30:20]};

5'd21: {C,Y} = {T[20], T[31], {21{T[31]}}, T[30:21]};

5'd22: {C,Y} = {T[21], T[31], {22{T[31]}}, T[30:22]};

5'd23: {C,Y} = {T[22], T[31], {23{T[31]}}, T[30:23]};

5'd24: {C,Y} = {T[23], T[31], {24{T[31]}}, T[30:24]};

5'd25: {C,Y} = {T[24], T[31], {25{T[31]}}, T[30:25]};

5'd26: {C,Y} = {T[25], T[31], {26{T[31]}}, T[30:26]};

5'd27: {C,Y} = {T[26], T[31], {27{T[31]}}, T[30:27]};

5'd28: {C,Y} = {T[27], T[31], {28{T[31]}}, T[30:28]};

5'd29: {C,Y} = {T[28], T[31], {29{T[31]}}, T[30:29]};

5'd30: {C,Y} = {T[29], T[31], {30{T[31]}}, T[30]   };

5'd31: {C,Y} = {T[30], T[31], {31{T[31]}}          };

endcase

5'h1A: // Rotate Left

case(shamt)

5'd0:  {C,Y} = T;

5'd1:  {C,Y} = {T[30:0], T[31]};

5'd2:  {C,Y} = {T[29:0], T[31:30]};

5'd3:  {C,Y} = {T[28:0], T[31:29]};

5'd4:  {C,Y} = {T[27:0], T[31:28]};

5'd5:  {C,Y} = {T[26:0], T[31:27]};

5'd6:  {C,Y} = {T[25:0], T[31:26]};

5'd7:  {C,Y} = {T[24:0], T[31:25]};

5'd8:  {C,Y} = {T[23:0], T[31:24]};

5'd9:  {C,Y} = {T[22:0], T[31:23]};

5'd10: {C,Y} = {T[21:0], T[31:22]};

5'd11: {C,Y} = {T[20:0], T[31:21]};

5'd12: {C,Y} = {T[19:0], T[31:20]};

5'd13: {C,Y} = {T[18:0], T[31:19]};

5'd14: {C,Y} = {T[17:0], T[31:18]};

5'd15: {C,Y} = {T[16:0], T[31:17]};

5'd16: {C,Y} = {T[15:0], T[31:16]};

5'd17: {C,Y} = {T[14:0], T[31:15]};

5'd18: {C,Y} = {T[13:0], T[31:14]};

5'd19: {C,Y} = {T[12:0], T[31:13]};

5'd20: {C,Y} = {T[11:0], T[31:12]};

5'd21: {C,Y} = {T[10:0], T[31:11]};

5'd22: {C,Y} = {T[9:0],  T[31:10]};

5'd23: {C,Y} = {T[8:0],  T[31:9]};

5'd24: {C,Y} = {T[7:0],  T[31:8]};

5'd25: {C,Y} = {T[6:0],  T[31:7]};

5'd26: {C,Y} = {T[5:0],  T[31:6]};

5'd27: {C,Y} = {T[4:0],  T[31:5]};

5'd28: {C,Y} = {T[3:0],  T[31:4]};

5'd29: {C,Y} = {T[2:0],  T[31:3]};

5'd30: {C,Y} = {T[1:0],  T[31:2]};

5'd31: {C,Y} = {T[0],    T[31:1]};

endcase

5'h1B: // Rotate Right

case(shamt)

5'd0:  {C,Y} = T;

5'd1:  {C,Y} = {T[0],    T[31:1]};

5'd2:  {C,Y} = {T[1:0],  T[31:2]};

5'd3:  {C,Y} = {T[2:0],  T[31:3]};

5'd4:  {C,Y} = {T[3:0],  T[31:4]};

5'd5:  {C,Y} = {T[4:0],  T[31:5]};

5'd6:  {C,Y} = {T[5:0],  T[31:6]};

5'd7:  {C,Y} = {T[6:0],  T[31:7]};

5'd8:  {C,Y} = {T[7:0],  T[31:8]};

5'd9:  {C,Y} = {T[8:0],  T[31:9]};

5'd10: {C,Y} = {T[9:0],  T[31:10]};

5'd11: {C,Y} = {T[10:0], T[31:11]};

5'd12: {C,Y} = {T[11:0], T[31:12]};

5'd13: {C,Y} = {T[12:0], T[31:13]};

5'd14: {C,Y} = {T[13:0], T[31:14]};

5'd15: {C,Y} = {T[14:0], T[31:15]};

5'd16: {C,Y} = {T[15:0], T[31:16]};

5'd17: {C,Y} = {T[16:0], T[31:17]};

5'd18: {C,Y} = {T[17:0], T[31:18]};

5'd19: {C,Y} = {T[18:0], T[31:19]};

5'd20: {C,Y} = {T[19:0], T[31:20]};

5'd21: {C,Y} = {T[20:0], T[31:21]};

5'd22: {C,Y} = {T[21:0], T[31:22]};

5'd23: {C,Y} = {T[22:0], T[31:23]};

5'd24: {C,Y} = {T[23:0], T[31:24]};

5'd25: {C,Y} = {T[24:0], T[31:25]};

5'd26: {C,Y} = {T[25:0], T[31:26]};

5'd27: {C,Y} = {T[26:0], T[31:27]};

5'd28: {C,Y} = {T[27:0], T[31:28]};

5'd29: {C,Y} = {T[28:0], T[31:29]};

5'd30: {C,Y} = {T[29:0], T[31:30]};

5'd31: {C,Y} = {T[30:0], T[31]};

endcase

endcase

endmodule

## ALU MIPS

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  MIPS_32.v

* Project:    Lab_Assignment_1

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 9/3/2017

*

* Purpose:    Processes two 32-bit input(S and T) according to

*             Function Select(FS) and then return a 32-bit result,

*             Overflow flag(V), and Carry flag(C).

*

* Notes:      For an OP code that does not affect overflow flag (V) or

*             Carry flag (C) will output as 'x'

*             U - unsigned

*             I - Immediate

*

****************************************************************************/

module MIPS_32(S, T, FS, Y, V, C);

input      [31:0] S, T;      // Input Registers

input      [ 4:0] FS;        // Function Select(OP code)

output reg [31:0] Y;         // Output below 32th bit

output reg        V, C;      // Overflow and Carry Flags

integer   int_s, int_t;      // parse interger of S and T

// Look-up table for Function Select(FS)

parameter PASS_S = 5'h00,  AND  = 5'h08,  INC     = 5'h0F,

PASS_T = 5'h01,  OR   = 5'h09,  DEC     = 5'h10,

ADD    = 5'h02,  XOR  = 5'h0A,  INC4    = 5'h11,

ADDU   = 5'h03,  NOR  = 5'h0B,  DEC4    = 5'h12,

SUB    = 5'h04,  SLL  = 5'h0C,  ZEROS   = 5'h13,

SUBU   = 5'h05,  SRL  = 5'h0D,  ONES    = 5'h14,

SLT    = 5'h06,  SRA  = 5'h0E,  SP_INIT = 5'h15,

SLTU   = 5'h07,  ANDI = 5'h16,

MUL    = 5'h1E,  ORI  = 5'h17,

DIV    = 5'h1F,  XORI = 5'h18,

LUI  = 5'h19;

// Parse S and T to integer

always@(*) begin

int_s <= S;

int_t <= T;

end

// Combination Logic for both Arithmetic and Logic Operations

always@(*) begin

case(FS)

PASS_S: {V,C,Y} = {2'bxx, S};                // Pass S

PASS_T: {V,C,Y} = {2'bxx, T};                // Pass T

ADD   : begin                                // Adding

{C,Y} = S + T;

V  = {  S[31] &  T[31] & ~Y[31] }|     // (+) + (+) = (-)

{ ~S[31] & ~T[31] &  Y[31] };     // (-) + (-) = (+)

end

ADDU  : begin                                // Adding Unsigned

{C,Y} = S + T;

V  = C;

end

SUB   : begin                                // Subtracting

{C,Y} = S - T;

V  = {  S[31] & ~T[31] & ~Y[31] }|     // (+) - (-) = (-)

{ ~S[31] &  T[31] &  Y[31] };     // (-) - (+) = (+)

end

SUBU  : begin                                // Subtracting Unsigned

{C,Y} = S - T;

V  = C;

end

SLT   : begin                                // Set Less Than

{V,C} = 2'bxx;

Y = int_s < int_t;                // comparing signed integers

end

SLTU  : begin                        // Set Less Than Unsigned

{V,C} = 2'bxx;

Y  = S < T;                       // comparing bits

end

AND   : {V,C,Y} = {2'bxx, S & T};    // And

OR    : {V,C,Y} = {2'bxx, S | T};    // Or

XOR   : {V,C,Y} = {2'bxx, S ^ T};    // Xor

NOR   : {V,C,Y} = {2'bxx, ~(S | T)}; // Nor

SLL   : begin                        // Shift Left Logic

{V,C} = {1'bx, T[31]};

Y = {T[30:0], 1'b0};

end

SRL   : begin                        // Shift Right Logic

{V,C} = {1'bx, T[0]};

Y = {1'b0, T[31:1]};

end

SRA   : begin                        // Shift Right Arithmetic

{V,C} = {1'bx, T[0]};

Y = {T[31], T[31:1]};

end

ANDI  : {V,C,Y} = {2'bxx, S & {16'b0, T[15:0]}};

ORI   : {V,C,Y} = {2'bxx, S | {16'b0, T[15:0]}};

XORI  : {V,C,Y} = {2'bxx, S ^ {16'b0, T[15:0]}};

LUI   : {V,C,Y} = {2'bxx, T[15:0], 16'b0};   // Load Upper Immediate

INC   : begin                                // Increment (by one)

{C,Y} = S + 1;

V  = {  S[31] & ~Y[31] }|              // (+) + 1 = (-)

{ ~S[31] &  Y[31] };              // (-) + 1 = (+)

end

DEC   : begin                                // Decrement (by one)

{C,Y} = S - 1;

V  = {  S[31] & ~Y[31] }|              // (+) - 1 = (-)

{ ~S[31] &  Y[31] };              // (-) - 1 = (+)

end

INC4  : begin                                // Increment (by four)

{C,Y} = S + 4;

V  = {  S[31] & ~Y[31] }|              // (+) + 4 = (-)

{ ~S[31] &  Y[31] };              // (-) + 4 = (+)

end

DEC4  : begin                                // Decrement (by four)

{C,Y} = S - 4;

V  = {  S[31] & ~Y[31] }|              // (+) - 4 = (-)

{ ~S[31] &  Y[31] };              // (-) - 4 = (+)

end

ZEROS : {V,C,Y} = {2'bxx, 32'h0};             // Set to Zero

ONES  : {V,C,Y} = {2'bxx, 32'hFFFFFFFF};      // Set to One

SP_INIT: {V,C,Y} = {2'bxx, 32'h3FC};          // Stack Pointer Init

default: {V,C,Y} = {2'bxx, S};                // Pass S

endcase

end

endmodule

## ALU Multiplier

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  MPY_32.v

* Project:    Lab_Assignment_1

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 9/3/2017

*

* Purpose:    Multiply two 32-bit inputs and return a 64-bit results.

*

* Notes:      Two inputs are needed to be casted as integers before

*             being operated.

*

****************************************************************************/

module MPY_32(S, T, Y);

input      [31:0] S, T;       // 32-bit inputs

output reg [63:0]    Y;       // 64-bit result

integer   int_s, int_t;       // integer for each inputs

// Cast S and T into integers

always@(*) begin

int_s <= S;

int_t <= T;

end

// Multiply the two integers

always@(*)

Y = int_s * int_t;

endmodule

## ALU Divider

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  DIV_32.v

* Project:    Lab_Assignment_1

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 9/3/2017

*

* Purpose:    Divide two 32-bit inputs and return remainder and qoutian

*

* Notes:      Two inputs are needed to be casted as integers.

*             division is qoutationg, or p reslt.

*             modulus has

*

****************************************************************************/

module DIV_32(S, T, rem, qout);

input      [31:0]      S, T;     // 32-bit inputs

output reg [31:0] rem, qout;     // Remainder and Qoutian outputs

integer int_s, int_t;            // Integer of each inputs

// Parse S and T into integers

always@(*) begin

int_s <= S;

int_t <= T;

end

// Divide the two integers

always@(*) begin

qout = int_s / int_t;      // Quotient

rem  = int_s % int_t;      // Remainder

end

endmodule

## Instruction Unit

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  Instruction_Unit.v

* Project:    Final Project

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Date 10/10/2017

*

* Rev. No.1:  Version 1.1

* Rev. Date:  Date 10/24/2017

* update:     Get rid of [31:0] D_in, and Add pc_sel for PC_MUX

*

* Rev. No.2:  Version 1.2

* Rev. Date:  Current Rev. Date 11/19/2017

* update:     Adjusts Instruction Memory address pin to only

*             get 12 bits from PC_out since the project's memory is small

*             and there is no need for using more than 12 bits.

*

* Purpose:    Instruction Unit(CPU_IU) is a memory holding instructions

*             could be accessed by an address from the Program Counter(PC),

*             then sending the called instruction through data output(D_OUT).

*

****************************************************************************/

module Instruction_Unit(

input               clk, reset,     // On-board clock and reset signal

input     pc_ld, pc_inc, ir_ld,     // PC and IR load, and PC increment

input     im_cs,  im_wr, im_rd,     // Instruction Memory (im)

//    Chip Select, Write, and Read

input     [1:0]         pc_sel,     // pc select for PC_MUX

input    [31:0]          PC_in,     // PC Input

output   [31:0]         PC_out,     // PC Output

output   [31:0]  IR_out, SE_16      // Instruction Output

//    and Sign-Extension output

);

wire     [31:0]         IM_out;     // Instruction Memory Output

wire     [31:0]         pc_mux;     // PC_MUX for pc_in

//PC_MUX

//if pc_sel = 0, select PC_in (pc+4)

//pc_sel = 1, select Jump instruction PC <- {pc[31:28], 26bit-imme, 00}

//pc_sel = 2, select Branch instruction PC <- PC + {sign_ext(ir[15:0]),00}

assign pc_mux = (pc_sel == 2'b01) ? {PC_out[31:28],IR_out[25:0],2'b00} :// Jump

(pc_sel == 2'b10) ? {PC_out + {SE_16[29:0],2'b00}}

:// Branch

PC_in;

// PC+4

// Program Counter(PC)

pc PC (.clk(clk),                   // On-board clock

.reset(reset),               // Reset

.pc_ld(pc_ld),               // Load

.pc_inc(pc_inc),             // Increment

.pc_in(pc_mux),              // Input

.pc_out(PC_out)              // Output

);

// Instrcution Memory

Memory IM (.clk(clk),               // On-board clock

.cs(im_cs),              // Chip-select

.wr(im_wr),              // Write Enable

.rd(im_rd),              // Read Enable

.Addr({20'b0,PC_out[11:0]}),// Address from PC

.D_In(32'b0),            // Deasserted

.D_Out(IM_out)           // Instruction output

);

// Instruction Register

//

reg32_w_ld IR (.clk(clk),           // On-board clock

.reset(reset),       // Reset

.load(ir_ld),        // Load

.d(IM_out),          // Data input

.q(IR_out)           // Data output

);

// Sign Extend 16 bits

//    Takes 15th bit of IR_out, creates 16-bit of the 15th bit,

//    then concatenates with lower bits of IR_out

assign SE_16 = { {16{IR_out[15]}}, IR_out[15:0]};

endmodule

## IDP

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  IDP.v

* Project:    Final Project

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.1

* Rev. Date:  Date 10/6/2017

* Update:     Add four registers as scratch register;

*             RS, RT, ALU_out_r, D_in.

*

* Rev. No.:   Version 1.2

* Rev. Date:  Rev. 1.2 Date 10/16/2017

* Update:     Added DA_Sel to select either D_Addr or T_Addr

*

* Rev. No.:   Version 1.3

* Rev. Date:  1.3 Date 10/24/2017

* Update:     Added another bit for DA_Sel to select

*             D_Addr, T_Addr, $ra(return Address), or $sp(stack Pointer)

*

* Rev. No.:   Version 1.4

* Rev. Date:  Current Rev. 1.3 Date 11/21/2017

* Update:     Added SP_Sel, S_Sel, sp_flags_in and sp_flags_out for

*             performing Stack Memory

*

* Purpose:    Integer Data Path (IDP) performs arithmetics calculation

*             from 32-bit data inside a register file and output the result.

*

* Notes:      IDP receives control from a control unit(CU) to control

*             the data flow (data path) in IDP by sending source

*             register addresses to the register file and execute the

*             arithmetic operation in Arithmetic Logic Unit (ALU), then

*             selects output by Y-MUX.

*

****************************************************************************/

module IDP(

input              clk,   reset,    // On-board clock and reset signal

input             D_En, HILO_ld,    // Load enable for regfiles

input   [1:0]            DA_Sel,    // DA_MUX

input   [1:0]             T_Sel,    // T-MUX select

input   [2:0]             Y_Sel,    // Y-MUX select

input   [4:0]                FS,    // ALU function select

input   [4:0]             shamt,    // Shifting amount

input   [4:0]            D_Addr,    // regfile Write address

input   [4:0]   S_Addr,  T_Addr,    // regfile Source addresses

input   [31:0]      DT,      DY,    // Data inputs

input   [31:0]            pc_in,    // Program Counter input

input             S_Sel, SP_Sel,    // S_MUX and Stack Pointer Select

input   [4:0]       sp_flags_in,    // input stack flags

output  [4:0]      sp_flags_out,    // output stack flags

output               C, V, N, Z,    // Status flags from ALU

output  [31:0]  ALU_OUT,  D_OUT     // Outputs

);

wire    [31:0]        S,      T;    // Wires S to ALU and T to T-MUX

wire    [31:0]            rt_in;    // A wire from T-MUX to RT

wire    [31:0]     Y_hi,   Y_lo;    // Wires to HILO regfile and Y-MUX

wire    [31:0]       hi,     lo;    // Outputs from HILO regfile

wire    [31:0]       rs,     rt;    // Scratch

wire    [31:0]  alu_out,   d_in;    //    registers  wires

wire    [4:0]            DA_Out;    // Wire to D_Addr

wire    [4:0]        SP_MUX_Out;    // SP_MUX output to regfile S_Addr

wire    [31:0]        S_MUX_Out;    // S_MUX output to ALU S input

// Register File

//    a memory unit for CPU

regfile32 regfile (.clk(clk),          // On-board clock

.reset(reset),      // Reset signal

.D_En(D_En),        // Load Enable

.D_Addr(DA_Out),    // Writing address

.S_Addr(SP_MUX_Out),// Source S address

.T_Addr(T_Addr),    // Source T address

.D_in(ALU_OUT),     // Data input

.S(S),              // S output

.T(T)               // T output

);

// DA_MUX

//    if DA_Sel = 1, choose T_Addr(rt)

//    if DA_Sel = 2, choose $ra (register 31)

//    if DA_Sel = 3, choose $sp (register 29)

//    otherwise,     choose D_Addr(rd) by default

assign DA_Out = (DA_Sel == 2'b01)? T_Addr:   // $rt

(DA_Sel == 2'b10)?  5'h1F:   // 31 for $ra

(DA_Sel == 2'b11)?  5'h1D:   // 29 for $sp

D_Addr;   // $rd default

// SP_MUX

//    selecting either S_Addr from Instuction or

//    Stack Pointer's address($sp at 5'h1D)

assign  SP_MUX_Out = (SP_Sel)? 5'h1D: S_Addr;

// S_MUX

//    selecting either rs or alu_out

//    and load it into S input of ALU

assign  S_MUX_Out = (S_Sel)? alu_out: rs;

// T-MUX

//    selecting an output to input T of ALU

//    if T_sel is 1, assign DT (SE_16 from Instruction Unit)

//                2, PC_in

//                3, flags from Data Memory before Interrupted

//    otherwise assign T from regfile32

assign rt_in = (T_Sel==2'b01) ? DT:

(T_Sel==2'b10) ? pc_in:

(T_Sel==2'b11) ? {27'b0, sp_flags_in}: // status flags from mem

T;

// Arithmetic Logic Unit (ALU)

//    performs Arithmetic and logic calculation

//    base on Function Select(FS)

ALU_32 alu (.S(S_MUX_Out),         // S input

.T(rt),                // T input

.FS(FS),               // Function Select(OPcode)

.shamt(shamt),         // Shift Amount

.Y_hi(Y_hi),           // upper part of 64-bit result

.Y_lo(Y_lo),           // lower part of 64-bit resutl

.C(C),                 // Carry flag

.V(V),                 // Overflow flag

.N(N),                 // Negative flag

.Z(Z)                  // Zero flag

);

// Hi-Lo register

//    holds result from multiplication

//    and division.

//    clk   -  On-board clock

//    reset -  Reset signal

//    load  -  Load enable

//    d     -  upper part of 64-bit input

//    q     -  lower part of 64-bit input

//             clk, reset,    load,    d,  q

reg32_w_ld HI (clk, reset, HILO_ld, Y_hi, hi),

LO (clk, reset, HILO_ld, Y_lo, lo);

// Scratch register

//    holding value for the next clock

//    clk   -  On-board clock

//    reset -  Reset signal

//    d     -  32-bit input

//    q     -  32-bit output

//              clk, reset,     d,       q

reg32_no_ld  RS (clk, reset,     S,      rs),

RT (clk, reset, rt_in,      rt),

ALU_out (clk, reset,  Y_lo, alu_out),

D_in (clk, reset,    DY,    d_in);

// Wire flags status to MCU

//    when IDP got flags from Memory(D_in)

assign sp_flags_out = d_in[4:0];

// Y-MUX

//    selecting output (ALU_OUT) from Y_Sel

assign ALU_OUT = (Y_Sel == 3'b001) ?    hi:  // upper part of 64-bit result

(Y_Sel == 3'b010) ?    lo:  // lower part of 64-bit result

(Y_Sel == 3'b011) ?  d_in:  // Data Input

(Y_Sel == 3'b100) ? pc_in:  // Program Counter Input

alu_out;  // Pass alu_out by default

// Assign D_OUT from the output of RT register

assign D_OUT = rt;

endmodule

## Data Memory

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  Data_Memory.v

* Project:    Final Project

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 10/7/2017

*

* Purpose:    a 4096x8 byte addressable memory in big endian format

*             storing data and instructions that could be accessed

*             by addressing Address(Addr) as a base of the location

*             of the memory and the add 1,2,3 for the rest of the base

*             (see details at Read section).

*

* Notes:      For the purposes of this lab Addr will be used only

*             12 bits out of 32 bits.

*

****************************************************************************/

module Data_Memory(clk, dm_cs, dm_wr, dm_rd, Addr, DM_In, DM_Out);

input                  clk;  // clock signal

input  dm_cs, dm_wr, dm_rd;  // Data memory "ChipSelect", "Write", "Read"

input  [31:0]         Addr;  // Address to the memory

input  [31:0]        DM_In;  // Data input

output [31:0]       DM_Out;  // Data output

reg    [7:0]  Mem [0:4095];  // 4Kx8 array of registers

// Read data from Memory

//    Addr is the ALU_out which contained

//    register in-direct address.

//       for instance, Addr <- ALU_out_r(0x0000_000F) // register 15

//       Addr is located at register 15 in this memory

//       and then read the value of base(reg15)

//       which are 0x03C to 0x03F [4 addresses at the time.]

//    If the memory is not being read, D_Out will output HighImpedance(z).

assign DM_Out = (dm_cs & dm_rd & !dm_wr)? {Mem[Addr+0],

Mem[Addr+1],

Mem[Addr+2],

Mem[Addr+3]} : 32'hz;

// Write data on Memory

//    Writing data on memory is synchronous with the clock(clk)

//    and only if Chip Select(cs) and Write Enable(wr) are HIGH.

//    Otherwise, the memory cannot be written.

always@(posedge clk)

if(dm_cs & dm_wr & !dm_rd)       // Write Data Input into the Memory

{Mem[Addr+0],

Mem[Addr+1],

Mem[Addr+2],

Mem[Addr+3]} <= DM_In;

else begin                       // Keep the same value

Mem[Addr+0] <= Mem[Addr+0];

Mem[Addr+1] <= Mem[Addr+1];

Mem[Addr+2] <= Mem[Addr+2];

Mem[Addr+3] <= Mem[Addr+3];

end

endmodule

## IO Memory

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  IO_Memory.v

* Project:    Final Project

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 10/7/2017

*

* Purpose:    a 4096x8 byte addressable memory in big endian format

*             storing data and instructions that could be accessed

*             by addressing Address(Addr) as a base of the location

*             of the memory and the add 1,2,3 for the rest of the base

*             (see details at Read section).

*

* Notes:      For the purposes of this lab Addr will be used only

*             12 bits out of 32 bits.

*

****************************************************************************/

module IO_Memory(clk, cs, wr, rd, int_r, int_ack, Addr, IO_In, IO_Out);

input             clk;       // clock signal

input      cs, wr, rd;       // "ChipSelect", "Write", "Read"

input         int_ack;       // interrupt acknowledge output

output  reg     int_r;       // interrupt request input

input  [31:0]    Addr;       // Address to the memory

input  [31:0]   IO_In;       // Data input

output [31:0]  IO_Out;       // Data output

reg    [7:0]  Mem [0:4095];  // 4Kx8 array of registers

// Send interrupt request to CPU

//    after 1000/10 ns = 100 clock cycles

//    Even though interrupt request is set,

//    if instruction SETIE is not called,

//    CPU will not be interrupted.

initial begin

int_r=0;

#1000 int_r=1;

@(posedge int_ack) int_r=0;

end

// Read data from Memory

//    Addr is the ALU_out which contained

//    register in-direct address.

//       for instance, Addr <- ALU_out_r(0x0000_000F) // register 15

//       Addr is located at register 15 in this memory

//       and then read the value of base(reg15)

//       which are 0x03C to 0x03F [4 addresses at the time.]

//    If the memory is not being read, D_Out will output HighImpedance(z).

assign IO_Out = (cs & rd & !wr)? {Mem[Addr+0],

Mem[Addr+1],

Mem[Addr+2],

Mem[Addr+3]} : 32'hz;

// Write data on Memory

//    Writing data on memory is synchronous with the clock(clk)

//    and only if Chip Select(cs) and Write Enable(wr) are HIGH.

//    Otherwise, the memory cannot be written.

always@(posedge clk)

if(cs & wr & !rd)                // Write Data Input into the Memory

{Mem[Addr+0],

Mem[Addr+1],

Mem[Addr+2],

Mem[Addr+3]} <= IO_In;

else begin                       // Keep the same value

Mem[Addr+0] <= Mem[Addr+0];

Mem[Addr+1] <= Mem[Addr+1];

Mem[Addr+2] <= Mem[Addr+2];

Mem[Addr+3] <= Mem[Addr+3];

end

endmodule

## PC

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  pc.v

* Project:    Lab_Assignment_5

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 10/10/2017

*

* Purpose:    Program Counter(PC) register holds an address of Instruction

*             Memory(CPU_IU). As pc_ld is high, the address is loaded

*             with data input(pc_in), and the address is increased by 4

*             (or move to the next memory location) as pc_inc is high.

*             Otherwise, there is no changed.

*

* Notes:      If both pc_ld and pc_inc are high, there is no changed.

*

****************************************************************************/

module pc(clk, reset, pc_ld, pc_inc, pc_in, pc_out);

input          clk, reset;             // on-board clock and reset

input       pc_ld, pc_inc;             // Load and Increment signals

input      [31:0]   pc_in;             // Address in

output reg [31:0]  pc_out;             // Address out

always @ (posedge clk, posedge reset)

if(reset) pc_out <= 32'b0;          // reset

else                                // Update pc_out on active edge clk

case({pc_ld, pc_inc})

2'b01: pc_out <= pc_out + 4;  // increment

2'b10: pc_out <= pc_in;       // load

default: pc_out <= pc_out;    // default

endcase

endmodule

## Memory

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  Memory.v

* Project:    Lab_Assignment_5

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 10/11/2017

*

* Purpose:    a 4096x8 byte addressable memory in big endian format

*             storing data and instructions that could be accessed

*             by addressing Address(Addr) as a base of the location

*             of the memory and the add 1,2,3 for the rest of the base

*             (see details at Read section).

*

* Notes:      For the purposes of this lab Addr will be used only

*             12 bits out of 32 bits.

*

****************************************************************************/

module Memory(clk, cs, wr, rd, Addr, D_In, D_Out);

input                  clk;  // clock signal

input           cs, wr, rd;  // Data memory "ChipSelect", "Write", "Read"

input  [31:0]         Addr;  // Address to the memory

input  [31:0]         D_In;  // Data input

output [31:0]        D_Out;  // Data output

reg    [7:0]  Mem [0:4095];  // 4Kx8 array of registers

// Read data from Memory

//    Addr is the ALU_out which contained

//    register in-direct address.

//       for instance, Addr <- ALU_out_r(0x0000_000F) // register 15

//       Addr is located at register 15 in this memory

//       and then read the value of base(reg15)

//       which are 0x03C to 0x03F [4 addresses at the time.]

//    If the memory is not being read, D_Out will output HighImpedance(z).

assign D_Out = (cs & rd & !wr)? {Mem[Addr+0],

Mem[Addr+1],

Mem[Addr+2],

Mem[Addr+3]} : 32'bz;

// Write data on Memory

//    Writing data on memory is synchronous with the clock(clk)

//    and only if Chip Select(cs) and Write Enable(wr) are HIGH.

//    Otherwise, the memory cannot be written.

always@(posedge clk)

if(cs & wr & !rd)                // Write Data Input into the Memory

{Mem[Addr+0],

Mem[Addr+1],

Mem[Addr+2],

Mem[Addr+3]} = D_In;

else begin                       // Keep the same value

Mem[Addr+0] = Mem[Addr+0];

Mem[Addr+1] = Mem[Addr+1];

Mem[Addr+2] = Mem[Addr+2];

Mem[Addr+3] = Mem[Addr+3];

end

endmodule

## 32-bit loadable register

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  reg32_w_ld.v

* Project:    Lab_Assignment_1

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Date 9/29/2017

*

* Rev. No.:   Version 2.0

* Rev. Date:  Current Rev. Date 10/10/2017

*    Notes:   - change port list to be reuseable as 32-bit register module

*

* Purpose:    A register to hold 32-bit result until load enable(load)

*             allows data input(d) to overwritten previous value.

*

* Notes:      This register operates on the active edge of the clock

*             signal input (clk). If the load enable (load) is HIGH,

*             the register will hold the new data input instead.

*

****************************************************************************/

module reg32_w_ld (clk, reset, load, d, q);

input      clk, reset;                // on-board clock, and reset signal

input            load;                // load enable

input      [31:0]   d;                // data inputs

output reg [31:0]   q;                // data outputs

always @ (posedge clk, posedge reset)

if(reset) q <= 32'b0;   else     // reset

if(load)  q <=     d;            // assign new value

else      q <=     q;            // keep previous value

endmodule

## 32-bit Non-Loadable Regiter

`timescale 1ns / 1ps

/* **************************** C E C S  4 4 0 ******************************

*

* File Name:  reg32_no_ld.v

* Project:    Lab_Assignment_4

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  10/7/2017

*

* Rev. No.:   Version 1.1

* Rev. Date:  Current Rev. 10/10/2017

*    Notes:   - filename changed

*             - comments added

*

* Purpose:    A 32-bit memory holding 32-bit value and output it

*             on next active clock.

*

* ***************************************************************************/

module reg32_no_ld(clk, reset, d, q);

input     clk, reset;      // on-board clock, and reset signal

input      [31:0]  d;      // data inputs

output reg [31:0]  q;      // data outputs

always@(posedge clk, posedge reset)

if(reset) q <= 32'b0;   // reset

else      q <= d;       // assign new value

endmodule

## Register File

`timescale 1ns / 1ps

/* **************************** C E C S  4 4 0 ******************************

*

* File Name:  regfile32.v

* Project:    Lab_Assignment_2

* Designer:   Chanartip Soonthornwan, Jonathan Shihata

* Email:      Chanartip.Soonthornwan@gmail.com, JonnyShihata@gmail.com

* Rev. No.:   Version 1.0

* Rev. Date:  Current Rev. Date 9/14/2017

*

* Purpose:    Represents an array of register that has 32x32 Width

*             (32 registers) and Depth(32 bits each). Being able to save

*             32-bit input(D_in) and any register(specified by D_Addr) when

*             Data Write Enable(D_En) is HIGH. Also being able to output

*             two 32-bit data through S and T paths where S and T will

*             get the data from S_Addr and T_Addrrespectively.

*

* Notes:      (*)At Write Section, if Data Write Enable(D_En) is HIGH,

*             it will write the input data into the register at D_Addr,

*             but it will notwrite on $r0 because Address $zero is restrict

*             for READ-ONLY.Therefore, when D_En is HIGH and D_Addr is

*             at $r0, the program will fall into Not changing content.

*

* ***************************************************************************/

module regfile32(clk, reset, D_En, D_Addr, S_Addr, T_Addr, D_in, S, T);

input             clk, reset;          // Clock and Reset Signal

input                   D_En;          // Data Write Enable

input  [ 4:0]         D_Addr;          // Address of register to write

input  [ 4:0] S_Addr, T_Addr;          // Address of register to read

input  [31:0]           D_in;          // 32-bit input data

output [31:0]           S, T;          // 32-bit read data

reg    [31:0]  reg_array [31:0];       // 32 of 32-bit registers

// Write Section

always@(posedge clk, posedge reset) begin

if(reset)                           // Get reset signal

reg_array[0] <= 32'h0;           //    assign $r0 to zeroes

else if(D_En && D_Addr != 5'h0)     // Load Input data

reg_array[D_Addr] <= D_in;       //    to a register that is not $r0

else                                // Not changing content

reg_array[D_Addr] <= reg_array[D_Addr]; // (*)

end

// Read Section

//    S and T read get 32-bit data from registers

//    at S_Addr and T_addr respectively

assign   S = reg_array[S_Addr];

assign   T = reg_array[T_Addr];

endmodule

# Verification

R1 <- 0x012345678

R2 <- 0x876543210

/* Instruction Memory Module 1 */

@0

3c 01 12 34

34 21 56 78

R3 <- R1

3c 02 87 65

34 42 43 21

BEQ R2, R1. Should not branch to no_eq

00 01 18 20

10 22 00 01

BEQ R3, R1. should branch to yes_eq.

10 23 00 03

no_eq:

Set fail flag.
R14 <- 0xFFFFFFFF
break

3c 0e ff ff

35 ce ff ff

00 00 00 0d

Set pass flag
R14 <-00000000

yes_eq:

00 00 70 20

14 23 00 01

BNE R3, R1. Should not branch to no_ne.
BNE R2, R1. Should branch to yest_ne.

14 22 00 03

no_ne:

3c 0f ff ff

35 ef ff ff

Set fail flag.
R15 <- 0xFFFFFFFF
break.

00 00 00 0d

yes_ne:

00 00 78 20

Set pass flag
R15 <- 00000000

3c 0d 10 01

35 ad 00 c0

ad a1 00 00

Set DM pointer
R13 <- 0x100100c0
DM[R13] <- R1
break

00 00 00 0d

/* Verification 1 */

Loaded R1-R3

BREAK INSTRUCTION FETCHED 625.0 ns

R E G I S T E R ' S A F T E R B R E A K

t= 631.0 ns $r0 = 00000000 || t= 631.0 ns $r16 = xxxxxxxx

t= 631.0 ns $r1 = 12345678 || t= 631.0 ns $r17 = xxxxxxxx

t= 631.0 ns $r2 = 87654321 || t= 631.0 ns $r18 = xxxxxxxx

DM pointer

t= 631.0 ns $r3 = 12345678 || t= 631.0 ns $r19 = xxxxxxxx

t= 631.0 ns $r4 = xxxxxxxx || t= 631.0 ns $r20 = xxxxxxxx

t= 631.0 ns $r5 = xxxxxxxx || t= 631.0 ns $r21 = xxxxxxxx

t= 631.0 ns $r6 = xxxxxxxx || t= 631.0 ns $r22 = xxxxxxxx

t= 631.0 ns $r7 = xxxxxxxx || t= 631.0 ns $r23 = xxxxxxxx

Pass flags for BEQ and BNE

t= 631.0 ns $r8 = xxxxxxxx || t= 631.0 ns $r24 = xxxxxxxx

t= 631.0 ns $r9 = xxxxxxxx || t= 631.0 ns $r25 = xxxxxxxx

t= 631.0 ns $r10 = xxxxxxxx || t= 631.0 ns $r26 = xxxxxxxx

t= 631.0 ns $r11 = xxxxxxxx || t= 631.0 ns $r27 = xxxxxxxx

t= 631.0 ns $r12 = xxxxxxxx || t= 631.0 ns $r28 = xxxxxxxx

t= 631.0 ns $r13 = 100100c0 || t= 631.0 ns $r29 = 000003fc

Stored by R3

t= 631.0 ns $r14 = 00000000 || t= 631.0 ns $r30 = xxxxxxxx

t= 631.0 ns $r15 = 00000000 || t= 631.0 ns $r31 = xxxxxxxx

time= 631.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t= 631.0 ns DM[000000c0] = 12345678 || t= 631.0 ns IOM[000000c0] = xxxxxxxx

t= 631.0 ns DM[000000c4] = xxxxxxxx || t= 631.0 ns IOM[000000c4] = xxxxxxxx

t= 631.0 ns DM[000000c8] = xxxxxxxx || t= 631.0 ns IOM[000000c8] = xxxxxxxx

t= 631.0 ns DM[000000cc] = xxxxxxxx || t= 631.0 ns IOM[000000cc] = xxxxxxxx

t= 631.0 ns DM[000000d0] = xxxxxxxx || t= 631.0 ns IOM[000000d0] = xxxxxxxx

t= 631.0 ns DM[000000d4] = xxxxxxxx || t= 631.0 ns IOM[000000d4] = xxxxxxxx

t= 631.0 ns DM[000000d8] = xxxxxxxx || t= 631.0 ns IOM[000000d8] = xxxxxxxx

t= 631.0 ns DM[000000dc] = xxxxxxxx || t= 631.0 ns IOM[000000dc] = xxxxxxxx

t= 631.0 ns DM[000000e0] = xxxxxxxx || t= 631.0 ns IOM[000000e0] = xxxxxxxx

t= 631.0 ns DM[000000e4] = xxxxxxxx || t= 631.0 ns IOM[000000e4] = xxxxxxxx

t= 631.0 ns DM[000000e8] = xxxxxxxx || t= 631.0 ns IOM[000000e8] = xxxxxxxx

t= 631.0 ns DM[000000ec] = xxxxxxxx || t= 631.0 ns IOM[000000ec] = xxxxxxxx

t= 631.0 ns DM[000000f0] = xxxxxxxx || t= 631.0 ns IOM[000000f0] = xxxxxxxx

t= 631.0 ns DM[000000f4] = xxxxxxxx || t= 631.0 ns IOM[000000f4] = xxxxxxxx

t= 631.0 ns DM[000000f8] = xxxxxxxx || t= 631.0 ns IOM[000000f8] = xxxxxxxx

t= 631.0 ns DM[000000fc] = xxxxxxxx || t= 631.0 ns IOM[000000fc] = xxxxxxxx

Stopped at time : 631 ns

R1 <- 0xFFFFFFFF
R2 <- 0x10
R15 <- 100100C0

/* Instruction Memory Module 2 */

@0

3c 01 ff ff

34 21 ff ff

20 02 00 10

3c 0f 10 01

35 ef 00 c0

- Logical Shift Right R1 1 bit.

- Store in DM[R15]

- Increment R15 by 4

- Decrement loop counter R2

- If R2 is zero, jump to exit. Else, branch to top

- Break.

Top:

00 01 08 42

ad e1 00 00

21 ef 00 04

20 42 ff ff

14 40 ff fb

08 10 00 0c

00 00 00 0d

Exit:

Set pass flag. R14 <- 5A5A3C3C

Then break.

3c 0e 5a 5a

35 ce 3c 3c

00 00 00 0d

/* Verification 2 */

BREAK INSTRUCTION FETCHED 3575.0 ns

Result after shifting by one 16 times

R E G I S T E R ' S A F T E R B R E A K

t=3581.0 ns $r0 = 00000000 || t=3581.0 ns $r16 = xxxxxxxx

t=3581.0 ns $r1 = 0000ffff || t=3581.0 ns $r17 = xxxxxxxx

Loop counter ended

t=3581.0 ns $r2 = 00000000 || t=3581.0 ns $r18 = xxxxxxxx

t=3581.0 ns $r3 = xxxxxxxx || t=3581.0 ns $r19 = xxxxxxxx

t=3581.0 ns $r4 = xxxxxxxx || t=3581.0 ns $r20 = xxxxxxxx

t=3581.0 ns $r5 = xxxxxxxx || t=3581.0 ns $r21 = xxxxxxxx

t=3581.0 ns $r6 = xxxxxxxx || t=3581.0 ns $r22 = xxxxxxxx

t=3581.0 ns $r7 = xxxxxxxx || t=3581.0 ns $r23 = xxxxxxxx

t=3581.0 ns $r8 = xxxxxxxx || t=3581.0 ns $r24 = xxxxxxxx

t=3581.0 ns $r9 = xxxxxxxx || t=3581.0 ns $r25 = xxxxxxxx

t=3581.0 ns $r10 = xxxxxxxx || t=3581.0 ns $r26 = xxxxxxxx

Passed flag.

t=3581.0 ns $r11 = xxxxxxxx || t=3581.0 ns $r27 = xxxxxxxx

t=3581.0 ns $r12 = xxxxxxxx || t=3581.0 ns $r28 = xxxxxxxx

DM pointer after looping

t=3581.0 ns $r13 = xxxxxxxx || t=3581.0 ns $r29 = 000003fc

t=3581.0 ns $r14 = 5a5a3c3c || t=3581.0 ns $r30 = xxxxxxxx

t=3581.0 ns $r15 = 10010100 || t=3581.0 ns $r31 = xxxxxxxx

time=3581.0 ns M[3F0]=xxxxxxxx

Result from logical shifting 0xFFFFFFFF

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=3581.0 ns DM[000000c0] = 7fffffff || t=3581.0 ns IOM[000000c0] = xxxxxxxx

t=3581.0 ns DM[000000c4] = 3fffffff || t=3581.0 ns IOM[000000c4] = xxxxxxxx

t=3581.0 ns DM[000000c8] = 1fffffff || t=3581.0 ns IOM[000000c8] = xxxxxxxx

t=3581.0 ns DM[000000cc] = 0fffffff || t=3581.0 ns IOM[000000cc] = xxxxxxxx

t=3581.0 ns DM[000000d0] = 07ffffff || t=3581.0 ns IOM[000000d0] = xxxxxxxx

t=3581.0 ns DM[000000d4] = 03ffffff || t=3581.0 ns IOM[000000d4] = xxxxxxxx

t=3581.0 ns DM[000000d8] = 01ffffff || t=3581.0 ns IOM[000000d8] = xxxxxxxx

t=3581.0 ns DM[000000dc] = 00ffffff || t=3581.0 ns IOM[000000dc] = xxxxxxxx

t=3581.0 ns DM[000000e0] = 007fffff || t=3581.0 ns IOM[000000e0] = xxxxxxxx

t=3581.0 ns DM[000000e4] = 003fffff || t=3581.0 ns IOM[000000e4] = xxxxxxxx

t=3581.0 ns DM[000000e8] = 001fffff || t=3581.0 ns IOM[000000e8] = xxxxxxxx

t=3581.0 ns DM[000000ec] = 000fffff || t=3581.0 ns IOM[000000ec] = xxxxxxxx

t=3581.0 ns DM[000000f0] = 0007ffff || t=3581.0 ns IOM[000000f0] = xxxxxxxx

t=3581.0 ns DM[000000f4] = 0003ffff || t=3581.0 ns IOM[000000f4] = xxxxxxxx

t=3581.0 ns DM[000000f8] = 0001ffff || t=3581.0 ns IOM[000000f8] = xxxxxxxx

t=3581.0 ns DM[000000fc] = 0000ffff || t=3581.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 3581 ns

R1 <- 0x8000FFFF
R2 <- 0x10
R15 <- 100100C0

/* Instruction Memory Module 3 */

@0

3c 01 80 00

34 21 ff ff

20 02 00 10

- Arithmetic Shift Right R1 1 bit.

- Store in DM[R15]

- Increment R15 by 4

- Decrement loop counter R2

- If R2 is zero, jump to exit. Else, branch to top

- Break.

3c 0f 10 01

Top:

35 ef 00 c0

00 01 08 43

ad e1 00 00

21 ef 00 04

20 42 ff ff

14 40 ff fb

08 10 00 0c

00 00 00 0d

Exit:

Set pass flag. R14 <- 5A5A3C3C

Then break.

3c 0e 5a 5a

35 ce 3c 3c

00 00 00 0d

/* Verification 3 */

BREAK INSTRUCTION FETCHED 3575.0 ns

Result after shifting by one 16 times

R E G I S T E R ' S A F T E R B R E A K

t=3581.0 ns $r0 = 00000000 || t=3581.0 ns $r16 = xxxxxxxx

t=3581.0 ns $r1 = ffff8000 || t=3581.0 ns $r17 = xxxxxxxx

t=3581.0 ns $r2 = 00000000 || t=3581.0 ns $r18 = xxxxxxxx

Loop counter ended

t=3581.0 ns $r3 = xxxxxxxx || t=3581.0 ns $r19 = xxxxxxxx

t=3581.0 ns $r4 = xxxxxxxx || t=3581.0 ns $r20 = xxxxxxxx

t=3581.0 ns $r5 = xxxxxxxx || t=3581.0 ns $r21 = xxxxxxxx

t=3581.0 ns $r6 = xxxxxxxx || t=3581.0 ns $r22 = xxxxxxxx

t=3581.0 ns $r7 = xxxxxxxx || t=3581.0 ns $r23 = xxxxxxxx

t=3581.0 ns $r8 = xxxxxxxx || t=3581.0 ns $r24 = xxxxxxxx

t=3581.0 ns $r9 = xxxxxxxx || t=3581.0 ns $r25 = xxxxxxxx

t=3581.0 ns $r10 = xxxxxxxx || t=3581.0 ns $r26 = xxxxxxxx

Passed flag.

t=3581.0 ns $r11 = xxxxxxxx || t=3581.0 ns $r27 = xxxxxxxx

t=3581.0 ns $r12 = xxxxxxxx || t=3581.0 ns $r28 = xxxxxxxx

DM pointer after looping

t=3581.0 ns $r13 = xxxxxxxx || t=3581.0 ns $r29 = 000003fc

t=3581.0 ns $r14 = 5a5a3c3c || t=3581.0 ns $r30 = xxxxxxxx

t=3581.0 ns $r15 = 10010100 || t=3581.0 ns $r31 = xxxxxxxx

time=3581.0 ns M[3F0]=xxxxxxxx

Result from arithemetic shifting 0xFFFFFFFF

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=3581.0 ns DM[000000c0] = c0007fff || t=3581.0 ns IOM[000000c0] = xxxxxxxx

t=3581.0 ns DM[000000c4] = e0003fff || t=3581.0 ns IOM[000000c4] = xxxxxxxx

t=3581.0 ns DM[000000c8] = f0001fff || t=3581.0 ns IOM[000000c8] = xxxxxxxx

t=3581.0 ns DM[000000cc] = f8000fff || t=3581.0 ns IOM[000000cc] = xxxxxxxx

t=3581.0 ns DM[000000d0] = fc0007ff || t=3581.0 ns IOM[000000d0] = xxxxxxxx

t=3581.0 ns DM[000000d4] = fe0003ff || t=3581.0 ns IOM[000000d4] = xxxxxxxx

t=3581.0 ns DM[000000d8] = ff0001ff || t=3581.0 ns IOM[000000d8] = xxxxxxxx

t=3581.0 ns DM[000000dc] = ff8000ff || t=3581.0 ns IOM[000000dc] = xxxxxxxx

t=3581.0 ns DM[000000e0] = ffc0007f || t=3581.0 ns IOM[000000e0] = xxxxxxxx

t=3581.0 ns DM[000000e4] = ffe0003f || t=3581.0 ns IOM[000000e4] = xxxxxxxx

t=3581.0 ns DM[000000e8] = fff0001f || t=3581.0 ns IOM[000000e8] = xxxxxxxx

t=3581.0 ns DM[000000ec] = fff8000f || t=3581.0 ns IOM[000000ec] = xxxxxxxx

t=3581.0 ns DM[000000f0] = fffc0007 || t=3581.0 ns IOM[000000f0] = xxxxxxxx

t=3581.0 ns DM[000000f4] = fffe0003 || t=3581.0 ns IOM[000000f4] = xxxxxxxx

t=3581.0 ns DM[000000f8] = ffff0001 || t=3581.0 ns IOM[000000f8] = xxxxxxxx

t=3581.0 ns DM[000000fc] = ffff8000 || t=3581.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 3581 ns :

/* Instruction Memory Module 4 */

@0

R1 <- 0xFFFFFFFF
R2 <- 0x10
R15 <- 100100C0

3c 01 ff ff

34 21 ff ff

20 02 00 10

3c 0f 10 01

- Logical Shift Left R1 1 bit.

- Store in DM[R15]

- Increment R15 by 4

- Decrement loop counter R2

- Set R3 = 1 if R0 < R2  (R3=0 when R2<R0)

- Branch to Top if R3 = 1

- Break.

35 ef 00 c0

Top:

00 01 08 40

ad e1 00 00

21 ef 00 04

20 42 ff ff

00 02 18 2a

14 60 ff fa

08 10 00 0d

00 00 00 0d

Jump to Exit

Exit:

3c 0e 5a 5a

35 ce 3c 3c

Break. Should not be fetched.

00 00 00 0d

Set pass flag. R14 <- 5A5A3C3C

Then break.

Result after shifting by one 16 times

/* Verification 4 */

BREAK INSTRUCTION FETCHED 4215.0 ns

R E G I S T E R ' S A F T E R B R E A K

Loop counter ended

t=4221.0 ns $r0 = 00000000 || t=4221.0 ns $r16 = xxxxxxxx

t=4221.0 ns $r1 = ffff0000 || t=4221.0 ns $r17 = xxxxxxxx

R3 = 0 when R2 < R0

t=4221.0 ns $r2 = 00000000 || t=4221.0 ns $r18 = xxxxxxxx

t=4221.0 ns $r3 = 00000000 || t=4221.0 ns $r19 = xxxxxxxx

t=4221.0 ns $r4 = xxxxxxxx || t=4221.0 ns $r20 = xxxxxxxx

t=4221.0 ns $r5 = xxxxxxxx || t=4221.0 ns $r21 = xxxxxxxx

t=4221.0 ns $r6 = xxxxxxxx || t=4221.0 ns $r22 = xxxxxxxx

t=4221.0 ns $r7 = xxxxxxxx || t=4221.0 ns $r23 = xxxxxxxx

t=4221.0 ns $r8 = xxxxxxxx || t=4221.0 ns $r24 = xxxxxxxx

t=4221.0 ns $r9 = xxxxxxxx || t=4221.0 ns $r25 = xxxxxxxx

t=4221.0 ns $r10 = xxxxxxxx || t=4221.0 ns $r26 = xxxxxxxx

Passed flag.

t=4221.0 ns $r11 = xxxxxxxx || t=4221.0 ns $r27 = xxxxxxxx

t=4221.0 ns $r12 = xxxxxxxx || t=4221.0 ns $r28 = xxxxxxxx

DM pointer after looping

t=4221.0 ns $r13 = xxxxxxxx || t=4221.0 ns $r29 = 000003fc

t=4221.0 ns $r14 = 5a5a3c3c || t=4221.0 ns $r30 = xxxxxxxx

t=4221.0 ns $r15 = 10010100 || t=4221.0 ns $r31 = xxxxxxxx

time=4221.0 ns M[3F0]=xxxxxxxx

Result from logical shifting 0xFFFFFFFF

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=4221.0 ns DM[000000c0] = fffffffe || t=4221.0 ns IOM[000000c0] = xxxxxxxx

t=4221.0 ns DM[000000c4] = fffffffc || t=4221.0 ns IOM[000000c4] = xxxxxxxx

t=4221.0 ns DM[000000c8] = fffffff8 || t=4221.0 ns IOM[000000c8] = xxxxxxxx

t=4221.0 ns DM[000000cc] = fffffff0 || t=4221.0 ns IOM[000000cc] = xxxxxxxx

t=4221.0 ns DM[000000d0] = ffffffe0 || t=4221.0 ns IOM[000000d0] = xxxxxxxx

t=4221.0 ns DM[000000d4] = ffffffc0 || t=4221.0 ns IOM[000000d4] = xxxxxxxx

t=4221.0 ns DM[000000d8] = ffffff80 || t=4221.0 ns IOM[000000d8] = xxxxxxxx

t=4221.0 ns DM[000000dc] = ffffff00 || t=4221.0 ns IOM[000000dc] = xxxxxxxx

t=4221.0 ns DM[000000e0] = fffffe00 || t=4221.0 ns IOM[000000e0] = xxxxxxxx

t=4221.0 ns DM[000000e4] = fffffc00 || t=4221.0 ns IOM[000000e4] = xxxxxxxx

t=4221.0 ns DM[000000e8] = fffff800 || t=4221.0 ns IOM[000000e8] = xxxxxxxx

t=4221.0 ns DM[000000ec] = fffff000 || t=4221.0 ns IOM[000000ec] = xxxxxxxx

t=4221.0 ns DM[000000f0] = ffffe000 || t=4221.0 ns IOM[000000f0] = xxxxxxxx

t=4221.0 ns DM[000000f4] = ffffc000 || t=4221.0 ns IOM[000000f4] = xxxxxxxx

t=4221.0 ns DM[000000f8] = ffff8000 || t=4221.0 ns IOM[000000f8] = xxxxxxxx

t=4221.0 ns DM[000000fc] = ffff0000 || t=4221.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 4221 ns :

/* Instruction Memory Module 5 */

R1 <-  0xFFFFFFFF

R2 <-  -16

R15 <- 100100C0

@0

3c 01 ff ff

34 21 ff ff

20 02 ff f0

3c 0f 10 01

35 ef 00 c0

- Logical Shift Left R1 1 bit.

- Store in DM[R15]

- Increment R15 by 4

- Increment loop counter R2

- Set R3 = 1 if R2 < zero  (R3=0 when R2 > zero)

- Branch to Top if R3 = 1

- Break.

Top:

00 01 08 40

ad e1 00 00

21 ef 00 04

20 42 00 01

28 43 00 00

14 60 ff fa

08 10 00 0d

00 00 00 0d

Exit:

3c 0e 5a 5a

Jump to Exit

35 ce 3c 3c

00 00 00 0d

Set pass flag. R14 <- 5A5A3C3C

Then break.

Break. Should not be fetched.

/* Verification 5 */

BREAK INSTRUCTION FETCHED 4215.0 ns

Result after shifting by one 16 times

R E G I S T E R ' S A F T E R B R E A K

t=4221.0 ns $r0 = 00000000 || t=4221.0 ns $r16 = xxxxxxxx

Loop counter ended

t=4221.0 ns $r1 = ffff0000 || t=4221.0 ns $r17 = xxxxxxxx

t=4221.0 ns $r2 = 00000000 || t=4221.0 ns $r18 = xxxxxxxx

t=4221.0 ns $r3 = 00000000 || t=4221.0 ns $r19 = xxxxxxxx

R3 = 0 when R2 > zero

t=4221.0 ns $r4 = xxxxxxxx || t=4221.0 ns $r20 = xxxxxxxx

Result from logical shifting 0xFFFFFFFF

t=4221.0 ns $r5 = xxxxxxxx || t=4221.0 ns $r21 = xxxxxxxx

t=4221.0 ns $r6 = xxxxxxxx || t=4221.0 ns $r22 = xxxxxxxx

t=4221.0 ns $r7 = xxxxxxxx || t=4221.0 ns $r23 = xxxxxxxx

t=4221.0 ns $r8 = xxxxxxxx || t=4221.0 ns $r24 = xxxxxxxx

t=4221.0 ns $r9 = xxxxxxxx || t=4221.0 ns $r25 = xxxxxxxx

Passed flag.

t=4221.0 ns $r10 = xxxxxxxx || t=4221.0 ns $r26 = xxxxxxxx

t=4221.0 ns $r11 = xxxxxxxx || t=4221.0 ns $r27 = xxxxxxxx

t=4221.0 ns $r12 = xxxxxxxx || t=4221.0 ns $r28 = xxxxxxxx

t=4221.0 ns $r13 = xxxxxxxx || t=4221.0 ns $r29 = 000003fc

t=4221.0 ns $r14 = 5a5a3c3c || t=4221.0 ns $r30 = xxxxxxxx

DM pointer after looping

t=4221.0 ns $r15 = 10010100 || t=4221.0 ns $r31 = xxxxxxxx

time=4221.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=4221.0 ns DM[000000c0] = fffffffe || t=4221.0 ns IOM[000000c0] = xxxxxxxx

t=4221.0 ns DM[000000c4] = fffffffc || t=4221.0 ns IOM[000000c4] = xxxxxxxx

t=4221.0 ns DM[000000c8] = fffffff8 || t=4221.0 ns IOM[000000c8] = xxxxxxxx

t=4221.0 ns DM[000000cc] = fffffff0 || t=4221.0 ns IOM[000000cc] = xxxxxxxx

t=4221.0 ns DM[000000d0] = ffffffe0 || t=4221.0 ns IOM[000000d0] = xxxxxxxx

t=4221.0 ns DM[000000d4] = ffffffc0 || t=4221.0 ns IOM[000000d4] = xxxxxxxx

t=4221.0 ns DM[000000d8] = ffffff80 || t=4221.0 ns IOM[000000d8] = xxxxxxxx

t=4221.0 ns DM[000000dc] = ffffff00 || t=4221.0 ns IOM[000000dc] = xxxxxxxx

t=4221.0 ns DM[000000e0] = fffffe00 || t=4221.0 ns IOM[000000e0] = xxxxxxxx

t=4221.0 ns DM[000000e4] = fffffc00 || t=4221.0 ns IOM[000000e4] = xxxxxxxx

t=4221.0 ns DM[000000e8] = fffff800 || t=4221.0 ns IOM[000000e8] = xxxxxxxx

t=4221.0 ns DM[000000ec] = fffff000 || t=4221.0 ns IOM[000000ec] = xxxxxxxx

t=4221.0 ns DM[000000f0] = ffffe000 || t=4221.0 ns IOM[000000f0] = xxxxxxxx

t=4221.0 ns DM[000000f4] = ffffc000 || t=4221.0 ns IOM[000000f4] = xxxxxxxx

t=4221.0 ns DM[000000f8] = ffff8000 || t=4221.0 ns IOM[000000f8] = xxxxxxxx

t=4221.0 ns DM[000000fc] = ffff0000 || t=4221.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 4221 ns

/* Instruction Memory Module 6 */

R15 <- 0x10010000

R14 <- 0x100100C0

R13 <- 16

@0

3c 0f 10 01

35 ef 00 00

3c 0e 10 01

35 ce 00 c0

20 0d 00 10

Load R01 to R12 from Data Memory based on where R15 points at and increment by 4 on each Load instruction

8d e1 00 04

8d e2 00 08

8d e3 00 0c

8d e4 00 10

8d e5 00 14

8d e6 00 18

8d e7 00 1c

- Use R17 as a temporary register loading data from memory to memory; R17 <- DM[R15]

- Store DM[R14] <- R17

- Increment both pointer R14 and R15 by 4

- Decrement loop counter R13 by 1

- Branch to mem2mem until R13=0

- Break

8d e8 00 20

8d e9 00 24

8d ea 00 28

8d eb 00 2c

8d ec 00 30

mem2mem:

8d f1 00 00

ad d1 00 00

21 ef 00 04

Data memory

@00            // Big Endian Format

C3 C3 C3 C3    // 0x00:03

12 34 56 78    // 0x04:07

89 AB CD EF    // 0x08:0B

A5 A5 A5 A5    // 0x0C:0F

5A 5A 5A 5A    // 0x10:13  //word 4

24 68 AC E0    // 0x14:17

13 57 9B DF    // 0x18:1B

0F 0F 0F 0F    // 0x1C:1F

F0 F0 F0 F0    // 0x20:23  //word 8

00 00 00 09    // 0x24:27

00 00 00 0A    // 0x28:2B

00 00 00 0B    // 0x2C:2F

00 00 00 0C    // 0x30:33  //word 12

00 00 00 0D    // 0x34:37

FF FF FF F8    // 0x38:3B

00 00 75 CC    // 0x3C:3F

@1CC

AB CD EF 01    // 0x1CC:1CF

@3F8

00 00 00 00    // 0x3F8:3FB

21 ce 00 04

21 ad ff ff

15 a0 ff fa

00 00 00 0d

/* Verification 6 */

BREAK INSTRUCTION FETCHED 4865.0 ns

Temporary register

R E G I S T E R ' S A F T E R B R E A K

t=4871.0 ns $r0 = 00000000 || t=4871.0 ns $r16 = xxxxxxxx

t=4871.0 ns $r1 = 12345678 || t=4871.0 ns $r17 = 000075cc

t=4871.0 ns $r2 = 89abcdef || t=4871.0 ns $r18 = xxxxxxxx

t=4871.0 ns $r3 = a5a5a5a5 || t=4871.0 ns $r19 = xxxxxxxx

t=4871.0 ns $r4 = 5a5a5a5a || t=4871.0 ns $r20 = xxxxxxxx

Data memory loaded to R1 through R12

t=4871.0 ns $r5 = 2468ace0 || t=4871.0 ns $r21 = xxxxxxxx

t=4871.0 ns $r6 = 13579bdf || t=4871.0 ns $r22 = xxxxxxxx

t=4871.0 ns $r7 = 0f0f0f0f || t=4871.0 ns $r23 = xxxxxxxx

Loop counter

t=4871.0 ns $r8 = f0f0f0f0 || t=4871.0 ns $r24 = xxxxxxxx

t=4871.0 ns $r9 = 00000009 || t=4871.0 ns $r25 = xxxxxxxx

t=4871.0 ns $r10 = 0000000a || t=4871.0 ns $r26 = xxxxxxxx

t=4871.0 ns $r11 = 0000000b || t=4871.0 ns $r27 = xxxxxxxx

Data memory pointers

t=4871.0 ns $r12 = 0000000c || t=4871.0 ns $r28 = xxxxxxxx

t=4871.0 ns $r13 = 00000000 || t=4871.0 ns $r29 = 000003fc

t=4871.0 ns $r14 = 10010100 || t=4871.0 ns $r30 = xxxxxxxx

t=4871.0 ns $r15 = 10010040 || t=4871.0 ns $r31 = xxxxxxxx

time=4871.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)________ ______||_________IO Memory(IOM)_____

t=4871.0 ns DM[000000c0] = c3c3c3c3 || t=4871.0 ns IOM[000000c0] = xxxxxxxx

Data memory from [000] stored to [0C0] through mem2mem loop.

t=4871.0 ns DM[000000c4] = 12345678 || t=4871.  ns IOM[000000c4] = xxxxxxxx

t=4871.0 ns DM[000000c8] = 89abcdef || t=4871.0 ns IOM[000000c8] = xxxxxxxx

t=4871.0 ns DM[000000cc] = a5a5a5a5 || t=4871.0 ns IOM[000000cc] = xxxxxxxx

t=4871.0 ns DM[000000d0] = 5a5a5a5a || t=4871.0 ns IOM[000000d0] = xxxxxxxx

t=4871.0 ns DM[000000d4] = 2468ace0 || t=4871.0 ns IOM[000000d4] = xxxxxxxx

t=4871.0 ns DM[000000d8] = 13579bdf || t=4871.0 ns IOM[000000d8] = xxxxxxxx

t=4871.0 ns DM[000000dc] = 0f0f0f0f || t=4871.0 ns IOM[000000dc] = xxxxxxxx

t=4871.0 ns DM[000000e0] = f0f0f0f0 || t=4871.0 ns IOM[000000e0] = xxxxxxxx

t=4871.0 ns DM[000000e4] = 00000009 || t=4871.0 ns IOM[000000e4] = xxxxxxxx

t=4871.0 ns DM[000000e8] = 0000000a || t=4871.0 ns IOM[000000e8] = xxxxxxxx

t=4871.0 ns DM[000000ec] = 0000000b || t=4871.0 ns IOM[000000ec] = xxxxxxxx

t=4871.0 ns DM[000000f0] = 0000000c || t=4871.0 ns IOM[000000f0] = xxxxxxxx

t=4871.0 ns DM[000000f4] = 0000000d || t=4871.0 ns IOM[000000f4] = xxxxxxxx

t=4871.0 ns DM[000000f8] = fffffff8 || t=4871.0 ns IOM[000000f8] = xxxxxxxx

t=4871.0 ns DM[000000fc] = 000075cc || t=4871.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 4871 ns

Data memory

@00            // Big Endian Format

C3 C3 C3 C3    // 0x00:03

12 34 56 78    // 0x04:07

89 AB CD EF    // 0x08:0B

A5 A5 A5 A5    // 0x0C:0F

5A 5A 5A 5A    // 0x10:13  //word 4

24 68 AC E0    // 0x14:17

13 57 9B DF    // 0x18:1B

0F 0F 0F 0F    // 0x1C:1F

F0 F0 F0 F0    // 0x20:23  //word 8

00 00 00 09    // 0x24:27

00 00 00 0A    // 0x28:2B

00 00 00 0B    // 0x2C:2F

00 00 00 0C    // 0x30:33  //word 12

00 00 00 0D    // 0x34:37

FF FF FF F8    // 0x38:3B

00 00 75 CC    // 0x3C:3F

@1CC

AB CD EF 01    // 0x1CC:1CF

@3F8

00 00 00 00    // 0x3F8:3FB

/* Instruction Memory Module 7 */

R15 <- 0x10010000

R14 <- 0x100100C0

R13 <- 16

@0

3c 0f 10 01

35 ef 00 00

3c 0e 10 01

35 ce 00 c0

20 0d 00 10

Load R01 to R12 from Data Memory based on where R15 points at and increment by 4 on each Load instruction

8d e1 00 04

8d e2 00 08

8d e3 00 0c

8d e4 00 10

8d e5 00 14

8d e6 00 18

8d e7 00 1c

Jump And Link (JAL) to mem2mem

8d e8 00 20

8d e9 00 24

8d ea 00 28

Set passed flag at R15 <- 0xFFFFFFFF

8d eb 00 2c

8d ec 00 30

Break

0c 10 00 15

3c 0f ff ff

- Use R17 as a temporary register loading data from memory to memory; R17 <- DM[R15]

- Store DM[R14] <- R17

- Increment both pointer R14 and R15 by 4

- Decrement loop counter R13 by 1

- Branch to mem2mem until R13=0

- Break

35 ef ff ff

00 00 00 0d

mem2mem:

8d f1 00 00

ad d1 00 00

21 ef 00 04

21 ce 00 04

Jump Register (JR) at R31

21 ad ff ff

15 a0 ff fa

Break

03 e0 00 08

00 00 00 0d

/* Verification 7 */

BREAK INSTRUCTION FETCHED 5015.0 ns

Temporary register

R E G I S T E R ' S A F T E R B R E A K

t=5021.0 ns $r0 = 00000000 || t=5021.0 ns $r16 = xxxxxxxx

t=5021.0 ns $r1 = 12345678 || t=5021.0 ns $r17 = 000075cc

t=5021.0 ns $r2 = 89abcdef || t=5021.0 ns $r18 = xxxxxxxx

t=5021.0 ns $r3 = a5a5a5a5 || t=5021.0 ns $r19 = xxxxxxxx

t=5021.0 ns $r4 = 5a5a5a5a || t=5021.0 ns $r20 = xxxxxxxx

Data memory loaded to R1 through R12

t=5021.0 ns $r5 = 2468ace0 || t=5021.0 ns $r21 = xxxxxxxx

t=5021.0 ns $r6 = 13579bdf || t=5021.0 ns $r22 = xxxxxxxx

t=5021.0 ns $r7 = 0f0f0f0f || t=5021.0 ns $r23 = xxxxxxxx

t=5021.0 ns $r8 = f0f0f0f0 || t=5021.0 ns $r24 = xxxxxxxx

t=5021.0 ns $r9 = 00000009 || t=5021.0 ns $r25 = xxxxxxxx

Loop counter

t=5021.0 ns $r10 = 0000000a || t=5021.0 ns $r26 = xxxxxxxx

t=5021.0 ns $r11 = 0000000b || t=5021.0 ns $r27 = xxxxxxxx

Data memory pointers

t=5021.0 ns $r12 = 0000000c || t=5021. 0 ns $r28 = xxxxxxxx

Data memory from [000] stored to [0C0] through mem2mem loop.

t=5021.0 ns $r13 = 00000000 || t=5021.0 ns $r29 = 000003fc

t=5021.0 ns $r14 = 10010100 || t=5021.0 ns $r30 = xxxxxxxx

t=5021.0 ns $r15 = ffffffff || t=5021.0 ns $r31 = 00000048

Passed flag R15

time=5021.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=5021.0 ns DM[000000c0] = c3c3c3c3 || t=5021.0 ns IOM[000000c0] = xxxxxxxx

t=5021.0 ns DM[000000c4] = 12345678 || t=5021.0 ns IOM[000000c4] = xxxxxxxx

t=5021.0 ns DM[000000c8] = 89abcdef || t=5021.0 ns IOM[000000c8] = xxxxxxxx

t=5021.0 ns DM[000000cc] = a5a5a5a5 || t=5021.0 ns IOM[000000cc] = xxxxxxxx

t=5021.0 ns DM[000000d0] = 5a5a5a5a || t=5021.0 ns IOM[000000d0] = xxxxxxxx

t=5021.0 ns DM[000000d4] = 2468ace0 || t=5021.0 ns IOM[000000d4] = xxxxxxxx

t=5021.0 ns DM[000000d8] = 13579bdf || t=5021.0 ns IOM[000000d8] = xxxxxxxx

t=5021.0 ns DM[000000dc] = 0f0f0f0f || t=5021.0 ns IOM[000000dc] = xxxxxxxx

t=5021.0 ns DM[000000e0] = f0f0f0f0 || t=5021.0 ns IOM[000000e0] = xxxxxxxx

t=5021.0 ns DM[000000e4] = 00000009 || t=5021.0 ns IOM[000000e4] = xxxxxxxx

t=5021.0 ns DM[000000e8] = 0000000a || t=5021.0 ns IOM[000000e8] = xxxxxxxx

t=5021.0 ns DM[000000ec] = 0000000b || t=5021.0 ns IOM[000000ec] = xxxxxxxx

t=5021.0 ns DM[000000f0] = 0000000c || t=5021.0 ns IOM[000000f0] = xxxxxxxx

t=5021.0 ns DM[000000f4] = 0000000d || t=5021.0 ns IOM[000000f4] = xxxxxxxx

t=5021.0 ns DM[000000f8] = fffffff8 || t=5021.0 ns IOM[000000f8] = xxxxxxxx

t=5021.0 ns DM[000000fc] = 000075cc || t=5021.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 5021 ns :

Set Memory Pointer R15 <- 0x10010000

/* Instruction Memory Module 8 */

@0

3c 0f 10 01

Load R1 through R7 from Data Memory

35 ef 00 00

8d e1 00 00

8d e2 00 04

- Multiply R01, R02

- Put [31:0] result into R08

- Branch to Fail1 if R05 not equal to R08. Should not branch.

8d e3 00 08

8d e4 00 0c

8d e5 00 10

8d e6 00 14

8d e7 00 18

00 22 00 18

00 00 40 12

- Multiply R03, R02

- Put [31:0] result into R09

- Put [63:32] result into R10

- Branch to Fail2L if R06 not equal to R09. Should not branch.

- Branch to Fail2H if R07 not equal to R10. Should not branch.

14 a8 00 10

00 62 00 18

00 00 48 12

00 00 50 10

14 c9 00 0f

14 ea 00 11

00 24 00 18

- Multiply R01, R04

- Put [31:0] result into R11

- Put [63:32] result into R12

- Branch to Fail3L if R06 not equal to R11. Should not branch.

- Branch to Fail3H if R07 not equal to R12. Should not branch.

00 00 58 12

00 00 60 10

14 cb 00 10

14 ec 00 12

00 64 00 18

00 00 68 12

14 ad 00 12

pass:

- Multiply R03, R04

- Put [31:0] result into R13

- Branch to Fail4 if R05 not equal to R13. Should not branch.

3c 0e 00 00

35 ce 00 00

00 00 00 0d

fail1:

3c 0e ff ff

35 ce ff ff

00 00 00 0d

Set passed flag to R14 <- 0x00000000

Then break.

fail2L:

3c 0e ff ff

35 ce ff fe

00 00 00 0d

fail2H:

3c 0e ff ff

Set failed flag to R14 <- 0xFFFFFFFF

Then break.

35 ce ff fd

00 00 00 0d

fail3L:

3c 0e ff ff

35 ce ff fc

00 00 00 0d

fail3H:

3c 0e ff ff

35 ce ff fb

00 00 00 0d

fail4:

3c 0e ff ff

35 ce ff fa

00 00 00 0d

/* Verification 8 */

BREAK INSTRUCTION FETCHED 1115.0 ns

R E G I S T E R ' S A F T E R B R E A K

Load R1 through R7 from Data Memory

t=1121.0 ns $r0 = 00000000 || t=1121.0 ns $r16 = xxxxxxxx

t=1121.0 ns $r1 = 00000019 || t=1121.0 ns $r17 = xxxxxxxx

t=1121.0 ns $r2 = 000003e8 || t=1121.0 ns $r18 = xxxxxxxx

t=1121.0 ns $r3 = ffffffe7 || t=1121.0 ns $r19 = xxxxxxxx

t=1121.0 ns $r4 = fffffc18 || t=1121.0 ns $r20 = xxxxxxxx

t=1121.0 ns $r5 = 000061a8 || t=1121.0 ns $r21 = xxxxxxxx

R8 to R13, results from multiplications

t=1121.0 ns $r6 = ffff9e58 || t=1121.0 ns $r22 = xxxxxxxx

t=1121.0 ns $r7 = ffffffff || t=1121.0 ns $r23 = xxxxxxxx

t=1121.0 ns $r8 = 000061a8 || t=1121.0 ns $r24 = xxxxxxxx

t=1121.0 ns $r9 = ffff9e58 || t=1121.0 ns $r25 = xxxxxxxx

t=1121.0 ns $r10 = ffffffff || t=1121.0 ns $r26 = xxxxxxxx

Passed flag

t=1121.0 ns $r11 = ffff9e58 || t=1121.0 ns $r27 = xxxxxxxx

t=1121.0 ns $r12 = ffffffff || t=1121.0 ns $r28 = xxxxxxxx

t=1121.0 ns $r13 = 000061a8 || t=1121.0 ns $r29 = 000003fc

Data Memory pointer

t=1121.0 ns $r14 = 00000000 || t=1121.0 ns $r30 = xxxxxxxx

t=1121.0 ns $r15 = 10010000 || t=1121.0 ns $r31 = xxxxxxxx

time=1121.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=1121.0 ns DM[000000c0] = xxxxxxxx || t=1121.0 ns IOM[000000c0] = xxxxxxxx

t=1121.0 ns DM[000000c4] = xxxxxxxx || t=1121.0 ns IOM[000000c4] = xxxxxxxx

t=1121.0 ns DM[000000c8] = xxxxxxxx || t=1121.0 ns IOM[000000c8] = xxxxxxxx

t=1121.0 ns DM[000000cc] = xxxxxxxx || t=1121.0 ns IOM[000000cc] = xxxxxxxx

t=1121.0 ns DM[000000d0] = xxxxxxxx || t=1121.0 ns IOM[000000d0] = xxxxxxxx

t=1121.0 ns DM[000000d4] = xxxxxxxx || t=1121.0 ns IOM[000000d4] = xxxxxxxx

t=1121.0 ns DM[000000d8] = xxxxxxxx || t=1121.0 ns IOM[000000d8] = xxxxxxxx

t=1121.0 ns DM[000000dc] = xxxxxxxx || t=1121.0 ns IOM[000000dc] = xxxxxxxx

t=1121.0 ns DM[000000e0] = xxxxxxxx || t=1121.0 ns IOM[000000e0] = xxxxxxxx

t=1121.0 ns DM[000000e4] = xxxxxxxx || t=1121.0 ns IOM[000000e4] = xxxxxxxx

t=1121.0 ns DM[000000e8] = xxxxxxxx || t=1121.0 ns IOM[000000e8] = xxxxxxxx

t=1121.0 ns DM[000000ec] = xxxxxxxx || t=1121.0 ns IOM[000000ec] = xxxxxxxx

t=1121.0 ns DM[000000f0] = xxxxxxxx || t=1121.0 ns IOM[000000f0] = xxxxxxxx

t=1121.0 ns DM[000000f4] = xxxxxxxx || t=1121.0 ns IOM[000000f4] = xxxxxxxx

t=1121.0 ns DM[000000f8] = xxxxxxxx || t=1121.0 ns IOM[000000f8] = xxxxxxxx

t=1121.0 ns DM[000000fc] = xxxxxxxx || t=1121.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 1121 ns

Set Memory Pointer R15 <- 0x10010000

Set failed flag to R14 <- 0xFFFFFFFF

Then break.

Set passed flag to R14 <- 0x00000000

Then break.

/* Instruction Memory Module 9 */

@0

3c 0f 10 01

R1 <- 0xFFFFFF8A; R2 <- 0x0000008A

35 ef 00 c0

20 01 ff 8a

20 02 00 8a

Jump and Link to slt_tests

0c 10 00 22

3c 0d 77 88

R13 <- 0x77887788;    
R12 <- 0x88778877;
R11 <- 0xFFFFFFFF;

35 ad 77 88

3c 0c 88 77

35 8c 88 77

3c 0b ff ff

35 6b ff ff

- XOR R10, R13, R12;  R10 <- FFFFFFFF

- Branch to xor_pass if R10 = R11

- If not branch, R14 <- 0xFFFFFFFB and break.

01 ac 50 26

11 4b 00 02

20 0e ff fb

00 00 00 0d

- R9 <- 0x00000000

- Branch to and_pass if R9 = zero

- If not branch, R14 <- 0xFFFFFFFA and break.

xor_pass:

01 ac 48 24

11 20 00 02

20 0e ff fa

00 00 00 0d

- R9 <- 0x100100CA; R8 <- 0x100100CA;

- Branch to or_pass if R9 = R8

- If not branch, R14 <- 0xFFFFFFF9 and break.

and_pass:

01 e2 48 25

3c 08 10 01

35 08 00 ca

11 09 00 02

20 0e ff f9

00 00 00 0d

- R9 <- 0xEFFEFF35; R8 <- 0xEFFEFF35;

- Branch to nor_pass if R9 = R8

- If not branch, R14 <- 0xFFFFFFF8 and break

or_pass:

01 e2 48 27

3c 08 ef fe

35 08 ff 35

11 09 00 02

- DM[D0] <- R8(0xEFFEFF35)

- Clear R14, and break.  PASS ALL !!

20 0e ff f8

00 00 00 0d

nor_pass:

ad e8 00 10

00 00 70 20

00 00 00 0d

If R1 < R2, branch to slt1.

If not, set fail flag. R14 <- 0xFFFFFFFF.

Set pass flag. DM[C0] <- 0xC0

subroutine:

00 22 18 2a

14 60 00 02

20 0e ff ff

00 00 00 0d slt1:

20 04 00 c0

ad e4 00 00

If R2 < R1, branch to slt2.

If not, set fail flag. R14 <- 0xFFFFFFFE.

Set pass flag. DM[C4] <- 0xC4

00 41 18 2b

14 60 00 02

20 0e ff fe

00 00 00 0d slt2:

If R2 < R1, branch to slt2.

If not, set fail flag. R14 <- 0xFFFFFFFE.

Set pass flag. DM[C4] <- 0xC4

20 05 00 c4

ad e5 00 04

00 41 18 2a

10 60 00 02

20 0e ff fd

If R2 !< R1, branch to slt3.

If not, set fail flag. R14 <- 0xFFFFFFFD.

Set pass flag. DM[C8] <- 0xC8

00 00 00 0d slt3:

20 06 00 c8

ad e6 00 08

00 22 18 2b

10 60 00 02

20 0e ff fc

00 00 00 0d slt4:

20 07 00 cc

If R1 !< R2, branch to slt4.

If not, set fail flag. R14 <- 0xFFFFFFFC.

Set pass flag. DM[CC] <- 0xCC

ad e7 00 0c

03 e0 00 08

Jump Register back to where Jump and Link was.

/* Verification 9 */

BREAK INSTRUCTION FETCHED 1735.0 ns

R E G I S T E R ' S A F T E R B R E A K

t=1741.0 ns $r0 = 00000000 || t=1741.0 ns $r16 = xxxxxxxx

t=1741.0 ns $r1 = ffffff8a || t=1741.0 ns $r17 = xxxxxxxx

t=1741.0 ns $r2 = 0000008a || t=1741.0 ns $r18 = xxxxxxxx

t=1741.0 ns $r3 = 00000000 || t=1741.0 ns $r19 = xxxxxxxx

Scratch registers R1 – R3 and R8 – R13

Pass flags R4 – R7

t=1741.0 ns $r4 = 000000c0 || t=1741.0 ns $r20 = xxxxxxxx

t=1741.0 ns $r5 = 000000c4 || t=1741.0 ns $r21 = xxxxxxxx

t=1741.0 ns $r6 = 000000c8 || t=1741.0 ns $r22 = xxxxxxxx

t=1741.0 ns $r7 = 000000cc || t=1741.0 ns $r23 = xxxxxxxx

t=1741.0 ns $r8 = effeff35 || t=1741.0 ns $r24 = xxxxxxxx

t=1741.0 ns $r9 = effeff35 || t=1741.0 ns $r25 = xxxxxxxx

t=1741.0 ns $r10 = ffffffff || t=1741.0 ns $r26 = xxxxxxxx

t=1741.0 ns $r11 = ffffffff || t=1741.0 ns $r27 = xxxxxxxx

If pass all tests, this should be 0x00

t=1741.0 ns $r12 = 88778877 || t=1741.0 ns $r28 = xxxxxxxx

t=1741.0 ns $r13 = 77887788 || t=1741.0 ns $r29 = 000003fc

t=1741.0 ns $r14 = 00000000 || t=1741.0 ns $r30 = xxxxxxxx

t=1741.0 ns $r15 = 100100c0 || t=1741.0 ns $r31 = 00000014

Data memory pointer.

time=1741.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=1741.0 ns DM[000000c0] = 000000c0 || t=1741.0 ns IOM[000000c0] = xxxxxxxx

Pass flags from slt_tests

t=1741.0 ns DM[000000c4] = 000000c4 || t=1741.0 ns IOM[000000c4] = xxxxxxxx

t=1741.0 ns DM[000000c8] = 000000c8 || t=1741.0 ns IOM[000000c8] = xxxxxxxx

t=1741.0 ns DM[000000cc] = 000000cc || t=1741.0 ns IOM[000000cc] = xxxxxxxx

t=1741.0 ns DM[000000d0] = effeff35 || t=1741.0 ns IOM[000000d0] = xxxxxxxx

t=1741.0 ns DM[000000d4] = xxxxxxxx || t=1741.0 ns IOM[000000d4] = xxxxxxxx

t=1741.0 ns DM[000000d8] = xxxxxxxx || t=1741.0 ns IOM[000000d8] = xxxxxxxx

t=1741.0 ns DM[000000dc] = xxxxxxxx || t=1741.0 ns IOM[000000dc] = xxxxxxxx

t=1741.0 ns DM[000000e0] = xxxxxxxx || t=1741.0 ns IOM[000000e0] = xxxxxxxx

t=1741.0 ns DM[000000e4] = xxxxxxxx || t=1741.0 ns IOM[000000e4] = xxxxxxxx

t=1741.0 ns DM[000000e8] = xxxxxxxx || t=1741.0 ns IOM[000000e8] = xxxxxxxx

t=1741.0 ns DM[000000ec] = xxxxxxxx || t=1741.0 ns IOM[000000ec] = xxxxxxxx

t=1741.0 ns DM[000000f0] = xxxxxxxx || t=1741.0 ns IOM[000000f0] = xxxxxxxx

t=1741.0 ns DM[000000f4] = xxxxxxxx || t=1741.0 ns IOM[000000f4] = xxxxxxxx

t=1741.0 ns DM[000000f8] = xxxxxxxx || t=1741.0 ns IOM[000000f8] = xxxxxxxx

t=1741.0 ns DM[000000fc] = xxxxxxxx || t=1741.0 ns IOM[000000fc] = xxxxxxxx

Stopped at time : 1741 ns :

Set Memory Pointer R15 <- 0x10010000

Set failed flag to R14 <- 0xFFFFFFFF

Then break.

Set passed flag to R14 <- 0x00000000

Then break.

/* Instruction Memory Module 10 */

@0

3c 0f 10 01

35 ef 00 00

Load R1 through R8 from Data Memory

8d e1 00 00

8d e2 00 04

8d e3 00 08

8d e4 00 0c

- Divide R1 by R2

- Put[31:0] result into R09

- Put[63:32] result into R10

- Branch to fail1Q if R9 not equal to R5. Should not branch.

- Branch to fail1R if R10 not equal to R6. Should not branch.

8d e5 00 10

8d e6 00 14

8d e7 00 18

8d e8 00 1c

00 22 00 1a

00 00 48 12

00 00 50 10

- Divide R2 by R3

- Put[31:0] result into R09

- Put[63:32] result into R10

- Branch to fail2Q if R9 not equal to R7. Should not branch.

- Branch to fail2R if R10 not equal to R8. Should not branch.

15 25 00 16

15 46 00 18

00 62 00 1a

00 00 48 12

00 00 50 10

15 27 00 17

15 48 00 19

- Divide R4 by R1

- Put[31:0] result into R09

- Put[63:32] result into R10

- Branch to fail3Q if R9 not equal to R7. Should not branch.

- Branch to fail3R if R10 not equal to R6. Should not branch.

00 24 00 1a

00 00 48 12

00 00 50 10

15 27 00 18

15 46 00 1a

00 64 00 1a

00 00 48 12

- Divide R4 by R3

- Put[31:0] result into R09

- Put[63:32] result into R10

- Branch to fail4Q if R9 not equal to R4. Should not branch.

- Branch to fail4R if R10 not equal to R8. Should not branch.

00 00 50 10

15 25 00 19

15 48 00 1b

3c 0b 00 00

35 6b 00 00

00 0b 60 20

00 0b 68 20

Clear R11-R14  <- 0x00000000; Then break.

00 0b 70 20

00 00 00 0d

Fail1Q: R14 <- 0xFFFFFFFF then break.

3c 0e ff ff

35 ce ff ff

Fail1R: R14 <- 0xFFFFFFFE then break.

00 00 00 0d

3c 0e ff ff

35 ce ff fe

Fail2Q: R14 <- 0xFFFFFFFD then break.

00 00 00 0d

3c 0e ff ff

Fail2R: R14 <- 0xFFFFFFFC then break.

35 ce ff fd

00 00 00 0d

3c 0e ff ff

35 ce ff fc

00 00 00 0d

3c 0e ff ff

Fail3Q: R14 <- 0xFFFFFFFB then break.

35 ce ff fb

Fail3R: R14 <- 0xFFFFFFFA then break.

00 00 00 0d

3c 0e ff ff

Fail4Q: R14 <- 0xFFFFFFF9 then break.

35 ce ff fa

00 00 00 0d

3c 0e ff ff

35 ce ff f9

Fail4R: R14 <- 0xFFFFFFF8 then break.

00 00 00 0d

3c 0e ff ff

35 ce ff f8

00 00 00 0d

/* Verification 10 */

BREAK INSTRUCTION FETCHED 1465.0 ns

R E G I S T E R ' S A F T E R B R E A K

Load R1 through R8 from Data Memory

t=1471.0 ns $r0 = 00000000 || t=1471.0 ns $r16 = xxxxxxxx

t=1471.0 ns $r1 = 00040911 || t=1471.0 ns $r17 = xxxxxxxx

t=1471.0 ns $r2 = 000003e8 || t=1471.0 ns $r18 = xxxxxxxx

t=1471.0 ns $r3 = fffbf6ef || t=1471.0 ns $r19 = xxxxxxxx

t=1471.0 ns $r4 = fffffc18 || t=1471.0 ns $r20 = xxxxxxxx

t=1471.0 ns $r5 = 00000108 || t=1471.0 ns $r21 = xxxxxxxx

Last result from R3 and R4 division

t=1471.0 ns $r6 = 000001d1 || t=1471.0 ns $r22 = xxxxxxxx

t=1471.0 ns $r7 = fffffef8 || t=1471.0 ns $r23 = xxxxxxxx

t=1471.0 ns $r8 = fffffe2f || t=1471.0 ns $r24 = xxxxxxxx

t=1471.0 ns $r9 = 00000108 || t=1471.0 ns $r25 = xxxxxxxx

Passed flags

t=1471.0 ns $r10 = fffffe2f || t=1471.0 ns $r26 = xxxxxxxx

t=1471.0 ns $r11 = 00000000 || t=1471.0 ns $r27 = xxxxxxxx

t=1471.0 ns $r12 = 00000000 || t=1471.0 ns $r28 = xxxxxxxx

Data Memory pointer

t=1471.0 ns $r13 = 00000000 || t=1471.0 ns $r29 = 000003fc

t=1471.0 ns $r14 = 00000000 || t=1471.0 ns $r30 = xxxxxxxx

t=1471.0 ns $r15 = 10010000 || t=1471.0 ns $r31 = xxxxxxxx

time=1471.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=1471.0 ns DM[000000c0] = xxxxxxxx || t=1471.0 ns IOM[000000c0] = xxxxxxxx

t=1471.0 ns DM[000000c4] = xxxxxxxx || t=1471.0 ns IOM[000000c4] = xxxxxxxx

t=1471.0 ns DM[000000c8] = xxxxxxxx || t=1471.0 ns IOM[000000c8] = xxxxxxxx

t=1471.0 ns DM[000000cc] = xxxxxxxx || t=1471.0 ns IOM[000000cc] = xxxxxxxx

t=1471.0 ns DM[000000d0] = xxxxxxxx || t=1471.0 ns IOM[000000d0] = xxxxxxxx

t=1471.0 ns DM[000000d4] = xxxxxxxx || t=1471.0 ns IOM[000000d4] = xxxxxxxx

t=1471.0 ns DM[000000d8] = xxxxxxxx || t=1471.0 ns IOM[000000d8] = xxxxxxxx

t=1471.0 ns DM[000000dc] = xxxxxxxx || t=1471.0 ns IOM[000000dc] = xxxxxxxx

t=1471.0 ns DM[000000e0] = xxxxxxxx || t=1471.0 ns IOM[000000e0] = xxxxxxxx

t=1471.0 ns DM[000000e4] = xxxxxxxx || t=1471.0 ns IOM[000000e4] = xxxxxxxx

t=1471.0 ns DM[000000e8] = xxxxxxxx || t=1471.0 ns IOM[000000e8] = xxxxxxxx

t=1471.0 ns DM[000000ec] = xxxxxxxx || t=1471.0 ns IOM[000000ec] = xxxxxxxx

t=1471.0 ns DM[000000f0] = xxxxxxxx || t=1471.0 ns IOM[000000f0] = xxxxxxxx

t=1471.0 ns DM[000000f4] = xxxxxxxx || t=1471.0 ns IOM[000000f4] = xxxxxxxx

t=1471.0 ns DM[000000f8] = xxxxxxxx || t=1471.0 ns IOM[000000f8] = xxxxxxxx

t=1471.0 ns DM[000000fc] = xxxxxxxx || t=1471.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 1471 ns :

Set Memory Pointer R15 <- 0x10010000

If R2 < R1, branch to slt2.

If not, set fail flag. R14 <- 0xFFFFFFFE.

Set pass flag. DM[C4] <- 0xC4

If R1 < R2, branch to slt1.

If not, set fail flag. R14 <- 0xFFFFFFFF.

Set pass flag. DM[C0] <- 0xC0

- DM[D0] <- R8(0xEFFEFF35)

- Clear R14, and break.  PASS ALL !!

/* Instruction Memory Module 11 */

@0

3c 0f 10 01

R1 <- 0xFFFFFF8A; R2 <- 0x0000008A

35 ef 00 c0

20 01 ff 8a

20 02 00 8a

Jump and Link to slt_tests

0c 10 00 1a

3c 0d ff ff

35 ad 55 55

R13 <- 0xFFFF5555;    
R12 <- 0xFFFFFAF5;
R11 <- 0xFFFFFFFF;
R10 <- 0x0000F0F0;

3c 0c ff ff

35 8c fa f5

3c 0b ff ff

35 6b ff ff

3c 0a 00 00

35 4a f0 f0

39 a9 aa aa

- R9 <- 0xFFFFFFFF; R8 <- 0x0

- Branch to xor_p1,

- else R14 <- 0xFFFFFFF9, and break. (fail)

01 2b 40 22

11 00 00 02

20 0e ff f9

00 00 00 0d xor_p1:

31 87 f5 fa

- R7 <- 0x0000F0F0;

- Branch to xor_p2,

- else R14 <- 0xFFFFFFF8 and break. (fail)

00 ea 40 22

11 00 00 02

20 0e ff f8

00 00 00 0d xor_p2:

ad e1 00 18

00 00 00 0d

DM[D8] <- 0xFFFFFF8A and break; (pass)

00 00 00 0d

subroutine

2c 23 ff 8b

- Branch to slt_p1 if R1 < 0xFF8B

- And DM[C0] < 0xC0

- Else R14 <- 0xFFFFFFFF (fail)

14 60 00 02

20 0e ff ff

00 00 00 0d slt_p1:

20 04 00 c0

ad e4 00 00

- Branch to slt_p2 if R1 !< 0xFF89

- And DM[C4] < 0xC4

- Else R14 <- 0xFFFFFFFE (fail)

2c 23 ff 89

10 60 00 02

20 0e ff fe

00 00 00 0d slt_p2:

20 05 00 c4

ad e5 00 04

- Branch to slt_3 if R1 !< 0xFF8A

- And DM[C8] < 0xC8

- Else R14 <- 0xFFFFFFFD (fail)

2c 23 ff 8a

10 60 00 02

20 0e ff fd

00 00 00 0d slt_p3:

20 06 00 c8

ad e6 00 08

- Branch to slt_p4 if R2 < 0x008B

- And DM[CC] < 0xCC

- Else R14 <- 0xFFFFFFFC (fail)

2c 43 00 8b

14 60 00 02

20 0e ff fc

00 00 00 0d slt_p4:

20 07 00 cc

ad e7 00 0c

2c 43 00 89

10 60 00 02

- Branch to slt_p5 if R2 !< 0x0089

- And DM[D0] < 0xD0

- Else R14 <- 0xFFFFFFFB (fail)

20 0e ff fb

00 00 00 0d slt_p5:

20 08 00 d0

- Branch to slt_p6 if R2 !< 0x008A

- And DM[D4] < 0xD4

- Else R14 <- 0xFFFFFFFA (fail)

ad e8 00 10

2c 43 00 8a

10 60 00 02

20 0e ff fa

00 00 00 0d slt_p6:

20 06 00 d4

Jump register to where jump and link was

ad e6 00 14

20 0e 00 00

03 e0 00 08

/* Verification 11 */

BREAK INSTRUCTION FETCHED 1895.0 ns

Comparison values

R E G I S T E R ' S A F T E R B R E A K

t=1901.0 ns $r0 = 00000000 || t=1901.0 ns $r16 = xxxxxxxx

t=1901.0 ns $r1 = ffffff8a || t=1901.0 ns $r17 = xxxxxxxx

t=1901.0 ns $r2 = 0000008a || t=1901.0 ns $r18 = xxxxxxxx

t=1901.0 ns $r3 = 00000000 || t=1901.0 ns $r19 = xxxxxxxx

Scratch registers

t=1901.0 ns $r4 = 000000c0 || t=1901.0 ns $r20 = xxxxxxxx

t=1901.0 ns $r5 = 000000c4 || t=1901.0 ns $r21 = xxxxxxxx

t=1901.0 ns $r6 = 000000d4 || t=1901.0 ns $r22 = xxxxxxxx

t=1901.0 ns $r7 = 0000f0f0 || t=1901.0 ns $r23 = xxxxxxxx

t=1901.0 ns $r8 = 00000000 || t=1901.0 ns $r24 = xxxxxxxx

t=1901.0 ns $r9 = ffffffff || t=1901.0 ns $r25 = xxxxxxxx

Pattern starters

t=1901.0 ns $r10 = 0000f0f0 || t=1901.0 ns $r26 = xxxxxxxx

t=1901.0 ns $r11 = ffffffff || t=1901.0 ns $r27 = xxxxxxxx

t=1901.0 ns $r12 = fffffaf5 || t=1901.0 ns $r28 = xxxxxxxx

t=1901.0 ns $r13 = ffff5555 || t=1901.0 ns $r29 = 000003fc

t=1901.0 ns $r14 = 00000000 || t=1901.0 ns $r30 = xxxxxxxx

Memory pointer

t=1901.0 ns $r15 = 100100c0 || t=1901.0 ns $r31 = 00000014

time=1901.0 ns M[3F0]=xxxxxxxx

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=1901.0 ns DM[000000c0] = 000000c0 || t=1901.0 ns IOM[000000c0] = xxxxxxxx

Pass flags

t=1901.0 ns DM[000000c4] = 000000c4 || t=1901.0 ns IOM[000000c4] = xxxxxxxx

t=1901.0 ns DM[000000c8] = 000000c8 || t=1901.0 ns IOM[000000c8] = xxxxxxxx

t=1901.0 ns DM[000000cc] = 000000cc || t=1901.0 ns IOM[000000cc] = xxxxxxxx

t=1901.0 ns DM[000000d0] = 000000d0 || t=1901.0 ns IOM[000000d0] = xxxxxxxx

t=1901.0 ns DM[000000d4] = 000000d4 || t=1901.0 ns IOM[000000d4] = xxxxxxxx

Stored R1 proving pasted all tests.

t=1901.0 ns DM[000000d8] = ffffff8a || t=1901.0 ns IOM[000000d8] = xxxxxxxx

t=1901.0 ns DM[000000dc] = xxxxxxxx || t=1901.0 ns IOM[000000dc] = xxxxxxxx

t=1901.0 ns DM[000000e0] = xxxxxxxx || t=1901.0 ns IOM[000000e0] = xxxxxxxx

t=1901.0 ns DM[000000e4] = xxxxxxxx || t=1901.0 ns IOM[000000e4] = xxxxxxxx

t=1901.0 ns DM[000000e8] = xxxxxxxx || t=1901.0 ns IOM[000000e8] = xxxxxxxx

t=1901.0 ns DM[000000ec] = xxxxxxxx || t=1901.0 ns IOM[000000ec] = xxxxxxxx

t=1901.0 ns DM[000000f0] = xxxxxxxx || t=1901.0 ns IOM[000000f0] = xxxxxxxx

t=1901.0 ns DM[000000f4] = xxxxxxxx || t=1901.0 ns IOM[000000f4] = xxxxxxxx

t=1901.0 ns DM[000000f8] = xxxxxxxx || t=1901.0 ns IOM[000000f8] = xxxxxxxx

t=1901.0 ns DM[000000fc] = xxxxxxxx || t=1901.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 1901 ns

/* Instruction Memory Module 12 */

Set memory pointer R15 <- 0x100100C0
R1 <- 0xFFFFFF8A
R2 <- 0x0000008A
Jump and Link to blt_tests
At the return, DM[D8] <- R1, DM[DC] <- R2, then break.

@0

3c 0f 10 01

35 ef 00 c0

20 01 ff 8a

20 02 00 8a

0c 10 00 08

ad e1 00 18

ad e2 00 1c

Branch to blez_p1 if R1 <= 0 (pass)

00 00 00 0d

18 20 00 02 blt_tests

Branch to blez_f2 if R2 <= 0
DM[C0] <- 0xC0, DM[C4] <- 0xC4
R14 <- 0XFFFFFFFE (fail)

20 0e ff ff

00 00 00 0d

20 03 00 c0 blez_p1:

ad e3 00 00

18 40 00 03

20 04 00 c4

Branch to blez_p3 if R0 <= 0 (pass)
DM[C8] <- 0xC8
R14 <- 0XFFFFFFFD (fail)

ad e4 00 04

08 10 00 13

20 0e ff fe blez_f2:

00 00 00 0d

18 00 00 02 blez_p2:

20 0e ff fd

00 00 00 0d

Branch to bgtz_p1 if (R2>0) (pass)

20 05 00 c8 blez_p3:

ad e5 00 08

1c 40 00 02

Branch to bgtz_f2 if (R1>0 ) (fail)
DM[CC] <- 0xCC, DM[D0] <- D0
R14 <- 0xFFFFFFFB (fail)

20 0e ff fc

00 00 00 0d

20 06 00 cc bgtz_p1:

ad e6 00 0c

1c 20 00 03

20 07 00 d0

Branch to bgtz_f2 if (R1 >0) (fail)
DM[D4 <- 0xD4
R14 <- 0xFFFFFFFA (fail)

ad e7 00 10

08 10 00 23

20 0e ff fb bgtz_f2:

00 00 00 0d

1c 20 00 03 bgtz_p2:

20 08 00 d4

ad e8 00 14

Return to where jump and link was and clear R14 <- 0x00000000

08 10 00 29

20 0e ff fa bgtz_f3:

00 00 00 0d

20 0e 00 00 bgtz_p3:

03 e0 00 08

/* Verification 12 */

BREAK INSTRUCTION FETCHED 1255.0 ns

R E G I S T E R ' S A F T E R B R E A K

Data for comparisions

t=1261.0 ns $r0 = 00000000 || t=1261.0 ns $r16 = xxxxxxxx

t=1261.0 ns $r1 = ffffff8a || t=1261.0 ns $r17 = xxxxxxxx

t=1261.0 ns $r2 = 0000008a || t=1261.0 ns $r18 = xxxxxxxx

t=1261.0 ns $r3 = 000000c0 || t=1261.0 ns $r19 = xxxxxxxx

Pass flags

t=1261.0 ns $r4 = 000000c4 || t=1261.0 ns $r20 = xxxxxxxx

t=1261.0 ns $r5 = 000000c8 || t=1261.0 ns $r21 = xxxxxxxx

t=1261.0 ns $r6 = 000000cc || t=1261.0 ns $r22 = xxxxxxxx

Return address

t=1261.0 ns $r7 = 000000d0 || t=1261.0 ns $r23 = xxxxxxxx

t=1261.0 ns $r8 = 000000d4 || t=1261.0 ns $r24 = xxxxxxxx

t=1261.0 ns $r9 = xxxxxxxx || t=1261.0 ns $r25 = xxxxxxxx

Memory pointer

t=1261.0 ns $r10 = xxxxxxxx || t=1261.0 ns $r26 = xxxxxxxx

t=1261.0 ns $r11 = xxxxxxxx || t=1261.0 ns $r27 = xxxxxxxx

t=1261.0 ns $r12 = xxxxxxxx || t=1261.0 ns $r28 = xxxxxxxx

t=1261.0 ns $r13 = xxxxxxxx || t=1261.0 ns $r29 = 000003fc

t=1261.0 ns $r14 = 00000000 || t=1261.0 ns $r30 = xxxxxxxx

t=1261.0 ns $r15 = 100100c0 || t=1261.0 ns $r31 = 00000014

time=1261.0 ns M[3F0]=xxxxxxxx

R1, R2 as pass flags after passed blt_test

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=1261.0 ns DM[000000c0] = 000000c0 || t=1261.0 ns IOM[000000c0] = xxxxxxxx

t=1261.0 ns DM[000000c4] = 000000c4 || t=1261.0 ns IOM[000000c4] = xxxxxxxx

t=1261.0 ns DM[000000c8] = 000000c8 || t=1261.0 ns IOM[000000c8] = xxxxxxxx

Pass flags

t=1261.0 ns DM[000000cc] = 000000cc || t=1261.0 ns IOM[000000cc] = xxxxxxxx

t=1261.0 ns DM[000000d0] = 000000d0 || t=1261.0 ns IOM[000000d0] = xxxxxxxx

t=1261.0 ns DM[000000d4] = 000000d4 || t=1261.0 ns IOM[000000d4] = xxxxxxxx

t=1261.0 ns DM[000000d8] = ffffff8a || t=1261.0 ns IOM[000000d8] = xxxxxxxx

R1, R2 as pass flags after passed all instructions

t=1261.0 ns DM[000000dc] = 0000008a || t=1261.0 ns IOM[000000dc] = xxxxxxxx

t=1261.0 ns DM[000000e0] = xxxxxxxx || t=1261.0 ns IOM[000000e0] = xxxxxxxx

t=1261.0 ns DM[000000e4] = xxxxxxxx || t=1261.0 ns IOM[000000e4] = xxxxxxxx

t=1261.0 ns DM[000000e8] = xxxxxxxx || t=1261.0 ns IOM[000000e8] = xxxxxxxx

t=1261.0 ns DM[000000ec] = xxxxxxxx || t=1261.0 ns IOM[000000ec] = xxxxxxxx

t=1261.0 ns DM[000000f0] = xxxxxxxx || t=1261.0 ns IOM[000000f0] = xxxxxxxx

t=1261.0 ns DM[000000f4] = xxxxxxxx || t=1261.0 ns IOM[000000f4] = xxxxxxxx

t=1261.0 ns DM[000000f8] = xxxxxxxx || t=1261.0 ns IOM[000000f8] = xxxxxxxx

t=1261.0 ns DM[000000fc] = xxxxxxxx || t=1261.0 ns IOM[000000fc] = xxxxxxxx

# Stopped at time : 1261 ns

/* Instruction Memory Module 13 */

Set interrupt flag to 1

@0

00 00 00 1f

3c 01 12 34

34 21 56 78

R1 <- 0x12345678
R2 <- 0x87654321
R3 <- 0xABCDEF01
R4 <-  0x0FEDCBA
R5 <- 0x5A5A5A5A
R6 <- 0xFFFFFF00

3c 02 87 65

34 42 43 21

3c 03 ab cd

34 63 ef 01

3c 04 01 fe

34 84 dc ba

3c 05 5a 5a

34 a5 5a 5a

3c 06 ff ff

34 c6 ff ff

Add R6 with R7 through R14 to R8 through R15.

3c 07 ff ff

34 e7 ff 00

00 c7 40 20

00 c8 48 20

00 c9 50 20

00 ca 58 20

R7 <- 100103F0
DM[3F0] <- R15

00 cb 60 20

00 cc 68 20

00 cd 70 20

00 ce 78 20

Interrupt Service Routine

3c 07 10 01

34 e7 03 f0

ac ef 00 00

00 00 00 0d

R16 <- 0x100100C0 // IOmem address pointer
R17 <- 0x8000FFFF // pattern
R18 <- 0x0010         // loop counter

@200

3c 10 10 01

36 10 00 c0

3c 11 80 00

36 31 ff ff

20 12 00 10

IOM[R16] <- R17
shift right arithmetic by 2 bits
Increment memory pointer
Decrement loop counter
Branch to out_IO if R18 != 0

76 11 00 00 out_IO:

00 11 88 83

22 10 00 04

22 52 ff ff

16 40 ff fb

3c 10 10 01

R16 <- 0x100100C0 // Set memory pointer

36 10 00 c0

72 13 00 00

72 14 00 04

72 15 00 08

72 16 00 0c

Input R19 to R24 from IOM[R16] 
Increment R16 by 4

72 17 00 10

72 18 00 14

03 e0 00 08

Jump register to return address (R31)

Memory pointer

Loaded R1-R6

/* Verification 13 */

BREAK INSTRUCTION FETCHED 4955.0 ns

Result from shifting

R E G I S T E R ' S A F T E R B R E A K

t=4961.0 ns $r0 = 00000000 || t=4961.0 ns $r16 = 100100c0

t=4961.0 ns $r1 = 12345678 || t=4961.0 ns $r17 = ffffffff

Loop counter

t=4961.0 ns $r2 = 87654321 || t=4961.0 ns $r18 = 00000000

Results from Add instructions

t=4961.0 ns $r3 = abcdef01 || t=4961.0 ns $r19 = 8000ffff

t=4961.0 ns $r4 = 01fedcba || t=4961.0 ns $r20 = e0003fff

Input from IOM

t=4961.0 ns $r5 = 5a5a5a5a || t=4961.0 ns $r21 = f8000fff

t=4961.0 ns $r6 = ffffffff || t=4961.0 ns $r22 = fe0003ff

t=4961.0 ns $r7 = 100103f0 || t=4961. 0 ns $r23 = ff8000ff

t=4961.0 ns $r8 = fffffeff || t=4961.0 ns $r24 = ffe0003f

t=4961.0 ns $r9 = fffffefe || t=4961.0 ns $r25 = xxxxxxxx

t=4961.0 ns $r10 = fffffefd || t=4961.0 ns $r26 = xxxxxxxx

t=4961.0 ns $r11 = fffffefc || t=4961.0 ns $r27 = xxxxxxxx

t=4961.0 ns $r12 = fffffefb || t=4961.0 ns $r28 = xxxxxxxx

t=4961.0 ns $r13 = fffffefa || t=4961.0 ns $r29 = 000003fc

t=4961.0 ns $r14 = fffffef9 || t=4961.0 ns $r30 = xxxxxxxx

Results from shifting

t=4961.0 ns $r15 = fffffef8 || t=4961.0 ns $r31 = 00000064

time=4961.0 ns M[3F0]=fffffef8

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=4961.0 ns DM[000000c0] = xxxxxxxx || t=4961.0 ns IOM[000000c0] = 8000ffff

t=4961.0 ns DM[000000c4] = xxxxxxxx || t=4961.0 ns IOM[000000c4] = e0003fff

t=4961.0 ns DM[000000c8] = xxxxxxxx || t=4961.0 ns IOM[000000c8] = f8000fff

t=4961.0 ns DM[000000cc] = xxxxxxxx || t=4961.0 ns IOM[000000cc] = fe0003ff

t=4961.0 ns DM[000000d0] = xxxxxxxx || t=4961.0 ns IOM[000000d0] = ff8000ff

t=4961.0 ns DM[000000d4] = xxxxxxxx || t=4961.0 ns IOM[000000d4] = ffe0003f

t=4961.0 ns DM[000000d8] = xxxxxxxx || t=4961.0 ns IOM[000000d8] = fff8000f

t=4961.0 ns DM[000000dc] = xxxxxxxx || t=4961.0 ns IOM[000000dc] = fffe0003

t=4961.0 ns DM[000000e0] = xxxxxxxx || t=4961.0 ns IOM[000000e0] = ffff8000

t=4961.0 ns DM[000000e4] = xxxxxxxx || t=4961.0 ns IOM[000000e4] = ffffe000

t=4961.0 ns DM[000000e8] = xxxxxxxx || t=4961.0 ns IOM[000000e8] = fffff800

t=4961.0 ns DM[000000ec] = xxxxxxxx || t=4961.0 ns IOM[000000ec] = fffffe00

t=4961.0 ns DM[000000f0] = xxxxxxxx || t=4961.0 ns IOM[000000f0] = ffffff80

t=4961.0 ns DM[000000f4] = xxxxxxxx || t=4961.0 ns IOM[000000f4] = ffffffe0

t=4961.0 ns DM[000000f8] = xxxxxxxx || t=4961.0 ns IOM[000000f8] = fffffff8

t=4961.0 ns DM[000000fc] = xxxxxxxx || t=4961.0 ns IOM[000000fc] = fffffffe

# Stopped at time : 4961 ns

/* Instruction Memory Module 14 */

R1 <- 0x12345678
R2 <- 0x87654321
R3 <- 0xABCDEF01
R4 <-  0x0FEDCBA
R5 <- 0x5A5A5A5A
R6 <- 0xFFFFFF00

Set interrupt flag to 1

Add R6 with R7 through R14 to R8 through R15.

R7 <- 100103F0
DM[3F0] <- R15

@0

00 00 00 1f

3c 01 12 34

34 21 56 78

3c 02 87 65

34 42 43 21

3c 03 ab cd

34 63 ef 01

3c 04 01 fe

34 84 dc ba

3c 05 5a 5a

34 a5 5a 5a

3c 06 ff ff

34 c6 ff ff

3c 07 ff ff

34 e7 ff 00

00 c7 40 20

00 c8 48 20

00 c9 50 20

Interrupt Service Routine

Push current PC and flags on the stack

00 ca 58 20

00 cb 60 20

00 cc 68 20

00 cd 70 20

00 ce 78 20

3c 07 10 01

34 e7 03 f0

ac ef 00 00

R16 <- 0x100100C0 // IOmem address pointer
R17 <- 0x8000FFFF // pattern
R18 <- 0x0010         // loop counter

00 00 00 0d

@200

3c 10 10 01

36 10 00 c0

3c 11 80 00

IOM[R16] <- R17
shift right arithmetic by 2 bits
Increment memory pointer
Decrement loop counter
Branch to out_IO if R18 != 0

36 31 ff ff

20 12 00 10

76 11 00 00

00 11 88 83

22 10 00 04

22 52 ff ff

16 40 ff fb

3c 10 10 01

36 10 00 c0

R16 <- 0x100100C0 // Set memory pointer

72 13 00 00

72 14 00 04

72 15 00 08

72 16 00 0c

Input R19 to R24 from IOM[R16] 
Increment R16 by 4

72 17 00 10

72 18 00 14

7B A0 00 00

Return Interrupt Routine
Pop the flags and PC back from stack

Results from shifting

Memory pointer

Loaded R1-R6

/* Verification 14 */

BREAK INSTRUCTION FETCHED 5025.0 ns

Result from shifting

R E G I S T E R ' S A F T E R B R E A K

t=5031.0 ns $r0 = 00000000 || t=5031.0 ns $r16 = 100100c0

Loop counter

t=5031.0 ns $r1 = 12345678 || t=5031.0 ns $r17 = ffffffff

Results from Add instructions

t=5031.0 ns $r2 = 87654321 || t=5031.0 ns $r18 = 00000000

t=5031.0 ns $r3 = abcdef01 || t=5031.0 ns $r19 = 8000ffff

t=5031.0 ns $r4 = 01fedcba || t=5031.0 ns $r20 = e0003fff

Input from IOM

t=5031.0 ns $r5 = 5a5a5a5a || t=5031.0 ns $r21 = f8000fff

t=5031.0 ns $r6 = ffffffff || t=5031.0 ns $r22 = fe0003ff

t=5031.0 ns $r7 = 100103f0 || t=5031.0 ns $r23 = ff8000ff

t=5031.0 ns $r8 = fffffeff || t=5031.0 ns $r24 = ffe0003f

t=5031.0 ns $r9 = fffffefe || t=5031.0 ns $r25 = xxxxxxxx

t=5031.0 ns $r10 = fffffefd || t=5031.0 ns $r26 = xxxxxxxx

t=5031.0 ns $r11 = fffffefc || t=5031.0 ns $r27 = xxxxxxxx

t=5031.0 ns $r12 = fffffefb || t=5031.0 ns $r28 = xxxxxxxx

t=5031.0 ns $r13 = fffffefa || t=5031.0 ns $r29 = 100103f0

t=5031.0 ns $r14 = fffffef9 || t=5031.0 ns $r30 = xxxxxxxx

t=5031.0 ns $r15 = fffffef8 || t=5031.0 ns $r31 = 00000064

time=5031.0 ns M[3F0]=fffffef8

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=5031.0 ns DM[000000c0] = xxxxxxxx || t=5031.0 ns IOM[000000c0] = 8000ffff

t=5031.0 ns DM[000000c4] = xxxxxxxx || t=5031.0 ns IOM[000000c4] = e0003fff

t=5031.0 ns DM[000000c8] = xxxxxxxx || t=5031.0 ns IOM[000000c8] = f8000fff

t=5031.0 ns DM[000000cc] = xxxxxxxx || t=5031.0 ns IOM[000000cc] = fe0003ff

t=5031.0 ns DM[000000d0] = xxxxxxxx || t=5031.0 ns IOM[000000d0] = ff8000ff

t=5031.0 ns DM[000000d4] = xxxxxxxx || t=5031.0 ns IOM[000000d4] = ffe0003f

t=5031.0 ns DM[000000d8] = xxxxxxxx || t=5031.0 ns IOM[000000d8] = fff8000f

t=5031.0 ns DM[000000dc] = xxxxxxxx || t=5031.0 ns IOM[000000dc] = fffe0003

t=5031.0 ns DM[000000e0] = xxxxxxxx || t=5031.0 ns IOM[000000e0] = ffff8000

t=5031.0 ns DM[000000e4] = xxxxxxxx || t=5031.0 ns IOM[000000e4] = ffffe000

t=5031.0 ns DM[000000e8] = xxxxxxxx || t=5031.0 ns IOM[000000e8] = fffff800

t=5031.0 ns DM[000000ec] = xxxxxxxx || t=5031.0 ns IOM[000000ec] = fffffe00

t=5031.0 ns DM[000000f0] = xxxxxxxx || t=5031.0 ns IOM[000000f0] = ffffff80

t=5031.0 ns DM[000000f4] = xxxxxxxx || t=5031.0 ns IOM[000000f4] = ffffffe0

t=5031.0 ns DM[000000f8] = xxxxxxxx || t=5031.0 ns IOM[000000f8] = fffffff8

t=5031.0 ns DM[000000fc] = xxxxxxxx || t=5031.0 ns IOM[000000fc] = fffffffe

Stopped at time : 5031 ns

/* Instruction Memory Module for Enhancement */

@0

R1 <- 0x01234567
R2 <- 0x87654321
R3 <- 0xABCDEF01

DM[3F8] <- R1; DM[3F4] <- R2; DM[3F0] <- R3

3c 01 12 34

34 21 56 78

7c 01 00 09

3c 02 87 65

34 42 43 21

7c 02 00 09

3c 03 ab cd

34 63 ef 01

No_operation

7c 03 00 09

7c 00 00 08

R4 <- DM[3F0] (0xABCDEF01)
R5 <- DM[3F4] (0x87654321)
R6 <- DM[3F8] (0x01234567)
Branch to blt_p if R1 < R2
else R16 <- 0xFFFFFFFF (fail) then break.

7c 04 00 0a

7c 05 00 0a

7c 06 00 0a

b0 22 00 03

3c 10 ff ff

36 10 ff ff

00 00 00 0d

3c 10 11 11 blt_p:

R16 <- 0x11111111 (pass)
Branch to bge_p if R6 >= zero
else R17 <- 0xFFFFFFFF (fail) then break.

36 10 11 11

b4 c0 00 03

3c 11 ff ff

36 31 ff ff

00 00 00 0d

7e 11 00 07 bge_p:

R17 <- R16(0x11111111) (pass)
Rotate Left R17 by 2; R7 <- 0x44444444
Rotate Right R17 by 3; R8 <- 0x22222222

7c 11 38 81

7c 11 40 c2

20 0d 00 07

20 0e 00 07

3c 09 00 00

35 29 00 01

Initiate loop_counter1 R13 <- 0x07
Initiate loop_counter2 R14 <- 0x07
R9 <- 0x00000001
set Memory pointer R15 <- 0x100100C0

3c 0f 10 01

35 ef 00 c0

75 e9 00 00 IO:

00 09 49 00

21 ef 00 04

b9 ad ff fc

7d 2a 00 07

Store R9 in IO memory at R15 address
IOM[R15] <- R9;
Shift left logic R9 by 4
Increment pointer R15 <- R15 +4
Loop until R13 = zero, then R10 <- R9

ad ea 00 00 DM:

00 0a 51 02

21 ef 00 04

b9 ce ff fc

7c 01 00 05

Break.

DM[R15] <- R10;
Shift right logic R10 by 4
Increment pointer R15 <- R15 +4
Loop until R14 = zero, then R1 <- 0x00

00 00 00 0d

Pass all instructions.

/* Verification Enhancement */

Pass flags

BREAK INSTRUCTION FETCHED 3565.0 ns

R E G I S T E R ' S A F T E R B R E A K

t=3571.0 ns $r0 = 00000000 || t=3571.0 ns $r16 = 11111111

t=3571.0 ns $r1 = 00000000 || t=3571.0 ns $r17 = 11111111

t=3571.0 ns $r2 = 87654321 || t=3571.0 ns $r18 = xxxxxxxx

t=3571.0 ns $r3 = abcdef01 || t=3571.0 ns $r19 = xxxxxxxx

Load Immediate to R1-R3 and push them on Data memory stack.

t=3571.0 ns $r4 = abcdef01 || t=3571.0 ns $r20 = xxxxxxxx

t=3571.0 ns $r5 = 87654321 || t=3571.0 ns $r21 = xxxxxxxx

t=3571.0 ns $r6 = 12345678 || t=3571.0 ns $r22 = xxxxxxxx

t=3571.0 ns $r7 = 44444444 || t=3571.0 ns $r23 = xxxxxxxx

t=3571.0 ns $r8 = 22222222 || t=3571.0 ns $r24 = xxxxxxxx

Pop data from the stack to R4-R6

t=3571.0 ns $r9 = 10000000 || t=3571.0 ns $r25 = xxxxxxxx

Shows that 0xABCDEF01 was pushed to this address

t=3571.0 ns $r10 = 00000001 || t=3571.0 ns $r26 = xxxxxxxx

t=3571.0 ns $r11 = xxxxxxxx || t=3571.0 ns $r27 = xxxxxxxx

t=3571.0 ns $r12 = xxxxxxxx || t=3571.0 ns $r28 = xxxxxxxx

Loop counters

t=3571.0 ns $r13 = 00000000 || t=3571.0 ns $r29 = 000003fc

t=3571.0 ns $r14 = 00000000 || t=3571.0 ns $r30 = xxxxxxxx

Memory pointer

Result from shiftings

Results from ROTL and ROTR

t=3571.0 ns $r15 = 100100f8 || t=3571.0 ns $r31 = xxxxxxxx

time=3571.0 ns M[3F0]=abcdef01

_______Data Memory(DM)______________||_________IO Memory(IOM)_____

t=3571.0 ns DM[000000c0] = xxxxxxxx || t=3571.0 ns IOM[000000c0] = 00000001

t=3571.0 ns DM[000000c4] = xxxxxxxx || t=3571.0 ns IOM[000000c4] = 00000010

t=3571.0 ns DM[000000c8] = xxxxxxxx || t=3571.0 ns IOM[000000c8] = 00000100

t=3571.0 ns DM[000000cc] = xxxxxxxx || t=3571.0 ns IOM[000000cc] = 00001000

t=3571.0 ns DM[000000d0] = xxxxxxxx || t=3571.0 ns IOM[000000d0] = 00010000

t=3571.0 ns DM[000000d4] = xxxxxxxx || t=3571.0 ns IOM[000000d4] = 00100000

t=3571.0 ns DM[000000d8] = xxxxxxxx || t=3571.0 ns IO [000000d8] = 01000000

t=3571.0 ns DM[000000dc] = 10000000 || t=3571.0 ns IOM[000000dc] = xxxxxxxx

t=3571.0 ns DM[000000e0] = 01000000 || t=3571.0 ns IOM[000000e0] = xxxxxxxx

Walking one forward from looping

t=3571.0 ns DM[000000e4] = 00100000 || t=3571.0 ns IOM[000000e4] = xxxxxxxx

t=3571.0 ns DM[000000e8] = 00010000 || t=3571.0 ns IOM[000000e8] = xxxxxxxx

t=3571.0 ns DM[000000ec] = 00001000 || t=3571.0 ns IOM[000000ec] = xxxxxxxx

t=3571.0 ns DM[000000f0] = 00000100 || t=3571.0 ns IOM[000000f0] = xxxxxxxx

t=3571.0 ns DM[000000f4] = 00000010 || t=3571.0 ns IOM[000000f4] = xxxxxxxx

Walking one backward from looping

t=3571.0 ns DM[000000f8] = xxxxxxxx || t=3571.0 ns IOM[000000f8] = xxxxxxxx

t=3571.0 ns DM[000000fc] = xxxxxxxx || t=3571.0 ns IOM[000000fc] = xxxxxxxx

Stopped at time : 3571 ns